3GPP TSG-RAN WG2 Meeting #117-e	draft R2-2203665

Online, 21st – 3rd February, 2022

Agenda item:	8.3.5

Source: 	Huawei, HiSilicon

Title: 	[AT117-e][233][MUSIM] MUSIM UE capabilities (Huawei)

Document for:	Discussion and Decision

# Introduction

The following offline was agreed to discuss MUSIM gap pattern capabilities and other MUSIM capabilities:

- [AT117-e][233][MUSIM] MUSIM UE capabilities (Huawei)

- Scope: Discuss MUSIM gap pattern capabilities and other capabilities. Provide updated capability descriptions (to be merged to the general capability CR).

- Intended outcome: Discussion report in R2-2203665

- Deadline: Deadline 4 (Monday W2, 1200 UTC for comments)

| Company | Contact Name, Email |
| --- | --- |
| MediaTek | Felix Tsai, chun-fan.tsai@mediatek.com |
| Ericsson | Lian Araujo, lian.araujo@ericsson.com |
| OPPO | Jiangsheng Fan, fanjiangsheng@oppo.com |
| Huawei/HiSilicon | Rama Kumar Mopidevi, rama.kumar@huawei.com |
| Charter Communications | Reza Hedayat, reza.hedayat@charter.com |
| Qualcomm | Ozcan Ozturk, oozturk@qti.qualcomm.com |
| Intel | Sudeep Palat (sudeep.k.palat@intel.com) |
| Samsung | Sangyeob Jung (sy0123.jung@samsung.com) |
| ZTE | Wenting Li(Li.wenting2zte.com.cn) |
| Sharp | Fangying Xiao, Fangying.xiao@cn.sharp-world.com |

# Discussion

Based on the summary of Stage-3 details of MUSIM, there are two open issues for MUSIM UE capabilities [1]:

OI5-1: Whether to indicate which MUSIM gap patterns are supported by UE (similar with UE capability supportedGapPattern in 38.306 ) based CR in RAN4 R4-2202760 OI5-2: Whether in TS 38.306 and TS 36.306 that paging cause feature is optional feature without UE radio access capability parameters?

## 2.1 UE Capability to indicate supported MUSIM gap patterns

Following is the summary of proposals from the contributions to address OI5-1:

- UE supported gap patterns for MUSIM is not signalled as part of the UE AS capability [2]

- UE indicates the supported MUSIM gap patterns information to the network [3][4][5][7]

- Whether to indicate which MUSIM gap patterns are supported by UE shall be decided by RAN4 group [6]

During the online, RAN2 made the following agreement related to gap configuration:

- Network should always provide at least one of the requested gap pattern or no gaps. Network providing an alternative gap pattern instead of the one requested by the UE is not supported in this release.

Based on the agreement, there is no need for the UE to indicate what gap patterns it supports.

Q1: Do you agree that there is no need for the UE to indicate its supported gap patterns?

| Company | Response | Comments |
| --- | --- | --- |
| MediaTek | Agree | There is no need to have the MUSIM gap patterns capability as the NW may only configures the pattern requested by UE. |
| Ericsson | Yes, but | There seems to be no need to capture any further agreement on this. |
| OPPO | Agree |  |
| Huawei/HiSilicon | Agree |  |
| Charter Communications | Agree |  |
| Qualcomm | Agree |  |
| Intel | Agree |  |
| Samsung | Agree |  |
| ZTE | Agree | Based on the agreement this meeting, there no need to indicate the supported gap patterns anymore. |
| Sharp | Agree | Considering that only the gap pattern request by UE can be configured in this release, there is no need to indicate its capability. |

Summary:

Proposal:

## 2.2 Capturing paging cause feature

It was proposed in [7] to capture paging cause feature in 38.306 and 36.306 as an optional capability without UE radio access capability parameters. Annex 1 & 2 contain the TP proposed in [7].

Q2: Do you agree to capture paging cause feature as an optional capability without UE radio access capability parameters in 38.306 and 36.306?

| Company | Response | Comments |
| --- | --- | --- |
| MediaTek | Agree |  |
| Ericsson | Agree |  |
| OPPO | Disagree | NAS-based Paging cause capability is already introduced in SA2 spec in R17, anyway Paging cause is a requirement from NAS and will be used by NAS. If NAS Paging cause capability is enabled, it’s nature that AS should support paging cause reading; vice versa, so what’s the necessity to define a similar capability in AS for a NAS feature, which is already controlled by NAS capability? |
| Huawei/HiSilicon | Disagree | As Oppo mentioned, Paging cause feature is a NAS UE capability and the UE is aware if it’s supported by the CN. CN transfers the UE capability of supporting paging cause to RAN. There is no chance for misunderstanding that paging cause feature is mandatory. Another point is 36.306 is not in the WID scope. Hence there is no need to capture paging cause feature as an optional capability in 38.306/36.306. |
| Charter Communications | Disgaree |  |
| Qualcomm | Disagree | We define an AS capability if there is a specified UE behavior in AS which is observable. For paging cause, AS just passes the cause to NAS for action, which is UE internal and thus doesn’t warrant a capability. |
| Intel | See comments | We don’t have a strong view either way. On one hand, it is signalled at NAS and the only AS function is to transparently pass the cause to upper layers and it is not needed to capture a UE AS capability for this. On the other hand, paging cause can also be included in RAN originated paging and supporting this in UE AS can be considered an AS function even if it is to provide it transparently to NAS. |
| Samsung | Maybe not | We have some sympathy with this as Intel commented but there seems no real need to capture it in TS 38.306 and TS 36.306 as support of paging cause feature is already specified in SA2 specification (e.g. TS 23.502). |
| ZTE | No strong view | We share the similar view to Intel. |
| Sharp | Disagree | Some view as OPPO. |

Summary:

Proposal:

# Conclusion

We propose the summary from offline as below:

# References

- R2-2203635 [Pre117-e][230][MUSIM] Summary Stage-3 details of MUSIM

- R2-2202646 MUSIM remaining issue on gap capability signalling, Intel.

- R2-2202700 Remaining issues for MUSIM UE Capabilities, Huawei.

- R2-2202885 Consideration on the MUSIM UE capability reporting, ZTE.

- R2-2202936 UE Capabilities for MUSIM Gap Pattern, Oppo.

- R2-2203435 Remaining aspects on UE capabilities for Multi-USIM and other issues, Ericsson.

- R4-2202966 Remaining issue on Multi-USIM UE capabilities, Vivo.

- Annex 1 <TP for 38.306>

- 5	Optional features without UE radio access capability parameters

- 5.x	MUSIM features

Definitions for feature

Paging cause It is optional for UE to support paging cause reception as specified in TS 38.331 [9].

- Annex 2 <TP for 36.306>

- 6	Optional features without UE radio access capability parameters

- 6.x	MUSIM features

- 6.x.1	Paging cause feature

It is optional for UE to support paging cause reception as specified in TS 36.331 [5].