3GPP TSG-RAN WG2 #117-e	R2-220xxxx

- Online, February 21 – March 3, 2022

Agenda item:	8.7.2.4

Source:	Samsung

Title:	[AT117-e][619][Relay] Flow control and pre-emptive BSR mechanisms

Document for:	Discussion & Decision

# Introduction

This is to discuss flow control and pre-emptive BSR mechanisms for L2 U2N relay based on P1-P3 of R2-2202955[1].

- [AT117-e][619][Relay] Flow control and pre-emptive BSR mechanisms (Samsung)

Scope: Discuss P1-P3 of R2-2202955 and determine if agreeable mechanisms can be developed.  The features can be considered independently of each other.

Intended outcome: Endorsable TPs to affected specifications

Deadline:  Thursday 2022-02-24 1200 UTC

# Discussion

(1) Flow control for L2 Relay

About flow control for L2 U2N Relay, we have the following proposals in the summary [1]. Rapporteur thinks that we can discuss basic mechanism for flow control based on proposal 2 and supporting companies input during the email discussion [Pre117-e][602].

Proposal 1: RAN2 to discuss the support of flow control in L2 U2N relay. Proposal 2: If flow control is supported, RAN2 to discuss that mode 2 Relay UE can transmit flow control indication over PC5 to its Remote UE for UL transmission and transmit flow control indication to its gNB over Uu for DL transmission where the flow control indication can be MAC signaling.

Q1. Do companies support flow control in L2 U2N relay?

| Company | Yes / No | Comments |
| --- | --- | --- |
| Qualcomm | No | - From technique perspective, because Rel-17 supports only single hop, we think flow control can be achieved by relay UE and gNB implementation. For example, the below solutions can be considered: - gNB or relay UE can limit number of remote UE connections based on the load and channel quality of the Uu and PC5 links. - In single hop relay, the traffic load of remote UEs can be roughly estimated based on SL BSR - We still don’t know what MAC signalling design for flow control indication is, which was not even discussed as candidate solution. We are not sure how an on-fly solution can be specified in last RAN2 meeting of Rel-17. |
| Ericsson | comments | We have no strong view on FC, it can be introduced only in case it is feasible to design with minimized standardization efforts. However, defining a new MAC CE seems to incur too much work for RAN2. |
| vivo | No | Flow control is an optimization feature. Furthermore, RAN2 had agreed that control PDU is supported in neither PC5 SRAP layer nor Uu SRAP layer in this release, which means that IAB like flow control mechanism may not be reused in relay. And defining a new MAC CE will need much specification efforts. Hence, a simple flow control mechanism may not be easily achieved. |
| CATT | Yes | - As we discussed in the pre-email discussion on QoS, we think flow control is necessary due to the following reasons: UL of L2 U2N relay Since only mode 2 is used by remote UE, PC5 UL transmission is controlled by remote UE itself. When the remote UE performs mode 2 resource allocation, according to the current MAC specification, it will only consider the available resource indicated by physical layer, the remaining PDB of SL data, the number of HARQ retransmission and etc. Hence, for UL, it is possible that a lot of data is sent from remote UE to relay UE, but it cannot be sent to gNB since the Uu UL is congested. Hence, UL flow control is necessary for L2 U2N relay. DL of L2 U2N relay There are two cases: - Case 1: Relay UE is configured with mode 1. In this case, both the DL Uu and PC5 transmission are controlled by gNB. gNB is aware whether PC5 link is congested or not, hence gNB implementation can handle the DL flow control if relay UE is configured with mode 1. - Case 2: Relay UE is configured with mode 2. In this case, only the DL Uu scheduling is controlled by gNB, but the DL PC5 scheduling is controlled by relay UE. Hence, it is possible that Uu is not congested but PC5 is congested. In order to avoid it, DL flow control is necessary for L2 U2N relay if relay UE is configured with mode 2. - If flow control is introduced, MAC CE is more suitable to be used to implement this function. |

Q2. For DL transmission flow control, do companies support that flow control indication can be transmitted by a Relay UE to its gNB over Uu?

| Company | Yes / No | Comments |
| --- | --- | --- |
| Qualcomm | No | 1. We are not convinced that flow control is necessary for single hop relay in Rel-17. 2. We still don’t know what Uu MAC-CE design for flow control indication is, which was not even discussed as candidate solution. We are not sure how an on-fly solution can be specified in last RAN2 meeting of Rel-17. |
| Ericsson | comments | We have no strong view on FC, it can be introduced only in case it is feasible to design with minimized standardization efforts. However, defining a new MAC CE seems to incur too much work for RAN2. |
| vivo | No | Flow control is an optimization feature. And defining a new MAC CE will need much specification efforts. |
| CATT | Yes | As commented in Q1, for DL, for relay UE configured with mode 1, gNB implementation can handle the DL flow control if relay UE is configured with mode 1. But for relay UE configured with mode 2, only the DL Uu scheduling is controlled by gNB, but the DL PC5 scheduling is controlled by relay UE. Hence, it is possible that Uu is not congested but PC5 is congested, DL flow control is necessary. |

Q3. If yes for Q2, do companies agree that the operation can be only applied for mode 2 Relay UE?

| Company | Yes / No | Comments |
| --- | --- | --- |
| CATT | Yes | See comment in Q2. |

Q4. If yes for Q2, do companies support that flow control indication can be MAC signalling i.e., MAC CE?

| Company | Yes / No | Comments |
| --- | --- | --- |
| CATT | Yes | MAC CE is the simplest way to implement the flow control. |

Q5. For UL transmission flow control, do companies support that flow control indication can be transmitted by a Relay UE to its Remote UE over PC5?

| Company | Yes / No | Comments |
| --- | --- | --- |
| Qualcomm | No | 1. We are not convinced that flow control is necessary for single hop relay in Rel-17. 2. We still don’t know what PC5 MAC CE design for flow control indication is, which was not even discussed as candidate solution. We are not sure how an on-fly solution can be specified in last RAN2 meeting of Rel-17. |
| Ericsson | comments | We have no strong view on FC, it can be introduced only in case it is feasible to design with minimized standardization efforts. However, defining a new MAC CE seems to incur too much work for RAN2. |
| vivo | No | Flow control is an optimization feature. And defining a new PC5 MAC CE will need much specification efforts. |
| CATT | Yes | See our comment in Q1. Since only mode 2 is used by remote UE, PC5 UL transmission is controlled by remote UE itself. When the remote UE performs mode 2 resource allocation, according to the current MAC specification, it will only consider the available resource indicated by physical layer, the remaining PDB of SL data, the number of HARQ retransmission and etc. Hence, for UL, it is possible that a lot of data is sent from remote UE to relay UE, but it cannot be sent to gNB since the Uu UL is congested. Hence, UL flow control is necessary for L2 U2N relay. |

Q6. If yes for Q5, do companies agree that the operation can be applied to any resource allocation mode of Relay UE (i.e., mode 1 and mode 2)?

| Company | Yes / No | Comments |
| --- | --- | --- |
| CATT | Yes | UL low control is sent from relay UE to remote UE, it should be supported no matter which resource allocation mode is used by relay UE. |

Q7. If yes for Q5, do companies support that flow control indication can be MAC signalling i.e., MAC CE?

| Company | Yes / No | Comments |
| --- | --- | --- |
| CATT | Yes | MAC CE is the simplest way to implement the flow control. |

Q8. For Q2 and Q5, do companies support that flow control indication can be triggered due to the buffer load at Relay UE?

| Company | Yes / No | Comments |
| --- | --- | --- |
| CATT | Yes | For DL, flow control indication can be sent to gNB if buffer load for a certain PC5 link is above the threshold. For UL, if the Uu buffer load of relay UE is above the threshold, flow control indication can be sent to remote UE. |

Q9. Any additional comments for flow control mechanism?

| Company | Yes / No | Comments |
| --- | --- | --- |

(2) Pre-emptive BSR

Regarding pre-emptive BSR for L2 U2N Relay, we have the following proposal in the summary [1]. In this email discussion Rapporteur thinks that we can discuss basic mechanism for pre-emptive BSR based on proposal 3.

Proposal 3: RAN2 to discuss the support of pre-emptive BSR by Relay UE. If supported, FFS how pre-emptive BSR is triggered.

During the email discussion [Pre117-e][602], the supporting companies propose to reuse pre-emptive BSR mechanism for IAB as much as possible and to use Remote UE’s SCI information to trigger the pre-emptive BSR. Therefore Rapporteur suggests to discuss the FFS in proposal 3 based on the supporting companies input.

Q10. Do companies support pre-emptive BSR transmission by a Relay UE to gNB?

| Company | Yes / No | Comments |
| --- | --- | --- |
| Ericsson | Yes | This is beneficial to reduce latency for delay sensitive relay traffic (e.g., public safety service) due to dynamic scheduling. In addition, such mechanism is applicable to relay UE regardless of relaying options (i.e., either L2 relaying or L3 relaying). |
| vivo | No | The benefit of pre-emptive BSR in relay link is very limited: - There is no BSR mechanism in PC5 link from remote UE reporting to relay UE. Hence relay UE may only base on SCI information to evaluate potential data volume. If the SCI indicates the current PSSCH slot, only a few milliseconds can be saved. For the SCI with reservice interval, destination is not clear and potential data volume cannot be decided either. - Based on SCI indication, e.g. PC5 priority, relay UE cannot deduce exact logical channel group information. Hence pre-emptive BSR is not useful for gNB scheduling if everything of reporting is left to relay UE implementation. |
| CATT | Yes | Agree with Ericsson. |
| OPPO | No | Besides the analysis by vivo (which we agree), if the concern is on latency reduction, it would be more straight forward to use sidelink configured grant in mode 1. On the other side, it is not a wise choice to introduce new feature at this late stage. |

Q11. If yes for Q10, do companies support that pre-emptive BSR provides the information about the amount of the data expected to arrive at the Relay UE from its Remote UE(s)?

| Company | Yes / No | Comments |
| --- | --- | --- |
| Ericsson | Yes | This would be the same as pre-emptive BSR for IAB |
| CATT | Yes |  |

Q12. If yes for Q10, do companies support that the LCGs to be reported, the expected data volume calculation, the exact time to report pre-emptive BSR and the associated LCH are left to Relay UE’s implementation?

| Company | Yes / No | Comments |
| --- | --- | --- |
| Ericsson | Yes | To minimize standardization efforts, it is sufficient to leave up to relay UE implementation |
| CATT | Yes | It can be based on the resource reservation information in SCI. For minimize the specification effort, we can agree to leave it to relay UE implementation. |

Q13. If yes for Q10, do companies support that the pre-emptive BSR can be triggered based on its connected Remote UE’s SCI information?

| Company | Yes / No | Comments |
| --- | --- | --- |
| Ericsson | Yes | In most of cases, it is feasible up to relay UE implementation to derive buffer status of remote UE based on information obtained in the SCI signaling (e.g., resource reservation), and together with SL HARQ feedback. In this release, it is sufficient to leave to relay UE implementation to determine when pre-emptive BSR can be triggered. |
| CATT | Yes | Same comment as in Q12. |

Q14. Any additional comments for pre-emptive BSR mechanism?

| Company | Yes / No | Comments |
| --- | --- | --- |

# Conclusion

In summary, the following are proposed:

# References

[1] R2-2202955, Summary of [Pre117-e][602][Relay] Open issues on relay QoS