3GPP TSG RAN WG2#117-e	R2-2204196

e-Meeting, 21st February - 3rd March, 2022

3GPP TSG-RAN WG4 Meeting # 102-e 		R4-2206580

Online Meeting, 21 February - 03 March 2022

Title:	Reply LS on Length of Maximum duration

Response to:	R1-2106212 “LS on joint channel estimation for PUSCH and PUCCH”

Release:	Rel-17

Work Item:	NR_cov_enh

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact person:	Shan YANG

yangshan@chinatelecom.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 replied to LS R1-2106212 from RAN1 regarding maximum duration earlier with R4-2206537, which indicated that RAN4 concluded that for per band UE capability on length of maximum duration, the set of values of duration lengths for FDD are {4, 8, 16, 32} slots. Meanwhile, RAN4 will not define the requirements with 32 slots for FDD in Rel-17.

RAN4 continued discussing the granularity of the UE capability and the length of maximum duration for TDD, and further concluded that:

For the granularity of the UE capability on length of maximum duration:

- Per band UE capability was agreed in RAN4 #101e-bis. RAN4 has not discussed requirements for combination of this feature with UL CA in Rel-17, and RAN4 suggests the final decision on the granularity to be made in RAN1.

For the length of maximum duration for TDD:

- Regarding the RAN1 request for maximum duration, RAN4 understands that the UE capability for TDD refers to the number of consecutively transmitted UL slots over which the UE can meet the phase consistency requirement, assuming no phase/power consistency violating events in between.

- For the above UE capability (as defined), RAN4 has further concluded that the set of values of duration lengths should at least include {2, 4, 8} slots, and whether 16 slots can be included in the set of values for UE capability reporting is up to RAN1 to decide, but RAN4 does not intend to define requirements for 16 slots for TDD in Rel-17.

# 2	Actions

To RAN1 and RAN2

ACTION: RAN4 respectfully asks RAN1 and RAN2 to take the above information in to consideration in their work.

# 3	Dates of next TSG RAN WG 4 meetings

RAN4#103-e	2022-05-16 - 2022-05-27 		Electronic

RAN4#104	2022-08-22 - 2022-08-26 		Toulouse, FR