3GPP TSG-RAN WG2 #118-e	R2-220xxxx

Electronic meeting, 9th May – 20th May 2022

Agenda Item:	5.1.4

Source:	Ericsson

Title:	[AT118-e][016][NR1516] Connection Control I (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

The following document is to provide and collect input about a way forward related to the following email discussion:

- [AT118-e][016][NR1516] Connection Control I (Ericsson)

Scope: Treat R2-2205965, R2-2205966, R2-2205867, R2-2205406, R2-2205407, R2-2205868, R2-2205614, R2-2205586, R2-2205599

Ph1 Determine agreeable parts, Ph2 for agreeable parts agree CRs (offline agreement, CB online only if necessary).

Intended outcome: Report, Agreed CRs

Deadline: Schedule 1

A first round with Deadline for comments W1 Thursd May 12th 1200 UTC to settle scope what is agreeable etc
A Final round with Final deadline W2 Wednesd May 18th 1200 UTC to settle details / agree CRs etc.

# 2	Contact information

| - Company | - Name | - Email address |
| --- | --- | --- |
| - Samsung | - Seungri Jin | - seungri.jin@samsung.com |

# 3	Discussion

## 3.1	L1 parameters

R2-2205965	Correction of Need Code in IE SearchSpace	Ericsson	CR	Rel-15	38.331	15.17.0	3140	-	F	NR_newRAT-Core, TEI16

R2-2205966	Correction of Need Code in IE SearchSpace	Ericsson	CR	Rel-16	38.331	16.8.0	3141	-	A	NR_newRAT-Core, TEI16

R2-2205967	Correction of Need Code in IE SearchSpace	Ericsson	CR	Rel-17	38.331	17.0.0	3142	-	A	NR_newRAT-Core, TEI16

The CRs correct a conflict between Need Code and Field Description. 
Strictly, the proposed change is not backwards compatible.

Note there is a typo in the Rel-15 CR. CR missed to add the Need Code “S” that replaces the “R”.

Question 1: Do companies agree with the changes proposed in CRs listed above?

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Samsung | - See the comments | - Either approach has no functional differences but this Need R with adding the description of absent condition violates the general guideline. - From our understanding, Need R without the description of absent condition is also possible (i.e. remove “If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0”) because this field is used for “Number of consecutive slots that a SearchSpace lasts in every occasion”. In other words, absent of this field, UE use the value 1 slot for monitoring of SearchSpace. - If we strictly apply the rule for handing need code, we share the view from this change but no strong view on this change. |

## 3.2	L2 parameters

R2-2205406	CR on 38.331 for sn-FieldLength	ZTE Corporation,Sanechips	CR	Rel-15	38.331	15.17.0	3079	-	F	NR_newRAT-Core

R2-2205407	CR on 38.331 for sn-FieldLength	ZTE Corporation,Sanechips	CR	Rel-16	38.331	16.8.0	3080	-	A	NR_newRAT-Core

The CRs proposes to correct the field description of sn-FieldLength as ‘The value of sn-FieldLength for a RLC shall be changed only using reconfiguration with sync’

Question 2: Do companies agree with the changes proposed in CRs listed above?

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Samsung | - Yes | - We think the proposed change is correct. |

## 3.3	n77

R2-2205968	WF for NS_55 in NR CA	Ericsson	discussion	Rel-16	NR_RF_FR1-Core, TEI16

The document proposes to send LS to RAN4 to ask RAN4 to decide on solution for NS_55 in NR CA.

Question 3: Do companies agree with sending LS to RAN4 and await further RAN4 input.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Samsung | - Yes | - We are fine to send LS to RAN4. |

## 3.4	SMTC configuration

R2-2205614	SMTC configuration for target cell 	Lenovo	CR	Rel-16	38.331	16.8.0	3103	-	F	NR_newRAT-Core, TEI16

R2-2205586	SMTC configuration for target cell	Lenovo (Beijing) Ltd	CR	Rel-15	36.331	15.17.0	4804	-	F	NR_newRAT-Core

R2-2205599	SMTC configuration for target cell	Lenovo (Beijing) Ltd	CR	Rel-16	36.331	16.8.0	4805	-	F	NR_newRAT-Core

The CRs suggest to change ‘SN change’ to ‘PSCell change’ in the field description of targetCellSMTC-SCG-r16.

Question 4: Do companies agree with the changes proposed in CRs listed above?

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Samsung | - Yes |  |

# 4	Conclusion

Tbd

# 5	References