3GPP TSG-RAN WG2 Meeting #118-electronic	   R2-220xxxx

Online, May 9th - May 20th, 2022

Agenda item:		5.1.4.1

Source:	vivo

Title:	Report of [AT118-e][019][NR1516] CP Miscellanous

Document for:		Discussion and Decision

# 1 Introduction

This contribution is aimed at reporting the discussion and results of the following offline discussion:

[AT118-e][019][NR1516] CP Miscellanous (vivo)

Scope: Treat R2-2204902, R2-2205428, R2-2205429, R2-2204845, R2-2204846, R2-2205827, R2-2204728, R2-2204729, R2-2204845, R2-2204846, R2-2205827, R2-2204728, R2-2204729, R2-2205503, R2-2205504, R2-2205298, R2-2205299, R2-2205300

Ph1 Determine agreeable parts, Ph2 for agreeable parts agree CRs (offline agreement, CB online only if necessary).

Intended outcome: Report, Agreed CRs

Deadline: Schedule 1

The discussion scope is to gather companies’ views on the contributions [1]-[13]. Companies are invited to provide their views by May 12th (Thursday), 2022, 12:00 UTC for phase-1 discussion.

# 2 Participants

To facilitate this offline discussion amongst the delegates, would you please fill in your name and the email address in the table below.

| Delegate name | E-mail address |
| --- | --- |
| Yitao Mo (Stephen) | yitao.mo@vivo.com |
| Nokia | amaanat.ali@nokia.com |

# 3 Phase-1 Discussion

## 3.1 Clarification for Inter-MN HO without SN change

In the previous RAN2 meeting, the need for Stage 3 CR regarding inter-MN handover without SN change was discussed but postponed without consensus. The corresponding agreement is given as follows,

RAN2#117 meeting agreements R2-2202807 Clarification on inter-MN handover without SN change NEC CR Rel-15 38.331 15.16.0 2907 - F NR_newRAT-Core R2-2202808 Clarification on inter-MN handover without SN change NEC CR Rel-16 38.331 16.7.0 2908 - A NR_newRAT-Core [029] Both Postponed

To completely solve this issue which has been discussed for almost one year, the following proposal is given in the contribution [1],

Proposal 1: RAN2 to agree to capture the following in a Chairman notes. - RAN2 confirms that according to the current RRC spec, both fields sourceConfigSCG and scg-RB-Config in CG-ConfigInfo can be sent in the following cases: - SN change procedure - Inter-MN HO with SN change - Inter-MN HO without SN change (Case 0) - Inter-MN HO without SN node change (Case 2)

Q1: Do companies agree with Proposal 1?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| Nokia | Yes | We are fine to capture the scenarios listed in P1 for chair notes |

Summary:

## 3.2 Clarification for p-maxNR-FR1 in NR-DC

In the CRs R2-2205428/5429 [2][3], it is clarified that the filed p-maxNR-FR1 is also used to indicate the maximum total transmit power to be used by the UE in the NR SCG across all serving cells in frequency range 1, which is not reflected in the current specification. The corresponding correction is quoted as follows,

p-maxNR-FR1 For (NG)EN-DC and NE-DC, the field indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 1 (FR1) (see TS 38.104 [12]). For NR-DC, it indicates the the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 1 (FR1) (see TS 38.104 [12]) the UE can use in NR SCG.

Q2: Do companies agree with the intention of CR?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| Nokia | Neutral | Proposed change is correct but we are not sure there is any misunderstanding as there is the p-maxUE-FR1 for full FR1 across all cell groups. So not sure why there should be particularly misunderstanding with this one. |

Summary:

## 3.3 Correction on rrc-ConfiguredUplinkGrant

In the CRs R2-2204845/4846/5827 [4]-[6], it is pointed out that the field description parts for both precodingAndNumberOfLayers and pathlossReferenceIndex are missing within the field rrc-ConfiguredUplinkGrant. So the CRs propose to add the following,

pathlossReferenceIndex Indicates the reference signal used as PUSCH pathloss reference (see TS 38.213 [13], clause 7.1.1).

precodingAndNumberOfLayers Indicates the precoding and number of layers (see TS 38.212 [17], clause 7.3.1.1.2).

Q3: Do companies agree with the intention of CR?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| Nokia | Yes | Propose to merge this to rapporteur CR |

Summary:

## 3.4 Correction on T345 for UAI overheating

In the CRs R2-2204728/4729 [7][8], it is mentioned that the stop conditions for T345 specified in the table in section 7.1.1 are not aligned with the procedure text that is specified in section 5.3.7. To this end, it is proposed that the stop conditions for T345 specified in the table in section 7.1.1 shall be revised from “Upon releasing overheatingAssistance during the connection re-establishment procedure” to “Upon releasing overheatingAssistanceConfig during the connection re-establishment procedure”. More specifically,

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T345 | Upon transmitting UEAssistanceInformation message with overheatingAssistance | Upon releasing overheatingAssistanceConfig during the connection re-establishment procedure, upon initiating the connection resumption procedure, and upon receiving overheatingAssistanceConfig set to release. | No action. |

Q4: Do companies agree with the intention of CR?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| Nokia | Yes | Yes, minor but in fact brings clarity. Can be also captured in the rapporteur CR, as reflects the intended behaviour |

Summary:

## 3.5 Need code correction for ReferenceTimeInfo

In NR, upon receiving reference time information in DL information transfer or SIB9, the UE action is to deliver the time to the upper layer, i.e., one shot. However, the need code of referenceTimeInfo-r16 is currently set to Need R, which requires the UE to unnecessarily store the reference time which will be useless after delivering to the upper layer. Thus, the CRs R2-2205503/5504 suggest changing the need code from Need R to Need N in DLInformationTransfer and SIB9, as follows,

referenceTimeInfo-r16               ReferenceTimeInfo-r16               OPTIONAL,   -- Need N

Q5: Do companies agree with the intention of CR?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| Nokia | Neutral | Not sure what is the additional implication to UE to discard v/s store. If it is just a variable storage then maybe change is not so critical. Is there a functionality impact that requires this as some sort of essential correction? If not, then we don’t think any reason to change. |

Summary:

## 3.6 Correction on NR serving frequency results reporting

According to the sub-clause 5.5.5.1 in TS 36.331, for the event A3/A4/A5/B1-NR/B2-NR measurement, if the purpose field is not configured or set to reportLocation, the UE will not include NR serving frequency results. However, for the case that the purpose field is not configured (i.e. general measurements other than sidelink or sensing measurements), the NR serving cell results are also expected to be reported. Thus, to realize the NR serving cell results reporting when the purpose field is not configured, the CRs R2-2205298/5299/5300 suggest the following changes,

1> if the triggerType is set to event; and if the corresponding measObject concerns NR; and if eventId is set to eventB1-NR or eventB2-NR; or 1> if the triggerType is set to event; and if eventId is set to eventA3 or eventA4 or eventA5: 2> if purpose for the reportConfig or reportConfigInterRAT associated with the measId that triggered the measurement reporting is set to a value other than reportLocation or purpose is not configured: 3> set the measResultServFreqListNR to include for each NR serving frequency that the UE is configured to measure according to TS 38.331 [82], if any, the following: 4> set measResultSCell to include the available results of the NR serving cell, as specified in 5.5.5.2; 4> if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas and if eventId is set to eventA3 or eventA4 or eventA5: 5> set measResultBestNeighCell to include the available results, as specified in 5.5.5.2, of the non-serving cell with the highest sorting quantity determined as specified in 5.5.5.3; 3> for each (serving or neighbouring) cell for which the UE reports results according to the previous, additionally include available beam results according to the following: 4> if maxReportRS-Index is configured, set measResultRS-IndexList to include available results, as specified in 5.5.5.2, of up to maxReportRS-Index beams, ordered based on the quantity determined as specified in 5.5.5.3;

Q6: Do companies agree with the intention of CR?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| Nokia | Rel-15, NO unless there is an IODT issue which is there to clarify | We are a bit careful not to update Rel-15 Is this a real issue now coming from IODT? As it needs to be considered this is for Rel-15 apparently (old release) and in our understanding the change is not so fundamental that it would require to now put at risk existing Rel-15 implementations. If 'purpose not configured' is listed explicitly, is it changing something, from the procedural perspective? We think the reporting would happen also today, as 'no purpose' matches the case 'if report purpose is other than reportLocation' so maybe no issue to fix, in fact? We would like to first have common understanding of what the problem really is… |

Summary:

# 4 Conclusion

This discussion report is summarized with final proposals as follows,

# 5 Reference

- R2-2204902, Confirmation for inter-MN HO without SN change, NEC.

- R2-2205428, Correction on FR1-FR1power control parameters of NR-DC, CATT.

- R2-2205429, Correction on FR1-FR1power control parameters of NR-DC, CATT.

- R2-2204845, Correction on rrc-ConfiguredUplinkGrant in Rel-15, vivo.

- R2-2204846, Correction on rrc-ConfiguredUplinkGrant in Rel-16, vivo.

- R2-2205827, Correction on rrc-ConfiguredUplinkGrant in Rel-17, vivo.

- R2-2204728, Correction on T345 for UAI overheating, OPPO.

- R2-2204729, Correction on T345 for UAI overheating, OPPO.

- R2-2205503, Need code correction for ReferenceTimeInfo, Ericsson.

- R2-2205504, Need code correction for ReferenceTimeInfo, Ericsson.

- R2-2205298, Correction on NR serving frequency results reporting for event-triggered measurement (R15), Huawei, HiSilicon.

- R2-2205299, Correction on NR serving frequency results reporting for event-triggered measurement (R16), Huawei, HiSilicon.

- R2-2205300, Correction on NR serving frequency results reporting for event-triggered measurement (R17), Huawei, HiSilicon.