3GPP TSG-RAN WG2 Meeting #118electronic							R2-220xxxx

Online, May 9th – May 20th, 2022

Agenda Item:	6.1.3.2

Source: 	OPPO

Title:	[AT118-e][031][MBS] MAC (OPPO)

Document for:	Discussion and decision

- Introduction

This paper is to trigger the following email discussion of MAC open issues in MBS.

[AT118-e][031][MBS] MAC (OPPO)

Scope: Treat R2-2205483, R2-2205129, R2-2205122, R2-2204609, R2-2204833, R2-2205457, R2-2205218, R2-2205437, R2-2205447, R2-2205540, R2-2204667, R2-2204744, R2-2204832, R2-2204969, R2-2205156, R2-2205449, R2-2205035, R2-2205154, R2-2205480, R2-2204831, R2-2204834, R2-2204891, R2-2204904, R2-2204905, R2-2205628, R2-2205629, R2-2205673, R2-2205709, R2-2205713, R2-2205128, R2-2205481, R2-2205748

Collect one round of comments, pave the way for on-line agreement (identify agreeable points, discussion points),

Intended outcome: Report

Deadline: For online CB W1 Friday

Contact Information

| Company | Email |
| --- | --- |

- Discussion

2.1 Multicast

2.1.1 CSI-mask on CSI reporting for multicast

Currently, csi-Mask IE is configured per MAC entity.

MAC-CellGroupConfig ::= SEQUENCE { ==omit some IEs==== csi-Mask BOOLEAN ==omit some IEs==== }

If the drx-onDurationTimer is not running, UE configured with the csi-Mask cannot report CSI on PUCCH even if the drx-onDurationTimerPTM is running and some companies think it will impact the MBS data secheuling. So they propose that when allowCSI-SRS-Tx-MulticastDRX-Active and csi-Mask are configured, the UE does not report CSI on PUCCH when both drx-onDurationTimer and drx-onDurationTimerPTM are not running.

However, some companies have different view based on some reasons, e.g. for the purpose of CSI masking if all MBS DRX on duration are not overlapped, or any multiplexing of individual PUCCH resources linked to MBS DRX would not be possible in time domain alone, or no need to further increase the complexity.

One company think new configuration (i.e. multicast-CSI-mask) to control the CSI report on PUCCH only during the multicast DRX on duration.

Option 1: When allowCSI-SRS-Tx-MulticastDRX-Active and csi-Mask are configured, the UE does not report CSI on PUCCH when both drx-onDurationTimer and drx-onDurationTimerPTM are not running.

Option 2: CSI masking only considers unicast DRX, i.e. excludes MBS DRX (No spec change).

Option 3: New configuration (i.e. multicast-CSI-mask) to control the CSI report on PUCCH only during the multicast DRX on duration.

Q1: Which option do companies prefer?

| Company | Option 1/2/3? | Comments |
| --- | --- | --- |

2.1.2 DCP on CSI reporting for multicast

RAN2 assumed that DCP monitoring may be configured when multicast DRX is configured. First, RAN2 should confirm whether DCP monitoring can be configured with multicast DRX.

Q2: Do companies agree DCP monitoring can be configured with multicast DRX?

| Company | Yes/No? | Comments |
| --- | --- | --- |

It is common understanding that the DCP monitoring only affects whether drx-onDurationTimer is started, no impact on the starting of drx-onDurationTimerPTM.

Some companies think that to make multicast scheduling efficient, UE should be allowed to report CSI/SRS even the DCP conditions are satisfied if multicast DRX is in Active Time.

However, some companies have different view, e.g. how to reduce the impact of DCP on multicast DRX can be left to gNB implementation.

Option 1: If allowCSI-SRS-Tx-MulticastDRX-Active is configured, UE can report CSI/SRS even when the conditions for DCP and unicast DRX in TS 38321 are satisfied, if multicast DRX is in Active Time.

Option 2: How to reduce the impact of DCP monitoring on multicast DRX can be implemented by gNB without the spec impacts.

Q3: Which option do companies prefer?

| Company | Option 1/2? | Comments |
| --- | --- | --- |

Currently, IE allowCSI-SRS-Tx-MulticastDRX-Active is configured per MAC entity and one company think it should be configured per MBS DRX to achieve better power efficiency and scheduling flexibility.

Q4: Do companies agree IE allowCSI-SRS-Tx-MulticastDRX-Active is configured per MAC (no spec change), not configured per multicast DRX?

| Company | Yes/No? | Comments |
| --- | --- | --- |

2.1.3 Others on CSI reporting for multicast

Currently, if UE is configured with both secondary DRX group and allowCSI-SRS-Tx-MulticastDRX-Active, and if one DRX group is not in Active Time, only when all multicast DRXs are not in Active Time, UE does not report CSI in the DRX group.

Considering dual DRXs are configured and one is for FR1 and another is for FR2, one company propose if allowCSI-SRS-Tx-MulticastDRX-Active is configured, UE does not report CSI in a DRX group if unicast DRX and all multicast DRXs of the DRX group are not in Active Time.

Q5: Do companies agree the below proposal:

Proposal: If allowCSI-SRS-Tx-MulticastDRX-Active is configured, UE does not report CSI in a DRX group if unicast DRX and all multicast DRXs of the DRX group are not in Active Time.

| Company | Yes/No? | Comments |
| --- | --- | --- |

If DRX is not configured for some multicasts, only when all multicast DRXs are not in Active Time, UE does not report CSI in the DRX group. This will prevent the UE from reporting CSI report for the multicast service that is not configured with multicast DRX and affects the scheduling efficiency. One company propose if allowCSI-SRS-Tx-MulticastDRX-Active is configured, UE is allowed to report CSI if some of the multicasts are not configured with multicast DRX.

Q6: Do companies agree the below proposal:

Proposal: If allowCSI-SRS-Tx-MulticastDRX-Active is configured, UE is allowed to report CSI if some of the multicasts are not configured with multicast DRX.

| Company | Yes/No? | Comments |
| --- | --- | --- |

2.1.4 Multicast DRX related changes

Due to L1 PTP retransmission for the initial transmission of PTM transmission controlled by DCI, the MAC entity is required to start the corresponding drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDL and then stop both drx-RetransmissionTimerDL and drx-RetransmissionTimerDL-PTM

HARQ process is shared by unicast and multicast and one company propose to stop both drx-RetransmissionTimerDL and drx-RetransmissionTimerDL-PTM in section 5.7.

The corresponding TP is as follows:

When DRX is configured, the MAC entity shall: 1> if a MAC PDU is received in a configured downlink assignment: 2> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; NOTE 1a: If Serving cell is configured with downlinkHARQ-FeedbackDisabled and DL HARQ feedback is disabled, drx-HARQ-RTT-TimerDL is not started for the corresponding HARQ process. NOTE 1b: If this Serving Cell is part of a non-terrestrial network, the latest UE-gNB RTT value shall be used to set drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL length prior to timer start (see TS 38.331 [5] clause [X]). 2> stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process; 2> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. … 1> if a DRX group is in Active Time: 2> monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission; or 2> if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or 2> if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]: 3> start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; NOTE 3: When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback. 3> stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process(es) whose HARQ feedback is reported; 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported. 3> if the PDSCH-to-HARQ_feedback timing indicate an inapplicable k1 value as specified in TS 38.213 [6]: 4> start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process.

Q7: Do companies agree the below proposal and the above proposed changes?

Proposal: Stop both drx-RetransmissionTimerDL and drx-RetransmissionTimerDL-PTM in section 5.7 if multicast DRX is configured.

| Company | Yes/No? | Comments |
| --- | --- | --- |

In Nokia paper [R2-2205156], it clarifies in MAC spec section 5.7 that DRX Command MAC CE refers to DRX Command MAC CE with DCI scrambled with C-RNTI or CS-RNTI and configured downlink assignment does not include configured downlink multicast assignment.

Q8: Do companies agree the changes in section 5.7 proposed in annex of [R2-2205156]?

| Company | Yes/No? | Comments |
| --- | --- | --- |

Due to L1 PTP retransmission for the initial transmission of PTM transmission controlled by DCI, the MAC entity is required to start the corresponding drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDL. One company proposed that drx-HARQ-RTT-TimerDL is only started when the corresponding HARQ-FeedbackOptionMulticast is set to ack-nack and when DRX is configured.

Q9: Do companies agree the changes proposed in [R2-2204834]?

| Company | Yes/No? | Comments |
| --- | --- | --- |

One company think whether HARQ feedback is enabled has no impact on UE behavior of stopping the retransmission timers after receiving a DL multicast transmission and propose TP in section 5.7b.

Q10: Do companies agree the changes proposed in [R2-2205481]?

| Company | Yes/No? | Comments |
| --- | --- | --- |

2.2 Broadcast

2.2.1 Broadcast DRX related changes

In [R2-2205218], it proposed to add one note to highlight the timing for DRX duration calculation when SCell is configured for broadcast MBS reception.

NOTE X:	If a SCell is configured for MBS broadcast reception, the SFN of this SCell is used to calculate the DRX duration, otherwise the SFN of the SpCell is used.

Q11: Do companies agree the below proposal and the changes proposed in [R2-2205218]?

Proposal: If a SCell is configured for MBS broadcast reception, the SFN of this SCell is used to calculate the DRX duration, otherwise the SFN of the SpCell is used.

| Company | Yes/No? | Comments |
| --- | --- | --- |

2.2.2 HARQ process related changes for broadcast MBS

There is no NDI and HARQ process id in DCI for broadcast scheduling, there is repetition for MTCH according to the text of beam sweeping of MTCH like OSI. At the same time, RAN1 agree to use pdsch-AggregationFactor also for broadcast MBS scheduling.

In [R2-2205437/ R2-2204609/ R2-2204833], companies proposed to add text for HARQ process handling for broadcast MBS reception, but the wordings are different.

Which text do you preferred?

| Option 1 R2-2204609 | For each received TB and associated HARQ information, the HARQ process shall: 1> if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or 1> if the HARQ process is equal to the broadcast process, and this is the first received transmission for the TB according to the system information schedule indicated by RRC; or 1> if the HARQ process is associated with a transmission indicated with a MCCH-RNTI or a G-RNTI for MBS broadcast, and this is the first received transmission for the TB according to the MCCH or MTCH schedule indicated by RRC; or |
| --- | --- |
| Option 2 R2-2205437 | For each received TB and associated HARQ information, the HARQ process shall: 1> if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or 1> if the HARQ process is equal to the broadcast process, and this is the first received transmission for the TB according to the system information schedule indicated by RRC; or - if the HARQ process is associated with a transmission indicated with a MCCH-RNTI or a G-RNTI for MBS broadcast, and this is the first received transmission for the TB according to the scheduling indicated by DCI as specified in TS 38.214 [7]; or |
| Option 3 R2-2204833 | For each received TB and associated HARQ information, the HARQ process shall: 1> if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or 1> if the HARQ process is equal to the broadcast process, and this is the first received transmission for the TB according to the system information schedule indicated by RRC; or 1> if the HARQ process is allocated for the received TB for MCCH or broadcast MTCH, and this is the first received transmission for the TB according to the scheduling information indicated by RRC; or |

Q12: Do companies agree the changes and which text do companies prefer in [R2-2205437/ R2-2204609/ R2-2204833]?

| Company | Yes/No? | Comments |
| --- | --- | --- |

In [R2-2205437], company proposed MCCH should be readily identified with the MCCH-RNTI and be delivered to upper layers due to no multiplexing for MCCH and proposed the following text:

1> if the data for this TB was successfully decoded before: 2> if the HARQ process is equal to the broadcast process; or 2> if the HARQ process is associated with a transmission indicated with a MCCH-RNTI: 3> deliver the decoded MAC PDU to upper layers. 2> else if this is the first successful decoding of the data for this TB: 3> deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

Q13: Do companies agree the changes above proposed in [R2-2205437]?

| Company | Yes/No? | Comments |
| --- | --- | --- |

In [R2-2205457], company proposed to add text to clarify how to select HARQ process for MCCH/MTCH reception.

The number of parallel DL HARQ processes per HARQ entity is specified in TS 38.214 [7]. The dedicated broadcast HARQ process is used for BCCH. For MCCH or broadcast MTCH, the UE implementation selects an HARQ process other than the dedicated broadcast HARQ process.

Q14: Do companies agree the changes above proposed in [R2-2205457]?

| Company | Yes/No? | Comments |
| --- | --- | --- |

2.2.3 Other proposed changes

In [R2-2204606], company proposed to capture text for MTCH reception via beam sweeping in 38.321, not in 38.331.

Q15: Do companies agree the below proposal and agree the corresponding changes proposed in [R2-2204606]?

Proposal: Capture text for MTCH reception via beam sweeping in 38.321, not in 38.331.

| Company | Yes/No? | Comments |
| --- | --- | --- |

In [R2-2205218], company proposed one note in 5.9 to clarify that the SCell cannot be deactivated by MAC CE if the SCell is configured for broadcast reception.

NOTE X: The SCell configured for MBS broadcast reception cannot be deactivated via the SCell Activation/Deactivation MAC CE and Enhanced SCell Activation/Deactivation MAC CE.

Q16: Do companies agree the below proposal and the changes proposed in [R2-2205218]?

Proposals: The SCell configured for MBS broadcast reception cannot be deactivated via the SCell Activation/Deactivation MAC CE and Enhanced SCell Activation/Deactivation MAC CE.

| Company | Yes/No? | Comments |
| --- | --- | --- |

The following changes proposed in [R2-2204833].

When the MAC entity needs to read BCCH, the MAC entity may, based on the scheduling information from RRC: 1> if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the SI-RNTI; 2> indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity. When the MAC entity needs to read MCCH, the MAC entity may, based on the scheduling information from RRC: 1> if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the MCCH-RNTI; 2> indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity.

Q17: Do companies agree the changes proposed in [R2-2204833]?

| Company | Yes/No? | Comments |
| --- | --- | --- |

In [R2-2205447], company proposed the text in MAC reset section to excluding broadcast related timer and HARQ process handling. Do you agree the changes?

Q18: Do companies agree the changes proposed in [R2-2205447]?

| Company | Yes/No? | Comments |
| --- | --- | --- |

2.3 others

In [R2-2205122/ R2-2205129], companies proposed text to clarify discarding unexpected sub PDU for broadcast MBS reception. In previour MAC running CR discussion, most companies agreed to add text in secion 5.3.3, not 5.13. it is better not to open this discussion again, i.e. the yellow highlight text in 5.3.3 below will be kept.

5.3.3 Disassembly and demultiplexing The MAC entity shall disassemble and demultiplex a MAC PDU as defined in clauses 6.1.2 and 6.1.5a. When a MAC entity receives a MAC PDU for MAC entity's G-RNTI or G-CS-RNTI, or by the configured downlink assignment for MBS multicast containing an LCID or eLCID which is not configured, the MAC entity shall at least: 1> discard the received subPDU.

Due to L1 PTP retransmission for the initial transmission of PTM transmission, the UE may receive a MAC PDU scambmed with C-RNTI or CS-RNTI for retrsnamission of MBS multicast scampbed with G-RNTI or SPS. It is not clear how to handle this case and it is already captured in 5.13 for a error case.

5.13 Handling of unknown, unforeseen and erroneous protocol data When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing a Reserved LCID or eLCID value, or an LCID or eLCID value the MAC Entity does not support, the MAC entity shall at least: 1> discard the received subPDU and any remaining subPDUs in the MAC PDU. When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value which is not configured, the MAC entity shall at least: 1> discard the received subPDU.

Option 1: Due to L1 PTP retransmission for the initial transmission of PTM transmission, when UE receive a MAC PDU scambmed with C-RNTI or CS-RNTI for retrsnamission of MBS multicast scampbed with G-RNTI or SPS, UE discard the unexpected subPDU according to 5.13, i.e. no spec change.

Option 2: Due to L1 PTP retransmission for the initial transmission of PTM transmission, when UE receive a MAC PDU scambmed with C-RNTI or CS-RNTI for retrsnamission of MBS multicast scampbed with G-RNTI or SPS, UE discard the unexpected subPDU according to 5.3.3, i.e. add corresponding text for CS-RNTI and C-RNTI case in multicast reception in 5.3.3.

Q19: Which option do companies prefer and do companies agree the changes proposed in [R2-2205122] if option 2 is chosen?

| Company | Yes/No? | Comments |
| --- | --- | --- |

In [R2-2205483], company proposed to change the HARQ model for MCCH and broadcast MTCH in Figure 4.2.2-1 and Figure 4.2.2-2.

Q20: Do companies agree the changes proposed in [R2-2205483]?

| Company | Yes/No? | Comments |
| --- | --- | --- |

2.4 Any other issues?

Q21: Any other open issues?

| Company | Yes/No? | Comments |
| --- | --- | --- |

- Conclusions

Based on the discussion above, we propose:

- Reference

General

R2-2205483	Correction on the figures of MAC structure overview	Huawei, HiSilicon	CR	Rel-17	38.321	17.0.0	1272	-	F	NR_MBS-Core

R2-2205129	Handling of MAC PDU for MBS with Reserved LCID	ASUSTeK	discussion	Rel-17	38.321	NR_MBS-Core

R2-2205122	Clarification on MBS MAC subPDU discard	LG Electronics Inc., Nokia, Nokia Shanghai Bell	draftCR	Rel-17	38.321	17.0.0	F	NR_MBS-Core

Broadcast

R2-2204609	38321CR-Corrections on MCCH and MTCH reception	OPPO	CR	Rel-17	38.321	17.0.0	1225	-	F	NR_MBS-Core

R2-2204833	Correction on DL Data Transfer for MBS	vivo	discussion	Rel-17	NR_MBS-Core

R2-2205457	Clarification on the HARQ process used for broadcast MBS	Xiaomi Communications	draftCR	Rel-17	38.321	17.0.0	F	NR_MBS-Core

R2-2205218	[RIL406]The timing for broadcast DRX and SCell deactivation restriction	OPPO Beijing	CR	Rel-17	38.321	17.0.0	1263	-	F	NR_MBS-Core

R2-2205437	HARQ Process Handling for MBS Broadcast	Samsung R&D Institute India	discussion	Rel-17	38.321

R2-2205447	MBS Broadcast Retention	Samsung R&D Institute India	discussion	Rel-17	38.321

Multicast

R2-2205540	Remaining MBS user plane open issues	Intel Corporation	discussion	Rel-17	NR_MBS-Core

R2-2204667	Consideration on MAC Remaining Issues of MBS	CATT	discussion	Rel-17	38.323	NR_MBS-Core

R2-2204744	Corrections on MBS	Spreadtrum Communications	discussion	Rel-17

R2-2204832	Discussion on the Coexistence of DCP and Multicast DRX	vivo	discussion	Rel-17	NR_MBS-Core

R2-2204969	Remaining issues on MBS user plane	Lenovo	discussion	Rel-17

R2-2205156	DCP monitoring/WUS and MBS DRX and miscellaneous corrections to DRX	Nokia, Nokia Shanghai Bell	discussion	Rel-17	38.321	NR_MBS-Core

R2-2205449	WUS and DCP monitoring for MBS Multicast	Samsung R&D Institute India	discussion	Rel-17	38.321

R2-2205035	Discussion on CSI and SRS reporting issues	CMCC	discussion	Rel-17	NR_MBS-Core

R2-2205154	CSI Mask for MBS	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_MBS-Core

R2-2205480	Remaining issues on CSI reporting for multicast	Huawei, HiSilicon	discussion	Rel-17	NR_MBS-Core

R2-2204831	Discussion on CSI-mask Configuration with Multicast DRX	vivo	discussion	Rel-17	NR_MBS-Core

R2-2204834	Correction on Multicast DRX	vivo	discussion	Rel-17	NR_MBS-Core

R2-2204891	Discussion on the impact of CSI and SRS due to multicast DRX 	NEC Europe Ltd	discussion	Rel-17	NR_MBS-Core

R2-2204904	The timing for broadcast DRX and editorial corrections for multicast DRX	OPPO	CR	Rel-17	38.321	17.0.0	1241	-	F	NR_MBS-Core

R2-2204905	Corrections on CSI-mask and DCP coexistence for multicast DRX	MediaTek inc.	discussion	Rel-17	NR_MBS-Core

R2-2205628	CSI and SRS reporting in MBS DRX	ZTE, Sanechips	discussion	Rel-17	NR_MBS-Core

R2-2205629	Correction on CSI and SRS reporting for multicast DRX to 38321	ZTE, Sanechips	CR	Rel-17	38.321	17.0.0	1276	-	F	NR_MBS-Core

R2-2205673	Leftover issues on multicast DRX mechanism	Apple	discussion	Rel-17	NR_MBS-Core

R2-2205709	Discussion on CSI reporting due to multicast DRX	LG Electronics Inc.	discussion	Rel-17	NR_MBS-Core

R2-2205713	Remaining Issues on Multicast DRX	Samsung	discussion	Rel-17	NR_MBS-Core

R2-2205128	Discussion on unicast retransmission for MBS transmission	ASUSTeK	discussion	Rel-17	38.321	NR_MBS-Core

R2-2205481	Clarification on DRX timers for multicast	Huawei, HiSilicon	discussion	Rel-17	NR_MBS-Core

R2-2205748	Multicast and CSI, SRS and DCP	Ericsson	discussion	Rel-17	NR_MBS-Core