3GPP TSG-RAN WG2 Meeting #118e                                  R2-220xxxx

eMeeting May 9th – May 20th, 2022

Agenda Item:	7.2.2

Source: 	OPPO

Title:  	Report of [AT118-e][048][IoT-NTN] New Issues (OPPO) – 1st round

Document for:	Discussion and Decision

- Introduction

This document is to collect companies’ views for the following offline discussion focusing on UP issues.

[AT118-e][048][IOTNTN] New Issues (OPPO)

Scope: Treat R2-2204740, R2-2205725, R2-2204741.

Ph1 determine agreeable part, Ph2 endorse TP

Intended outcome: Report, Endorsed TP (if applicable)

Deadline: Schedule 1 (CB online W2 if needed)

- Contact information

| Company | Delegate contact |
| --- | --- |
| COMPANY_NAME | NAME (email@address.com) |

- Round-1 Discussion

3.1 Msg3 retransmission

In RAN2#117e, blind Msg3 retransmission has been agreed to be supported for NR NTN, which enables NW to schedule Msg3 retransmission during the UE-gNB RTT in case NW wants to improve the coverage through blind retransmission. See below chairman notes.

Agreements via email - from offline 103 - third round:

- Blind Msg3 retransmission is supported in Rel-17 NTN. FFS whether this is enabled by a NOTE (P4), or explicit configuration (P5a and P5b).

- The following NOTE is captured: “UE should attempt to re-aquire SIBxx prior to validity timer expiry by UE implementation.” Details of NOTE (potentially including additional clarification if needed) may be finalized in Stage 3. This is captured in RRC specification (e.g. Section 5.2.2.x)

It is proposed in both [1] and [2] that IoT NTN should align with NR NTN and support blind Msg3 retransmission.

Question 1: Do companies agree that blind Msg3 retransmission should be supported for IoT NTN, similar to NR NTN?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Regarding how to implement in the spec on blind Msg3 retransmission, following options are proposed in [1] and [2].

- Option 1: If mac-ContentionResolutionTimer expires during the UE-eNB RTT after Msg3 retransmission, (to wait for new CR timer restart) the UE does not consider the Contention Resolution unsuccessful.

- Option 2: If mac-ContentionResolutionTimer expires and no PDCCH addressed to TC-RNTI indicating uplink grant for a MSG3 retransmission is received after the start of the mac-ContentionResolutionTimer, the UE considers the Contention Resolution not successful

Note that the same issue is being discussed in NR NTN, i.e. phase 2 of [AT118-e][104]. Rapporteur assumes that IoT NTN can follow NR NTN’s conclusion.

Question 2: Do companies agree that IoT NTN follows NR NTN’s conclusion on how to implement blind Msg3 retransmission in the MAC spec (i.e. modifications to mac-ContentionResolutionTimer operation)?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

In [1], it is proposed to introduce an explicit configuration to support blind Msg3 retransmission in IoT NTN and UE behaviour would be different depending on whether blind Msg3 retransmission is configured or not. Note that this is also being discussed in NR NTN, i.e. phase 1 of [AT118-e][104] and rapporteur assumes that IoT NTN can follow NR NTN’s conclusion.

Question 3: Do companies agree that IoT NTN follows NR NTN’s conclusion on whether to introduce an explicit configuration to support blind Msg3 retransmission?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

3.2 TA report

Whether SR can be triggered?

In RAN2#117e, following agreement has been made for NR NTN.

- If a TA report is triggered and there are no available UL-SCH resources, the network may optionally configure UE to trigger an SR. A UE capability is introduced for this.

In [1], it is proposed that IoT NTN follows NR NTN’s agreements. Given that NR NTN agreements contain many aspects. Following questions are asked to see to which extend companies would like to align with NR NTN.

Question 4: Do companies agree that in IoT NTN, UE can trigger SR if a TA report is triggered and there are no available UL-SCH resources?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Question 5: Do companies agree that in IoT NTN, whether TA report can trigger SR is up to network’s configuration?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Question 6: Do companies agree that in IoT NTN, a UE capability is introduced for TA report triggering SR?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Whether configured uplink grant can be used to transmit TA report?

In the current TS 36.321, following is specified in 5.10.2 for NB-IoT.

For NB-IoT UEs, a configured uplink grant shall be used only for BSR or SPS confirmation transmission, and skipUplinkTxSPS is implicitly configured.

In [1], it is stated that TA report is also important for UL transmission, and proposed to allow a configured uplink grant to be used for TA report as well.

Question 7: Do companies agree that for NB-IoT over NTN, a configured uplink grant shall also be used for TA report?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

4. Round-1 summary

To be added…

5. References

- R2-2204740	Discussion on mac-ContentionResolutionTimer in IoT NTN	OPPO

- R2-2205725	Alignment with NR NTN for Msg3 blind retransmission	Nokia, Nokia Shanghai Bell

- R2-2204741	Discussion on TA report in IoT NTN	OPPO