3GPP TSG-RAN WG2 Meeting #118 electronic	R2-220

Online, May 9 – 20, 2022

Agenda Item:	6.17.3.1

Source:	Ericsson

Title:	[AT118-e][053][feMIMO] Discussion on H060

Document for:	Discussion, Decision

# Introduction

# Contact Information

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Ericsson | Helka-Liina Määttänen | Helka-liina.maattanen@ericsson.com |
| MediaTek | Li-Chuan TSENG | li-chuan.tseng@mediatek.com |
| LGE | SungHoon Jung | Sunghoon.jung@lge.com |
| Samsung | Seungri Jin | seungri.jin@samsung.com |
| ZTE | Fei Dong | Dong.fei@zte.com.cn |
| Huawei, HiSilicon | David Lecompte | david.lecompte@huawei.com |
| Vivo | Chenli | Chenli5g@vivo.com |
| CATT | Erlin Zeng | erlin.zeng@catt.cn |
| Apple | Fangli XU | fangli_xu@apple.com |
| OPPO | Zhongda Du | duzhongda@oppo.com |
| Nokia, Nokia Shanghai Bell | Tero Henttonen | tero.henttonen@nokia.com |

# PCI in resources in CSI-SSB-ResourceSet (H060)

The below contributions discuss the PCI associated to SSB indexes in same CSI-SSB-ResourceSet:

R2-2205916 M	[H060] Inter-cell beam measurement configuration	Huawei, HiSilicon

R2-2204599 M	Discussion on RILs:F001, F002, V101,V102,H059,H060, I105,V112,V109,I115,Z095	OPPO

The corresponding LS text is as follows:

Question 1.13: Should it be possible for different SSB indexes in the same CSI-SSB-ResourceSet to be associated with different additionalPCI?

Answer 1.13:

Yes, it should be possible that different SSB indexes in the same CSI-SSB-ResourceSet are associated with different additionalPCI.

The LS response says quite clearly that the different SSB indexes can be associated with different additionalPCI. The additionalPCI does not include serving cell PCI by definition.

additionalPCI

PCI of the additional SSB different from serving cell PCI.

Hence there is contradiction between RAN1 response to Ran2 and what RAN1 has specified in TS 38.214 refferred to in R2-2205916.

TS 38.214 V17.1.0, Clause 5.2.1.4.2

If the UE is configured with the higher layer parameter [NumberOfAdditionalPCI], the UE is allowed to report in a single reporting instance up to four SSBRIs for each report setting, where SSB resources are associated with PCI indices referring to the PCI of the serving cell and PCI(s) different from the PCI of the serving cell within the set of PCIs configured.

Hence RAN2 should discuss what to do. R2-2205916 suggests TP as copied also in Annex of this document to make changes according to TS 38.214 while R2-2204599 suggests not to agree on this TP.

Q2: Please give your view whether

- RAN2 should agree on TP in R2-2205916(also in Annex here)

- LS to RAN1 is needed and what should be the content

| Company | - Agree the TP | - Views whether LS to RAN1 is needed and what should be the content |
| --- | --- | --- |
| Ericsson | TP is not necessary | To inform RAN1 RAN2 has specified the feature according to information received from RAN1 |
| Intel | Intention of TP is ok. But think that we can still make serving cell ID absent by changing structure of servingAdditionalPCIList-r17 ? | Our RAN1 believes that there is no restriction discussed in RAN1 for SSB and thus, it should be possible to associate with serving cell ID. If majority of companies are not confident, we would be ok to send LS to RAN1. |
| MediaTek | The intention of TP is correct. But currently the value of AdditionalPCIIndex-r17 starts from 1, does this already imply that the value ‘0’ means the serving cell? | We may inform RAN1 that current structure can support the case mentioned. |
| LGE | We agree with the intention of TP, and Intel’s suggestion is also fine. So, we can either take a) the TP or b) change the structure of additionalPCIList-r17 to make absence of the field mean a serving cell. No strong preference between them. | We can simply inform both the current structure and alternative and ask their preference, so that we can just take the RAN1 preference. |
| Samsung | Same vie with Intel and LG | Same view with MediaTek, if RAN1 have concerns on the LS we think RAN1 will point out the view on our structure. |
| ZTE | WE agree with the intention of TP. Just have a suggestion on ‘ServingAdditionalPCIIndex-r17’ in the TP. In the TP, the value 0 of the servingAdditionalPCIIndex-r17 is to represent the serving cell the CSI-SSB-ResourceSet belong to,and other values are to represent the indicated non-serving cell. That’s why we also need change the value range of the additionalPCIIndex in SSB-MTC from {0, maxNrofAdditionalPCI-r17-1} to {1, maxNrofAdditionalPCI-r17} For minimizing the impact from the correction, we suggest to use the value maxNrofAdditionalPCI-r17 of servingAdditionalPCIIndex-r17 to represent the serving cell the CSI-SSB-ResourceSet belong to, then the additionalPCIIndex can be kept as it is. | We are not sure whether the LS is needed since the change is caused by the answer from a question we asked them. If companies think we still have two interpretations as we discussed during online, we can send it to RAN1 for confirmation. Otherwise, there is no need for us to send the LS. |
| Huawei, HiSilicon | Yes (proponent). With respect to Intel's suggestion: we need a way to know which PCI (serving cell PCI or additional PCI) corresponds to which entry in csi-SSB-ResourceList. One way is to have the same number of entries, which is what is done in the TP. If we want to indicate the additionalPCI only for entries that don't use the serving cell PCI, we need to have a list of (entry number, AdditionalPCIIndex). Both work, we have no preference. With respect to ZTE's suggestion: if the number of additional PCI would need to be extended in a later release, using maxNrofAdditionalPCI-r17 to mean the serving cell PCI seems not a great idea. | We don't see the need for a LS to RAN1 but can send one if that is the general preference. If we send a LS to RAN1, the LS should describe the feature in an English sentence (not rely on an ASN.1 TP which is not RAN1's expertise). So the LS could say: "RAN2 assume that a CSI SSB resource set can include CSI SSB resources associated with the serving cell PCI and CSI SSB resources associated with additional PCIs. Please indicate if this is not RAN1's intention." |
| vivo | We agree with the intention of TP, and also the Intel’s suggestion. What we understand is it should be possible to associate with serving cell. | It is better to send LS to RAN1 to confirm our understanding. If companies donot agree with the TP, we are also fine to check with RAN1 which is the correct understanding. |
| CATT | We agree with the intention of the TP. | We agree with HW’s TP. If R2 could agree on this, then no need to send LS to R1. Besides, we do not see any inconsistence btw the previous R1 reply LS and R1 agreemeent/spec. Question 1.13: Should it be possible for different SSB indexes in the same CSI-SSB-ResourceSet to be associated with different additionalPCI? Answer 1.13: Yes, it should be possible that different SSB indexes in the same CSI-SSB-ResourceSet are associated with different additionalPCI. The R1 answer in 1.13 is only talking about “different additionalPCI” because the question was formulated that way. |
| Apple | Agree with the intention and fine with the TP. | It’s no need to send LS to RAN1, since the change is just based on RAN1 feedback. |
| OPPO | After further checking with our RAN1 colleague we now also intend to agree the intention of TP | No LS to RAN1 and RAN2 can further discuss the detail of the TP |
| Nokia, Nokia Shanghai Bell | Agree with the intent of the TP. Note also that CSI-SSB-ResourceSet is meant for a single serving cell, and RRC states the following: #### – CSI-SSB-ResourceSet The IE CSI-SSB-ResourceSet is used to configure one SS/PBCH block resource set which refers to SS/PBCH as indicated in ServingCellConfigCommon. Hence, since additionalPCI is not part of ServingCellConfigCommon, something needs to be clarified in RAN2 anyway. | Overall agree with Huawei. But before sending LSs to RAN1, let's agree what could be done in RAN2. the intent is clear in that it has to be possible to have SSBs indicated for both serving cell and additional PCI, the rest is just a matter of finding the right place for configuration. |
| Xiaomi | Agree with the intention of the TP It seems that RAN1 has not discussed the reporting details. We can accept to have the flexible ASN.1 structure as proposed so as to facilitate the ASN.1 frozen, but we should allow future update in the field description depending on the RAN1 feedback. | Depending on how fast RAN1 can reply to us, we think that we should probably adopt the flexible RRC ASN. structure as given in the Annex, to allow both serving cell PCI and additional PCI included in the same resource set. The field description can be updated based on the RAN1 feedbacks. For example, we need to know whether the serving cell PCI is always included in the reporting instance when the additional PCI is configured. |

# Conclusion

In the previous sections we made the following observations:

- Annex: Text Proposal to TS 38.331

#### - CSI-SSB-ResourceSet

The IE CSI-SSB-ResourceSet is used to configure one SS/PBCH block resource set which refers to SS/PBCH as indicated in ServingCellConfigCommon and ServingCellConfig.

CSI-SSB-ResourceSet information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCESET-START

CSI-SSB-ResourceSet ::=             SEQUENCE {

csi-SSB-ResourceSetId               CSI-SSB-ResourceSetId,

csi-SSB-ResourceList                SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...,

[[

servingAdditionalPCIList-r17           SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF  ServingAdditionalPCIIndex-r17    OPTIONAL  -- Need R

]]

}

ServingAdditionalPCIIndex-r17  ::=  INTEGER(0..maxNrofAdditionalPCI-r17)

-- TAG-CSI-SSB-RESOURCESET-STOP

-- ASN1STOP

CSI-SSB-ResourceSet field descriptions

servingAdditionalPCIList Indicates the physical cell IDs (PCI) of the SSBs in the csi-SSB-ResourceList. If present, the list has the same number of entries as csi-SSB-ResourceList and the first entry of this list indicates the value of the PCI for the first entry of csi-SSB-ResourceList, the second entry of this list indicates the value of the PCI for the second entry of csi-SSB-ResourceList, and so on. For each entry; - if the value is zero, the PCI is the PCI of the serving cell in which this CSI-SSB-ResourceSet is defined; - otherwise, the value is additionalPCIIndex-r17 of an SSB-MTC-AdditionalPCI-r17 in the additionalPCIList-r17 in ServingCellConfig, and the PCI is the additionalPCI-r17 in this SSB-MTC-AdditionalPCI-r17.

#### –	SSB-MTC

The IE SSB-MTC is used to configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs.

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

SSB-MTC2 ::=                        SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need M

periodicity                         ENUMERATED {sf5, sf10, sf20, sf40, sf80, spare3, spare2, spare1}

}

SSB-MTC2-LP-r16 ::=                 SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need R

periodicity                         ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

SSB-MTC3-r16 ::=                    SEQUENCE {

periodicityAndOffset-r16            CHOICE {

sf5-r16                                     INTEGER (0..4),

sf10-r16                                    INTEGER (0..9),

sf20-r16                                    INTEGER (0..19),

sf40-r16                                    INTEGER (0..39),

sf80-r16                                    INTEGER (0..79),

sf160-r16                                   INTEGER (0..159),

sf320-r16                                   INTEGER (0..319),

sf640-r16                                   INTEGER (0..639),

sf1280-r16                                  INTEGER (0..1279)

},

duration-r16                        ENUMERATED {sf1, sf2, sf3, sf4, sf5},

pci-List-r16                        SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,  -- Need M

ssb-ToMeasure-r16                   SetupRelease { SSB-ToMeasure }                                          OPTIONAL   -- Need M

}

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need R

offset-r17                   INTEGER (0..159)

}

-- Editor's note: UE assistance information for SMTC/MG could be captured, and the content is FFS

SSB-MTC-AdditionalPCI-r17 ::=       SEQUENCE {

additionalPCIIndex-r17              AdditionalPCIIndex-r17,

additionalPCI-r17                   PhysCellId,

periodicity-r17                     ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,  -- Need S

ssb-PositionsInBurst-r17            CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

},

ss-PBCH-BlockPower-r17              INTEGER (-60..50)

}

--Editor's note: more RAN1 input may be coming for this IE

AdditionalPCIIndex-r17  ::=  INTEGER(1..maxNrofAdditionalPCI-r17)

-- TAG-SSB-MTC-STOP

-- ASN1STOP