3GPP TSG-RAN WG2 #118                                                                                         R2-220xxxx

eMeeting, 09th May – 20th May, 2022

Agenda Item:	    6.22.3.3

Source: 	    Apple

Title:		    Summary of [AT118-e][061][MGE] Network Configured Small Gaps (Apple)

Document for:	    Discussion and decision

# 1 Introduction

This is the summary of the following email discussion. R2-2205692 will be handled in offline #059 as a follow-up discussion thus is removed from here.

[AT118-e][061][MGE] Network Configured Small Gaps (Apple)

Scope: Progress remaining issues and attempt to converge. Treat R2-2204545, R2-2205727, R2-2205692, R2-2206070, R2-2206071.

Intended outcome: Report with agreements, TP if needed.

Deadline: CB W2 TUE

# 2 Discussion

## 2.1 On deriveSSB-IndexFromCellInter [R2-2204545][R2-2205727]

The relationship between deriveSSB-IndexFromCellInter and legacy deriveSSB-IndexFromCell was touched in last RAN2 meeting without achieving a conclusion. Two options were mentioned in last RAN2 meeting. It was also raised up in RIL Z142.

- Option 1: When deriveSSB-IndexFromCellInter is included, the network must set legacy deriveSSB-IndexFromCell IE to true;

- Option 2: UE ignores legacy deriveSSB-IndexFromCell IE once deriveSSB-IndexFromInter is received.

There are two different kinds of change in the reference contributions.

Option 1: Change in R2-2205727

Proposal 1: For an MO, when deriveSSB-IndexFromCellInter is included, the network should set legacy deriveSSB-IndexFromCell IE to true.

Proposal 2: RAN2 to update the field description of deriveSSB-IndexFromCellInter.

If this field is present, UE assumes SFN and frame boundary alignment between the reference serving cell indicated by ServCellIndex and all neighbour cells in this MeasObjectNR as specified in TS 38.133 [14]. This field also indicates that the UE can utilize the timing of the reference serving cell indicated by ServCellIndex to derive the index of SS block transmitted by all neighbour cells with same frequency as this MeasObjectNR. The neighbour cell(s) is on a frequency different than serving cell frequency from the reference serving cell. When this field is included, the network should set deriveSSB-IndexFromCell to true.

Option 2: Change in R2-2204545

deriveSSB-IndexFromCellInter If this field is present, UE assumes SFN and frame boundary alignment between the reference serving cell indicated by ServCellIndex and all neighbour cells in this MeasObjectNR as specified in TS 38.133 [14]. This field indicates whether the UE can utilize the timing of the reference serving cell indicated by ServCellIndex to derive the index of SS block transmitted by all neighbour cells with same frequency as this MeasObjectNR. In addition, the field also indicates whether the UE may use the timing of any detected cell on that target frequency to derive the SSB index of all neighbour cells on that frequency. When this field is configured, the UE ignores the deriveSSB-IndexFromCell.

Question 1: Which option is preferred by companies?

| Company | Option preferred | Comments |
| --- | --- | --- |
| MediaTek | Option 1 with comment | We think adding the last sentence “ When this field is included, the network should set deriveSSB-IndexFromCell to true.” is enough. Not sure why we need “ The neighbour cell(s) is on a frequency different than serving cell frequency from the reference serving cell.” |

## 2.2 Corrections on MGTA and MGL [R2-2206070]

The CR in R2-2206070 have several changes to MGTA/MGL and are addressing RIL(s) [H804][H805][H806].

Reason for changes:

- [H804] As metioned in LS from RAN4 (R4-2206890), the mgta = 0.25ms should not be used for NCSG. - [H805] There are some typos in the MeasGapConfig IE, i.e. “nscg” should be “ncsg”. - [H806] The mgta-r17 and mgl-r17 fields are adding new values for an existing filed, so “-v17xy” should be used instead of “-r17”. The following was agreed in the RAN2 ASN.1 AdHoc meeting: H020 Suffix v1700 or r17 ConfiguredGrantConfig: noOfHARQ-ProcessesExt-r17 [Description]: This extends an existing field, so the suffix should be v1700 [Proposed Change]: Change the suffix to v1700. DISCUSSION - Ericsson think we havent been completely consistent, have a weak preference for removing the “Ext” and using the -v1700. Intel support to remove the Ext. Remove the “Ext”, and use -v1700 (NCE with only new values) and apply this consistently. 4) The values in mgta-r17 IE and mgl-r17 IE can be only used for NCSG, which should be embodied in the spec.

Summary of changes:

- Add clarfication that mgta=0.25ms cannot be configured to NCSG in field descriptions of mgta IE. - Change “nscg” to “ncsg” in the MeasGapConfig IE. - Change mgta-r17 and mgl-r17 to mgta-v1700 and mgl-v1700 respectively. - Add conditions to mgta-r17 IE and mgl-r17 IE.

Question 2: Do companies agree the changes in the CR?

| Company | Yes / No on changes 1) – 4) | Comments |
| --- | --- | --- |
| MediaTek | 1) - Yes 2) – Already agreed 3)– Already rejected 4) – Yes with comment | For 2), it is already agreed in P1 of R2-2205220. For 3), it is already rejected by P2 of R2-2205220. Also, it is irreverent if we adopt E033/E034 now. For 4), Instead of conditional code, we suggest to describe this in field description based on the new IE structure proposed in R2-2205229. |

## 2.3 Correction on ncsgind [R2-2206071]

The CR in R2-2206071 is to capture the statement in RAN4 LS [R4-2206890] that “NCSG for CSI-RS based inter-frequency measurement with gap is not supported in R17”.

The change is copied below for reference:

nscgInd Indicates that the measurement gap is a NCSG as specified in 38.133 [14]. This field is not present if the measurement gap is used for CSI-RS based inter-frequency measurements.

Question 3: Do companies agree with the change above?

It is also fine to add the comments into the CR.

| Company | Yes / No | Comments |
| --- | --- | --- |
| MediaTek | See comment | We understand the intention but does not really think this change is needed. Isn’t already from capability part that the NCSG is used for SSB measurement only ? |

# 3 Conclusions

[TBA]

# 4 References

[1] R2-2204545	[Z142] Correction on deriveSSB-IndexFromCellInter field	ZTE Corporation, Sanechips	draftCR	Rel-17	38.331	17.0.0	F	NR_MG_enh-Core

[2] R2-2205727	[Z142]On relationship between deriveSSB-IndexFromCellInter and deriveSSB-IndexFromCell	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_MG_enh-Core

[3] R2-2206070	[H804][H805][H806] Corrections on mgta and mgl	Huawei, HiSilicon	CR	Rel-17	38.331	17.0.0	3156	-	F	NR_MG_enh-Core

[4] R2-2206071	[H807] Clarification on ncsgInd	Huawei, HiSilicon	CR	Rel-17	38.331	17.0.0	3157	-	F	NR_MG_enh-Core