3GPP TSG-RAN WG2 Meeting #118e                                             	 R2-2206425

E-Meeting, May 09th – May 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | -draft | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE capability bit to support 2 per-UE gap only for UE concurrent gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of UE capability to support two per-UE concurrent gap only |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding UE capability for support of two per-UE concurrent measurement gap |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Two per-UE concurrent measurement gap is not introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.306 CR XXX |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

### 6.3.3	UE capability information elements

<< OMMITED>>

#### –	MeasAndMobParameters

The IE MeasAndMobParameters is used to convey UE capabilities related to measurements for radio resource management (RRM), radio link monitoring (RLM) and mobility (e.g. handover).

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::=                    SEQUENCE {

measAndMobParametersCommon              MeasAndMobParametersCommon              OPTIONAL,

measAndMobParametersXDD-Diff                MeasAndMobParametersXDD-Diff        OPTIONAL,

measAndMobParametersFRX-Diff                MeasAndMobParametersFRX-Diff        OPTIONAL

}

MeasAndMobParameters-v1700 ::=          SEQUENCE {

measAndMobParametersFR2-2-r17           MeasAndMobParametersFR2-2-r17           OPTIONAL

}

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

ssb-RLM                                 ENUMERATED {supported}                  OPTIONAL,

ssb-AndCSI-RS-RLM                       ENUMERATED {supported}                  OPTIONAL,

...,

[[

eventB-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

handoverFDD-TDD                         ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting                     ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting                        ENUMERATED {supported}                  OPTIONAL

]],

[[

independentGapConfig                    ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport             ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2                         ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]],

[[

nr-CGI-Reporting-ENDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

eutra-CGI-Reporting-NEDC                ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting-NRDC                ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NEDC                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NRDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

reportAddNeighMeasForPeriodic-r16       ENUMERATED {supported}                  OPTIONAL,

condHandoverParametersCommon-r16        SEQUENCE {

condHandoverFDD-TDD-r16                  ENUMERATED {supported}              OPTIONAL,

condHandoverFR1-FR2-r16                  ENUMERATED {supported}              OPTIONAL

}                                                                               OPTIONAL,

nr-NeedForGap-Reporting-r16             ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-NRonly-r16          BIT STRING (SIZE (10))                  OPTIONAL,

supportedGapPattern-NRonly-NEDC-r16     ENUMERATED {supported}                  OPTIONAL,

maxNumberCLI-RSSI-r16                   ENUMERATED {n8, n16, n32, n64}          OPTIONAL,

maxNumberCLI-SRS-RSRP-r16               ENUMERATED {n4, n8, n16, n32}           OPTIONAL,

maxNumberPerSlotCLI-SRS-RSRP-r16        ENUMERATED {n2, n4, n8}                 OPTIONAL,

mfbi-IAB-r16                            ENUMERATED {supported}                  OPTIONAL,

dummy                                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NPN-r16                ENUMERATED {supported}                  OPTIONAL,

idleInactiveEUTRA-MeasReport-r16        ENUMERATED {supported}                  OPTIONAL,

idleInactive-ValidityArea-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-r16                ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NEDC-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NRDC-r16           ENUMERATED {supported}                  OPTIONAL,

pcellT312-r16                           ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-r16                 BIT STRING (SIZE (2))                   OPTIONAL

]],

[[

-- R4 19-2 Concurrent measurement gaps

concurrentMeasGap-r17                   CHOICE{

concurrentPerUE-OnlyMeasGap-r17			ENUMERATED {supported},

concurrentPerUE-PerFRCombMeasGap-r17	ENUMERATED {supported}

}                 OPTIONAL,

-- R4 19-1 Network controlled small gap (NCSG)

ncsg-MeasGap-r17                        ENUMERATED {supported}                  OPTIONAL,

ncsg-MeasGapEUTRAN-r17                  ENUMERATED {supported}                  OPTIONAL,

-- R4 19-3-2 pre-configured measurement gap

preconfiguredUE-AutonomousMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-3-1 pre-configured measurement gap

preconfiguredNW-ControlledMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2-2-r17                   ENUMERATED {supported}                  OPTIONAL,

handoverFR2-1-FR2-2-r17                 ENUMERATED {supported}                  OPTIONAL,

-- RAN4 14-1: per-FR MG for PRS measurement

independentGapConfigPRS-r17             ENUMERATED {supported}                  OPTIONAL

]]

}

<< OMMITED>>

END OF CHANGE