3GPP TSG RAN WG2 Meeting #118-e           		         		            R2-220xxxx

Electronic meeting, 9th – 20th May 2022

Agenda Item:	6.4.5

Source:	Intel Corporation (Rapporteur)

Title:	[AT118-e][069][eIAB] UE caps (Intel)

Document for:	Discussion and Decision

# Introduction

This document captures the following discussion:

[AT118-e][069][eIAB] UE caps (Intel)

Scope: Address the corrections / remaining issues from tdocs submitted under AI 6.4.5. 2. Progress UE caps draft CRs (38306, 38331). Identify new impact if any.

Intended outcome: Report (if needed), endorsed draft CRs (for merge with mega CRs

Deadline: CB W2 Wed (if needed), Endorsed Draft CRs ready at EOM

The discussion consists of two phases, Phase 1 and Phase 2, and the deadline of each phase is given below:

Phase 1: Deadline: Friday W1, 5:00pm UTC.

Address the corrections / remaining issues from tdocs submitted under AI 6.4.5.

Phase 2: Deadline: Wednesday W2, 10:00am UTC.

Review updated draft CRs for UE capabilities (38306, 38331).

Contact

To make it easier to find the correct contact delegate in each company for potential follow-up questions, the rapporteur encourages the delegates who provided input to provide their contacts information in this table:

| Company | Contact: Name (Email) |
| --- | --- |
| Intel (Rapporteur) | Ziyi.li@intel.com |

# Discussion

## Header rewriting based Re-routing IAB-MT capability

As discussed in [1], a single bit is agreed to be used as IAB-MT capability for BAP header rewriting based re-routing of all scenarios. [1] proposes with following changes by removing ‘/or’:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| bapHeaderRewriting-Rerouting-r17 Indicates whether the IAB-MT supports BAP header rewriting based re-routing, including inter-donor DU local re-routing and inter-donor CU re-routing, as specified in TS 38.340 [23]. | IAB-MT | No | No | No |

#### Q1. Do you agree with above change?

| Company | Y/N | Comment |
| --- | --- | --- |

## Header rewriting based Routing IAB-MT capability

As discussed in [2], BH RLF recovery under inter-donor CU scenario is missed in the field description of bapHeaderRewriting-Routing-r17, which also requires to perform BAP header rewriting. [2] proposes with following changes by adding ‘inter-donor CU RLF recovery’:

| bapHeaderRewriting-Routing-r17 Indicates whether the IAB-MT supports BAP header rewriting based inter-donor CU routing, including inter-donor CU partial migration, inter-donor CU RLF recovery, and inter-donor CU routing for topology redundancy, as specified in TS 38.340 [23]. | IAB-MT | No | No | No |
| --- | --- | --- | --- | --- |

#### Q2. Do you agree with above changes?

| Company | Y/N | Comment |
| --- | --- | --- |

## LCG Extention IAB-MT capability

As discussed in [2], extended BSR is also supported when IAB-MT supports LCG extension. The field description should be updated accordingly with following changes:

| lcg-ExtensionIAB-r17 Indicates whether the IAB-MT supports extended logical channel group and extended Buffer Status Report as specified in TS 38.321 [8]. | IAB-MT | No | No | No |
| --- | --- | --- | --- | --- |

#### Q3. Do you agree with above changes?

| Company | Y/N | Comment |
| --- | --- | --- |

# Conclusion

Based on the discussion above, the following is proposed:

# References

[1] R2-2204791, Correction on IAB-MT capability of header rewriting based re-routing (ZTE)

[2] R2-2205258, Corrections on the bapHeaderRewriting-Routing and lcg-ExtensionIAB for eIAB (Huawei)