3GPP TSG-RAN WG2 Meeting #118-e	(draft) R2-220xxxx

# Online, 09 – 20 May 2022

Agenda Item: 	8.19.2

Source: 			ZTE Corporation

Title: 	[AT117-e][118][CovEnh] MAC CR (ZTE)

Document for: 	Discussion and decision

- Introduction

This document summarizes the following offline discussion.

[AT118-e][118][CovEnh] MAC CR (ZTE)

Scope: Update MAC CR considering the submitted contributions

Intended outcome: Agreeable MAC CR

Deadline (for companies' feedback):  Thursday 2022-05-19 12:00 UTC

Deadline (for final CR in R2-2206412):  Friday 2022-05-20 08:00 UTC

In this offline document, we discuss the following contributions:

38.321 CRs

R2-2204739	Correction to 38.321 on redundancy version for Msg3 repetition	OPPO	CR	Rel-17	38.321	17.0.0	1227	-	F	NR_cov_enh-Core

- Continue in offline 118

R2-2205067	Clarification on Msg3 repetition RV determination to MAC spec	Huawei, HiSilicon	CR	Rel-17	38.321	17.0.0	1251	-	F	NR_cov_enh-Core

- Continue in offline 118

In addition, we will also discuss the potential MAC spec impact for supporting CE only BWP.

- Contact from companies

| Company | Email |
| --- | --- |

- Discussion

  - Redundancy version for Msg3 repetition

In current MAC spec, the redundancy version (RV) is applied on the nth transmission occasion within a bundle of dynamic grant or configure grant. In R2-2204739[1] and R2-2205067[2], companies pointed out the redundancy version for Msg3 repetition is agreed, and it is specified in RAN1 spec as follows, So [1][2] propose to update the MAC CR to also capture this scenario.

from TS 38.214 clause 6.1.2.1 For a PUSCH transmission scheduled by DCI format 0_1, or 0_2, or 0_0 with CRC scrambled by TC-RNTI, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to table 6.1.2.1-2. For a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to the first row of Table 6.1.2.1-2. Table 6.1.2.1-2: Redundancy version for PUSCH transmission rvid indicated by the DCI scheduling the PUSCH; rvid to be applied to nth transmission occasion (repetition Type A) or TB processing over multiple slots) or nth actual repetition (repetition Type B). ((n-(n mod N))/N) mod 4 = 0; ((n-(n mod N))/N) mod 4 = 0; ((n-(n mod N))/N) mod 4 = 0; ((n-(n mod N))/N) mod 4 = 0. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3.

Q1. Do company agree with the intention of CRs in [1][2]?

| Company | Yes or No | Comments |
| --- | --- | --- |

If answer “Yes” to Q1, regarding the MAC TP, the content of [1][2] are given as below:

from R2-2204739[1] #### 5.4.2.1 HARQ Entity ***omit non-related part*** For each transmission within a bundle of the dynamic grant or uplink grant received in a MAC RAR, the sequence of redundancy versions is determined according to clause 6.1.2.1 of TS 38.214 [7]. For each transmission within a bundle of the configured uplink grant, the sequence of redundancy versions is determined according to clause 6.1.2.3 of TS 38.214 [7]. from R2-2205067[2] #### 5.4.2.1 HARQ Entity ***omit non-related part*** For each transmission within a bundle of the dynamic grant, the sequence of redundancy versions is determined according to clause 6.1.2.1 of TS 38.214 [7]. For each transmission within a bundle of the configured uplink grant, the sequence of redundancy versions is determined according to clause 6.1.2.3 of TS 38.214 [7]. For each transmission within a bundle of the uplink grant received in a MAC RAR, the sequence of redundancy versions is determined according to clause 6.1.2.1 of TS 38.214 [7].

Q2. Which TP do you prefer, [1] or [2]? (Please elaborate your comment if you prefer other wording)

| Company | [1] or [2] | Comments |
| --- | --- | --- |

  - CE only BWP

Based on RAN1 feedback, RAN2 confirms the feasibility of supporting CE only BWP in R17, regarding how to specify CE only BWP and corresponding UE behaviour, it is now discussed under offline-103, and the following 3 options are provided.

Option 1: If the BWP selected for the Random Access procedure is only configured with CE RACH resources, the UE shall assume Msg3 repetition is applicable for the current Random Access resources as in R2-2205841 and R2-2205068; Option 2: If the BWP selected for the Random Access procedure is NOT configured with rsrp-ThresholdMsg3, the UE shall assume Msg3 repetition is applicable for the current Random Access resources as in R2-2205851; Option 3: If the BWP selected for the Random Access procedure is only configured with CE RACH resources, the network shall set the value of rsrp-ThresholdMsg3 to infinity as commented online.

Regarding above 3 options, Option 1 has MAC spec impact as pointed out in [3][4]. For Option 2, it also affect MAC spec, because the UE is not required to evaluate rsrpThresholdMsg3, but Option 3 has no MAC impact, because there is no special handling at UE side, just the rsrp-ThresholdMsg3 will be set to a very large value so the UE always meets the criterion.

However, besides offline-103, rapporteur noticed the similar issue is also discussed under offline-507 and offline-508 in RACH common session. So rapporteur suggests to wait for the progress in other offline discussion. No question on MAC TP regarding CE only BWP is provided in this version of document.

  - Other

Except redundancy version and CE-only BWP, any other changes to MAC spec?

| Company | Comments |
| --- | --- |

- Conclusions

TBD

- Reference

R2-2204739	Correction to 38.321 on redundancy version for Msg3 repetition	OPPO	CR	Rel-17	38.321	17.0.0	1227	-	F	NR_cov_enh-Core

R2-2205067	Clarification on Msg3 repetition RV determination to MAC spec	Huawei, HiSilicon	CR	Rel-17	38.321	17.0.0	1251	-	F	NR_cov_enh-Core

R2-2205841	CE RACH only BWP handling	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.0.0	1289	-	F	NR_cov_enh-Core

R2-2205068	Discussion on the leftover issues for CE-specific RACH	Huawei, HiSilicon	discussion	Rel-17	NR_cov_enh-Core