3GPP TSG-RAN WG2 #118-e	draft_Summary

Electronic meeting, 2022-05-09 - 2022-05-20

Agenda Item:	6.11.2.9

Source:	Ericsson

Title:	[AT118-e][623][POS] 38331 positioning CR (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

# 1	Introduction

This document is to collect comments for the CR:

- [AT118-e][623][POS] 38331 positioning CR (Ericsson)

Scope: Review and update the rapporteur CR, taking into account decisions of this meeting.  Discussion should coordinate with the handling of agenda item summaries.

Intended outcome: Agreeable CR

Deadline:  Tuesday 2022-05-17 1800 UTC

Further, there are RILs which need input.

R2-2205811	[RILE064] Moving TEG Reporting Configuration from SRS-Config to RRCReconfig	Ericsson	CR	Rel-17	38.331	17.0.0	3118	-	F	NR_pos_enh-Core

R2-2204998	[H568] Correction for periodic TEG reporting	Huawei, HiSilicon	CR	Rel-17	38.331	17.0.0	3025	-	F	NR_pos_enh-Core

R2-2205498	[E066] Correction on structure of UEPositioningAssistInfo message contents for reducing unnecessary data transmission	Samsung R&D Institute UK	discussion

R2-2205585	Discussion on positioning RRC ASN.1 issues	vivo	discussion	Rel-17	NR_pos_enh-Core

# 2	Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |
| Huawei, HiSilicon | Yinghao Guo (yinghaoguo@huawei.com) |
| CATT | Jianxiang Li (lijianxiang@catt.cn) |
| Samsung | June Hwang (june77.hwang@samsung.com) |

# 3	Comments

## 3.1 Moving TEG Reporting Configuration from SRS-Config to RRCReconfig

For RIL E064 R2-2205811

The CR provides the motivation as why TEG reporting should be decouple from SRS-Config and the corresponding changes.

Please provide the comments on the RIL E064 here:

| Company | CR Acceptable (Yes/No) | Comments |
| --- | --- | --- |
| Qualcomm | No | This has been discussed and agreed previously and keeping it in the SRS config looks more clear and simpler, since only applicable to SRS. |
| Huawei, HiSilicon | Yes | This report should be a per UE report. No need to configure it under BWP |
| CATT | Yes | Agree with Huawei |
| Apple | Yes |  |
| Xiaomi | Yes |  |
| vivo | Yes |  |
| Samsung | No | By quoting QC’s comment that this is already discussed and agreed, then we think keeping current structure doesn’t do harm, and also agree with that this is only for SRS configuration. |
| Ericsson | yes |  |

RIL E064 is agreed; i.e TEG reporting config is decoupled from SRS-Config

## 3.2 [H568] Correction for periodic TEG reporting

R2-2204998 provides the changes on how to capture the periodic TEG reporting. Pls check the current rapporteur CR (R2-22xxxxx RRC Positioning CR_v00) and express if any particular details from R2-2204998 is still needed.

| #### OPTION 1 #### 5.7.14.2 Initiation A UE capable of providing the association between UL SRS Resource for positioning and UE Tx TEG ID in RRC_CONNECTED may initiate the procedure upon being configured to provide this association information. Upon initiation of the procedure, the UE shall: 1> initiate transmission of the UEPositioningAssistanceInfo message in accordance with 5.7.14.3 to provide the association. #### 5.7.14.3 Actions related to transmission of UEPositioningAssistanceInfo message The UE shall set the contents of the UEPositioningAssistanceInfo message as follows: 1> if transmission of the UEPositioningAssistanceInfo message is initiated to provide the association between UL SRS Resource for positioning and Tx TEG according to 5.7.14.2; 2> if ue-TxTEG-RequestUL-TDOA-Config in SRS-Config IE is configured with periodicReporting; 3> include ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message on expiry of each configured period; 2> else if ue-TxTEG-RequestUL-TDOA-Config in SRS-Config IE is configured with oneShot: 3> include ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message only one time. | #### OPTION 2 #### 5.7.14.2 Initiation A UE capable of providing the association between UL SRS Resource for positioning and UE Tx TEG ID in RRC_CONNECTED may initiate the procedure upon being configured to provide this association information. Upon initiation of the procedure, the UE shall: 1> initiate transmission of the UEPositioningAssistanceInfo message in accordance with 5.7.14.3 to provide the association. 1> if the UE is configured with periodicReporting within the field ue-TxTEG-RequestUL-TDOA-Config: 2> start the periodical reporting timer with the value of periodicReporting as defined within the field ue-TxTEG-RequestUL-TDOA-Config The UE shall: 1> upon expiry of the periodical reporting timer: 2> initiate the reporting procedure for UE Tx TEG assocaition, as specified in 5.7.14.3. 2> re-start the periodical reporting timer with the value of periodicReporting as defined within the field ue-TxTEG-RequestUL-TDOA-Config. #### 5.7.14.3 Actions related to transmission of UEPositioningAssistanceInfo message The UE shall set the contents of the UEPositioningAssistanceInfo message as follows: 1> if oneShot is configured within the field ue-TxTEG-RequestUL-TDOA-Config; or 1> if periodicReporting is configured within the field ue-TxTEG-RequestUL-TDOA-Config and this is the first transmission of UEPositioningInfo message after initiation: 2> include in ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message the association between UL-SRS resources for positioning and the UE Tx TEG at the time when the configuration is received; 1> if periodicReporting is configured within the field ue-TxTEG-RequestUL-TDOA-Config and this is not the first transmission of UEPositioningInfo message after initiation: 2> include in the ue-TxTEG-AssocaitionList in the UEPositioningAssistanceInfo the associations between the UL-SRS resources for positioning and the UE Tx TEG since the last transmission of the UEPositioningAssistanceInfo. |
| --- | --- |

| Company | Which version is preferred (Option 1 or Option 2) | Comments |
| --- | --- | --- |
| Huawei, HiSIlicon | OPTION1 | 5.7.14.2 is used to capture the conditions for Initiation |
| CATT | Option 1 | No need to specify the report timer here. |
| Apple | Option 1, but see comments | There is an ongoing discussion in the other thread about periodic reporting, so need to check the status of that discussion before agreeing |
| Xiaomi | Option 1 | For the option 2 on 5.7.14.3, we think this is correct understanding, but it is not needed to capture it in the spec. |
| vivo | Option 1, see comments | ue-TxTEG-RequestUL-TDOA-Config will not be configured in SRS-Config IE if it is moved to RRCReconfiguration. |
| Samsung | Option 1 |  |

The procedure text for periodic TEG reporting does not need further change as suggested by RIL H568.

## 3.3 [E066] Correction on structure of UEPositioningAssistInfo

R2-2205498 provides option to have addMod and release list delta style code structure for signaling optimization of TEG reporting.

Companies are requested to provide their view.

Note: SRS-PosResourceSet is however has to be removed from TEG reporting based upon RAN1 LS.

E066 is not agreed; i.e delta signalling structure of TEG reporting is not introduced.

| Company | CR Acceptable (Yes/No) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | NO | AddModList is used for configuration. not for a UL RRC mesage |
| CATT | No | It will bring extra effort to UE to store the previous reports. |
| Apple | No |  |
| Xiaomi | No |  |
| vivo | No |  |
| Samsung | Yes | This optimization doesn’t have only cons as there must be a reduction of data waste for reporting repeated information. However if majority wants to keep the legacy style as commented above, i.e., UL RRC message doesn’t have delta style message etc, we are ok with not pursuing. |

## 3.4 Discussion on positioning RRC ASN.1 issues

R2-2205585 discuees the need of below two fields and proposes to add it.

UE-TxTEG-Association-r17 ::= 	       SEQUENCE {

ue-TxTEG-ID-r17              	       INTEGER (1.. maxUE-Tx-TEG-ID-r17),

absoluteFrequencyPointA                ARFCN-ValueNR,

offsetToPointA                         INTEGER (0..2199),

nr-TimeStamp-r17                            NR-TimeStamp-r17,

Proposal : Add the absoluteFrequencyPointA and offsetToPointA in UE-TxTEG-Association to provide frequency information about the SRS for positioning resources in multiple CCs.

Do companies agree to the proposal?

Serving cell ID is used for the frequency information for TEG reporting.

| Company | Proposal Acceptable (Yes/No) | Comments |
| --- | --- | --- |
| Qualcomm | Yes | This is also missing in current LPP. |
| Huawei, HiSIlicon | No | Serving cell id will be sufficient |
| CATT | Partial | offsetToPointA is not required. |
| Apple | Yes |  |
| Xiaomi | Yes | Fine to add the frequency information based on the RAN1 LS. |
| vivo | Yes, but | Share some sympathy with HW’s comment that cellIndex can be used to identify the frequency information, in this case, the gNB shall transfer it into specific frequency info of SRS when it retransmit the TEG report to LMF. |
| Samsung | Yes | We need to keep the RAN1 indicated information. |

# 4 Comments For Complete CR “R2-22xxxxx RRC Positioning CR_v00”

Please provide the comments on the CR here:

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | 1> else if cell reselection occurs when srs-PosRRC_InactiveConfig is configured: 2> indicate to the lower layer to stop srs-TimeAlignmentTimer transmission in RRC_INACTIVE; 2> release the srs-PosRRC-InactiveConfig. name of the timer is not correct Rapporteur: this has been addressed. |
| Qualcomm | DL-PPW-ID-r17 needs to be INTEGER (0..15). It is currently (0...3): "The maximum number of preconfigured PRS processing window per DL BWP is 4" The constant maxNrofPPW-Config can not be re-used for the PPW ID definition. The introduction text for DL-PPW-PreConfig is not fully correct, and only applicable to Type2.: #### – DL-PPW-PreConfig The IE DL- PPW-PreConfig specifies measurement window where a UE may receive data (PDCCH/PDSCH) and CSI-RS while also perform DL-PRS measurements in the configured window. This should be aligned with RAN1 agreements, e.g.: The IE DL-PPW-PreConfig specifies a measurement window where a UE is expected to measure the DL PRS, if it is inside the active DL BWP and with the same numerology as the active DL BWP, subject to the UE determining that the DL PRS priority is higher than that of the other DL signals or channels; otherwise, the UE is not expected to measure the DL PRS and is expected to receive other DL signals and channels. Rapporteur: this has been taken into account |
| CATT | 1. In section “4.2.1 UE states and state transitions including inter RAT” We think the bullet of “Transmission of SRS for Positioning” should be moved under “The UE” part, because this is only related to UE behaviour. Additionally, the “in RRC_INACTIVE” may be deleted. - RRC_INACTIVE: - … - Transmission of SRS for Positioning in RRC_INACTIVE. The UE: - Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5); - ... - If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data. - Transmission of SRS for Positioning. 2. In section “5.3.8.3 Reception of the RRCRelease by the UE” Expect for srs-PosConfig,there are also some other fields in srs-PosRRC-InactiveConfig, like BWP and RSRP change thresh information, which are also needed for SRS transmission. So we suggest to modify “srs-PosConfig” as “srs-PosRRC-InactiveConfig”. 2> if the srs-PosRRC-InactiveConfig is configured: 3> instruct MAC to start the srs-TimeAlignmentTimer and apply the configuration srs-PosRRC-InactiveConfigsrs-PosConfig; 3. In section “5.7.14.3 Actions related to transmission of UEPositioningAssistanceInfo message”. The sentence “The UE shall set the contents of the UEPositioningAssistanceInfo message as follows:” already implied that the transmission of the UEPositioningAssistanceInfo message is initiated. So similarly to RIL H568 and B101, we think “1> if transmission of the UEPositioningAssistanceInfo message is initiated to provide the association between UL SRS Resource for positioning and Tx TEG according to 5.7.14.2;” should be also deleted. The UE shall set the contents of the UEPositioningAssistanceInfo message as follows: 1> if transmission of the UEPositioningAssistanceInfo message is initiated to provide the association between UL SRS Resource for positioning and Tx TEG according to 5.7.14.2; 21> if ue-TxTEG-RequestUL-TDOA-Config in SRS-Config IE is configured with periodicReporting; 32> include ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message on expiry of each configured period; 21> else if ue-TxTEG-RequestUL-TDOA-Config in SRS-Config IE is configured with oneShot: 32> include ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message only one time. 4. In section “5.7.14.3 Actions related to transmission of UEPositioningAssistanceInfo message”. The procedure only described to “ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message”, however the purpose of this section is to transmit the UEPositioningAssistanceInfo message to network, so a step of send this message should be added. 1> if transmission of the UEPositioningAssistanceInfo message is initiated to provide the association between UL SRS Resource for positioning and Tx TEG according to 5.7.14.2; 2> if ue-TxTEG-RequestUL-TDOA-Config in SRS-Config IE is configured with periodicReporting; 3> include ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message on expiry of each configured period; 2> else if ue-TxTEG-RequestUL-TDOA-Config in SRS-Config IE is configured with oneShot: 3> include ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message only one time. 2> submit the UEPositioningAssistanceInfo message to lower layers for transmission, upon which the procedure ends. Rapporteur: This has been taken into account. |

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RIL E064 is agreed; i.e TEG reporting config is decoupled from SRS-Config

Proposal 2	The procedure text for periodic TEG reporting does not need further change as suggested by RIL H568.

Proposal 3	E066 is not agreed; i.e delta signalling structure of TEG reporting is not introduced.

Proposal 4	Serving cell ID is used for the frequency information for TEG reporting.