3GPP TSG RAN WG2#118-e	R2-220xxxx

e-Meeting, 9th - 20th May, 2022

Title:	[Draft] Reply LS to SA2 on Tx Profile

Response to:	Reply LS on Tx Profile (R2-2204525/ S2-2203595)

Release:	Rel-17

Work Item:	NR_SL_enh-Core, eV2XARC_Ph2, 5G_ProSe

Source:	vivo [to be RAN2]

To:	SA2

Cc:	CT1

Contact Person:

#### Name:	Jing Liang

E-mail Address:	liangjing@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN2 thanks SA2 for their reply LS on Tx Profile in R2-2204525/S2-2203595.

Regarding the questions from SA2, RAN2 would like to provide the following answers.

SA2 Question 1: Would this behaviour be compliant with RAN2's assumption for V2X, or would AS layer always expect a NR Tx Profile from V2X layer?

- the upper layer does not provide NR Tx Profile to the AS layer when there is no NR Tx Profile mapped for the relevant service. In this case, the AS layer can consider that SL DRX is not supported. How the AS layer operates in this case is up to RAN2.

[RAN2 answer] RAN2 discussed the issue and has made the following agreement:

RAN2 #118e Agreement: When the upper layer does not provide NR Tx Profile associated with an L2 ID to the AS layer, no SL DRX is applied for the L2 ID.

SA2 Question 2: Would the use of "default SL DRX configuration" also require the NR Tx Profile?

[RAN2 answer] RAN2 discussed the issue and has made the following agreement:

RAN2 #118e Agreement: For default SL DRX operation, SL DRX needs to be supported in the TX profile associated with service type/L2 id which the UE is interested to receive. No need of special TX profile only for a default SL DRX operation.

In addition to SA2’s questions, RAN2 also made the following agreements related to TX profile.

1. RAN2 agree to revert the following working assumptions: - “No additional RAN2 work if SA2 confirms it’s feasible for Rel-17 SL DRX operation, L2 id is only associated with either DRX-based TX profile(s) or non-DRX based TX profile(s)”. - “For GC, we will check with SA2 whether the mapping from L2 id to TX profile is feasible in the gNB (like what we did in LTE). Working assumption: no additional RAN2 work if SA2 confirms it’s feasible.” 2. RAN2 assumption: For a given L2 id, all TX and RX UEs should be configured with the same set of TX profile(s) (including DRX on/off). We need to check with SA2. 3. For GC, UE reports L2 id and SL DRX on/off indication to the gNB. 4. In case multiple TX profiles (w/ SL DRX and/or w/o SL DRX) are associated with an L2 ID, SL DRX is supported only when all TX profiles support SL DRX.

# 2. Actions:

To SA2

ACTION:  	RAN2 kindly asks SA2 to take the above agreements/assumption into account for further work and to provide feedbacks if any concern.

# 3. Dates of Next RAN2 Meetings:

3GPP RAN2#119e	August 22 – 26, 2022	[Electronic] Meeting