3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2207330

Elbonia, 17 – 26 August 2022

Agenda item:	8.8.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On subscription-based UAV UE identification

WID/SID:	NR_UAV - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In December 2021 RAN#94 has approved Rel-18 work on NR Uncrewed Aerial Vehicles (UAVs), see [1]. One of the objectives is defined as follows:

2. Specify the signaling to support subscription-based aerial-UE identification [RAN3/SA2 interaction/RAN2] Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

This paper is aimed at outlining our view regarding how to implement the objective quoted above.

# 2	Discussion

In this section we discuss how to ensure UE can be identified based on its subscription. It seems a lot has been done already in SA2. In [2], section 5.2.3.3.1 the following can be found:

| AerialUESubscriptionInfo | Aerial UE Subscription Information. It contains an Indication on whether Aerial service for the UE is allowed or not. |
| --- | --- |

This implies AM subscription data in Unified Data Manager (UDM) has AerialUESubscriptionInfo. This indication on whether aerial service is allowed for the UE or not is provided from UDM to Access Management Function (AMF) during registration. Thus, the only thing that seems to be missing is to send this information further from the AMF to gNB.

Observation 1: As per TS 23.502 AerialUESubscriptionInfo is provided from UDM to AMF.

Observation 2: Similar indication (AerialUESubscriptioInfo) should be provided from the AMF to the gNB.

In LTE Rel-15 a similar functionality has been defined. Aerial UE Subscription Information IE (with the possible values “allowed” or “not allowed”) has been included in the messages sent over S1 and X2 interfaces. For details, see the following excerpts from [3], while a similar definition can be also found in [4]:

### 9.2.129 Aerial UE subscription information This information element is used by the eNB to know if the UE is allowed to use aerial UE function, refer to TS 23.401[12]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Aerial UE subscription information; M; ENUMERATED (allowed, not allowed, ...).

In S1 this IE is included in INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST and PATH SWITCH REQUEST ACKNOWLEDGE.

In X2 this IE is included in HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE. In our understanding, reusing the LTE solution in NR shall be sufficient to address the associated objective from the WID [1]. However, a detailed work and final assessment on this matter shall be initiated in RAN WG3 in the fourth quarter of 2022.

Observation 3: LTE Rel-15 specifications (TS 36.413 and TS 36.423) comprise Aerial UE subscription information which is sent over X2 and S1 interface.

Proposal 1: The equivalent of LTE Rel-15 IEs defined for X2 and S1 interfaces is adopted by NR UAV WI. A detailed work is pursued in RAN3 starting from Q4 of 2022.

# 3	Conclusion

This paper elaborated on subscription-based aerial UE identification. The following observations and proposals have been made:

Observation 1: As per TS 23.502 AerialUESubscriptionInfo is provided from UDM to AMF.

Observation 2: Similar indication (AerialUESubscriptioInfo) should be provided from the AMF to the gNB.

Observation 3: LTE Rel-15 specifications (TS 36.413 and TS 36.423) comprise Aerial UE subscription information which is sent over X2 and S1 interface.

Proposal 1: The equivalent of LTE Rel-15 IEs defined for X2 and S1 interfaces is adopted by NR UAV WI. A detailed work is pursued in RAN3 starting from Q4 of 2022.

# References

- RP-213600 New WID on NR support for UAV (Uncrewed Aerial Vehicles) 3GPP TSG RAN Meeting #94e Electronic Meeting, Dec. 6 - 17, 2021

- 3GPP Technical Specification 23.502 Procedures for the 5G System (5GS), version 17.5.0, June 2022

- 3GPP Technical Specification 36.423 Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP), version 17.1.0, June 2022

- 3GPP Technical Specification 36.413 Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP), version 17.1.0, June 2022