3GPP TSG-RAN WG2 Meeting #119e	R2-2207411

# Electronic, 17th - 29th August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3260 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change request about Maximum PPW configuration value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN1 has specified below two restrictions when it comes to PPW configurations: - The maximum number of PPW configurations is 4 per DL BWP, - the maximum number of activated PRS processing windows across all active DL BWPs is 4 In 38.331, we have the following: DL-PRS-ProcessingWindowPreConfigAddModList-r17 ::= SEQUENCE (SIZE (1..maxNrofPPW-Config-r17)) OF DL-PRS-ProcessingWindowPreConfig-r17 DL-PRS-ProcessingWindowPreConfigReleaseList-r17 ::= SEQUENCE (SIZE (1..maxNrofPPW-Config-r17)) OF DL-PRS-ProcessingWindowPreConfig-r17. Where ‘DL-PRS-ProcessingWindowPreConfigAddModList-r17’ and ‘DL-PRS-ProcessingWindowPreConfigReleaseList-r17’ are defined per BWP-DownlinkDedicated. So then ‘maxNrofPPW-Config-r17’ is the maximum number of PPW configurations per DL BWP rather than the maximum number of activated PRS processing windows across all active DL BWPs. Hence, the TS 38.331 needs correction for the description of maxNrofPPW-Config The second restriction ‘maximum number of activated PRS processing windows across all active DL BWPs is 4’ has already been captured in TS 38.214 as shown below. Hence, the RAN2 specification does not need to capture this. The UE is not expected to be indicated with more than 4 activated PRS processing windows by higher layer parameter [PRSProcessingWindow] across all active DL BWPs and is not expected to be indicated with the activated PRS processing windows [PRSProcessingWindow] that overlap in time.. |
| Summary of change: |  | Revise the description of maxNrofPPW-Config from ‘Maximum number of activated PRS processing windows across all active DL BWPs’ to ‘Maximum number of Preconfigured PRS processing windows per BWP’. Impact Analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: PPW Configuration Inter-operability: No Inter-operability seen |  |  |  |  |  |  |  |  |
| In |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.4	RRC multiplicity and type constraint values

–	Multiplicity and type constraint definitions

maxNrofSPS-Config-r16                   INTEGER ::= 8       -- Maximum number of SPS configurations per BWP

maxNrofSPS-Config-1-r16                 INTEGER ::= 7       -- Maximum number of SPS configurations per BWP minus 1

maxNrofSPS-DeactivationState            INTEGER ::= 16      -- Maximum number of deactivation state for SPS per BWP

maxNrofPPW-Config-r17                   INTEGER ::= 4       -- Maximum number of Preconfigured PRS processing windows per BWP

maxNrofPPW-ID-1-r17                     INTEGER ::= 15      -- Maximum number of Preconfigured PRS processing windows minus 1

maxNrOfTxTEGReport-r17                  INTEGER ::= 256     -- Maximum number of UE Tx Timing Error Group Report