3GPP TSG-RAN WG2#119-e	R2-2208319

Online, 17-26 August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | 0104 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Positioning method support for L2 UE-to-network remote UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., CATT, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Supported positioning methods for a layer 2 UE-to-network remote UE may differ from supported positioning methods for a UE in direct Uu coverage. The methods that can be supported with a remote UE are not documented in 38.305. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A table of supported positioning methods is added to section 4.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec will not indicate positioning support for a remote UE, leading to potential uncertainty about what methods and versions should be expected, tested, and/or implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 CR 3245 TS 37.355 CR 0357 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.3.1	Introduction

The standard positioning methods supported for NG-RAN access are:

-	network-assisted GNSS methods;

-	observed time difference of arrival (OTDOA) positioning based on LTE signals;

-	enhanced cell ID methods based on LTE signals;

-	WLAN positioning;

-	Bluetooth positioning;

-	terrestrial beacon system (TBS) positioning;

-	sensor based methods:

-	barometric Pressure Sensor;

-	motion sensor.

-	NR enhanced cell ID methods (NR E-CID) based on NR signals;

-	Multi-Round Trip Time Positioning (Multi-RTT based on NR signals);

-	Downlink Angle-of-Departure (DL-AoD) based on NR signals;

-	Downlink Time Difference of Arrival (DL-TDOA) based on NR signals;

-	Uplink Time Difference of Arrival (UL-TDOA) based on NR signals;

-	Uplink Angle-of-Arrival (UL-AoA), including A-AoA and Z-AoA based on NR signals.

Hybrid positioning using multiple methods from the list of positioning methods above is also supported.

Standalone mode (e.g. autonomous, without network assistance) using one or more methods from the list of positioning methods above is also supported.

These positioning methods may be supported in UE-based, UE-assisted/LMF-based, and NG-RAN node assisted versions. Table 4.3.1-1 indicates which of these versions are supported in this version of the specification for the standardised positioning methods.

Table 4.3.1-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, LMF-based | NG-RAN node assisted | SUPL Note 8 |
| --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | Yes |
| OTDOA Note1, Note 2 | No | Yes | No | Yes |
| E-CID Note 4, Note 7 | No | Yes | Yes | Yes for E-UTRA |
| Sensor | Yes | Yes | No | No |
| WLAN | Yes | Yes | No | Yes |
| Bluetooth | No | Yes | No | No |
| TBS Note 5 | Yes | Yes | No | Yes (MBS) |
| DL-TDOA | Yes | Yes | No | Yes |
| DL-AoD | Yes | Yes | No | Yes |
| Multi-RTT | No | Yes | Yes | Yes |
| NR E-CID | No | Yes | Yes | Yes (DL NR E-CID) |
| UL-TDOA | No | No | Yes | Yes |
| UL-AoA | No | No | Yes | Yes |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: In this version of the specification only OTDOA based on LTE signals is supported. NOTE 3: Void NOTE 4: This includes Cell-ID for NR method when UE is served by gNB. NOTE 5: In this version of the specification only for TBS positioning based on MBS signals. NOTE 6: Void NOTE 7: Enhanced Cell ID based on LTE signals. NOTE 8: This shows whether the positioning method is supported by SUPL ULP [16]. |  |  |  |  |

Sensor, WLAN, Bluetooth, and TBS positioning methods based on MBS signals are also supported in standalone mode, as described in the corresponding clauses.

In NR access, operation as a Layer 2 UE-to-Network Remote UE is supported as defined in TS 38.331[14]. Table 4.3.1-2 indicates which positioning methods and versions are supported for a Layer 2 UE-to-Network Remote UE.

Table 4.3.1-2: Supported versions of NR positioning methods for L2 UE-to-Network Remote UE

| Method | UE-based (relay posSIBs/LPP PDU via PC5) | UE-assisted, LMF-based (relay posSIBs/LPP /RRC via PC5) | NG-RAN node assisted (relay RRC via PC5) | SUPL Note 1 |
| --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes (relay posSIBs/LPP PDU) | No | Yes |
| Sensor | Yes | Yes (relay posSIBs/LPP PDU) | No | No |
| WLAN | Yes | Yes (relay posSIBs/LPP PDU) | No | Yes |
| Bluetooth | No | Yes (relay posSIBs/LPP PDU) | No | No |
| DL-TDOA | Yes | Yes (relay posSIBs/LPP PDU) | No | Yes |
| DL-AoD | Yes | Yes (relay posSIBs/LPP PDU) | No | Yes |
| Multi-RTT | No | Yes (relay LPP PDU/RRC; periodic SRS only) | Yes (relay LPP PDU/RRC; periodic SRS only) | Yes |
| NR E-CID | No | Yes (relay LPP PDU) | Yes (relay RRC) | Yes (DL NR E-CID) |
| UL-TDOA | No | No | Yes (relay RRC; periodic SRS only) | Yes |
| UL-AoA | No | No | Yes (relay RRC; periodic SRS only) | Yes |
| NOTE 1: This shows whether the positioning method is supported by SUPL ULP [16]. |  |  |  |  |