3GPP TSG-RAN WG2 #119-e	R2-2208692

# Electronic, 17 - 26 Aug, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3288 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR of 38.331 on RRCReconfigurationCompleteSidelink message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR introduces changes to 38.331 on RRCReconfigurationCompleteSidelink message. Issue: SL DRX is an optional feature introduced in Rel.17, i.e., it may not be supported by all Rel.17 UEs. However, in RRCReconfigurationCompleteSidelink message, the field “sl-DRX-ConfigReject-r17” in “RRCReconfigurationCompleteSidelink-v1710-IE” is currently a mandatory field, which is not correct. Impact analysis Impacted functionality Sidelink RRC reconfiguration Inter-operability: - there is no inter-operability issue since the changes are only concerning UE behaviors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change “sl-DRX-ConfigReject-r17” to be an optional field |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Rel.17(+) UE does not know how to set “sl-DRX-ConfigReject-r17” if it does not support SL-DRX and it is unnecessary to have the field always present in the signaling even when SL-DRX is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unmodified parts omitted >

### 6.6.1	General message structure

< Unmodified parts omitted >

#### RRCReconfigurationCompleteSidelink

The RRCReconfigurationCompleteSidelink message is used to confirm the successful completion of a PC5 RRC AS reconfiguration. It is only applied to unicast of NR sidelink communication.

Signalling radio bearer: SL-SRB3

RLC-SAP: AM

Logical channel: SCCH

Direction: UE to UE

RRCReconfigurationCompleteSidelink message

-- ASN1START

-- TAG-RRCRECONFIGURATIONCOMPLETESIDELINK-START

RRCReconfigurationCompleteSidelink ::=         SEQUENCE {

rrc-TransactionIdentifier-r16                  RRC-TransactionIdentifier,

criticalExtensions                             CHOICE {

rrcReconfigurationCompleteSidelink-r16         RRCReconfigurationCompleteSidelink-r16-IEs,

criticalExtensionsFuture                       SEQUENCE {}

}

}

RRCReconfigurationCompleteSidelink-r16-IEs ::= SEQUENCE {

lateNonCriticalExtension                       OCTET STRING                                                       OPTIONAL,

nonCriticalExtension                           RRCReconfigurationCompleteSidelink-v1710-IEs                       OPTIONAL

}

RRCReconfigurationCompleteSidelink-v1710-IEs ::=   SEQUENCE {

sl-DRX-ConfigReject-r17                            ENUMERATED {true}                                              OPTIONAL,

nonCriticalExtension                               SEQUENCE {}                                                    OPTIONAL

}

-- TAG-RRCRECONFIGURATIONCOMPLETESIDELINK-STOP

-- ASN1STOP

RRCReconfigurationCompleteSidelink field descriptions

sl-DRX-ConfigReject Indicates the rejection of sidelink DRX configuration received from the peer UE for the corresponding NR sidelink unicast communication.