3GPP TSG-RAN WG2 Meeting #119 	R2-220

# Electronic August 2022

Title:	DRAFTLS on further questions on feMIMO RRC parameters

Response to: 	-

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	ERICSSON to be replaced by 3GPP TSG-RAN WG2

To:	3GPP TSG-RAN WG1

Contact Person:

#### Name:	Helka-Liina Määttänen

E-mail Address:	Helka-liina.maattanen@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 discussed the field description of addtionalPCI in TCI-state and cell in QCL-Info.

The current definition of TCI-state in the RRC specification is so that for each TCI-state, single additionalPCI  and two qcl-Types (i.e., qcl-Type1 and qcl-Type2) may be configured. For a qcl-Type, parameters such as cell, referenceSignal, etc., are configured.

The current field descriptions states:

additionalPCI

Indicates that this TCI state refers to an additional PCI different from serving cell PCI, as configured in ServingCellConfig.

cell

The UE's serving cell in which the referenceSignal is configured. If the field is absent, it applies to the serving cell in which the TCI-State is configured. The RS can be located on a serving cell other than the serving cell in which the TCI-State is configured only if the qcl-Type is configured as typeC or typeD. See TS 38.214 [19] clause 5.1.5.

Question 1

RAN2 would like to ask RAN1

- whether current field description of additionalPCI  is correct or whether the additionalPCI should refer to the “cell” configured in the QCL-info.

- RAN2 assumes additionalPCI is per TCI-state, i.e., there is no such case where qcl-Type1 and qcl-Type2 for the same TCI-state associate with different additionalPCI values. Please confirm whether this is also RAN1’s understanding.

- if b) is confirmed, would there be need to state that “cell” cannot be two different values for qcl-Type1 and qcl-Type2?

RAN2 also discussed the same matter for the IE TCI-UL-State with respect to the need to update field descriptions of  additionalPCI or the servingCellId or ul-powerControl

Question 2

RAN2 would like to ask RAN1

- whether the RAN2 assumption is correct?

- whether it is true that the additionalPCI  in IE TCI-UL-State is associated with the serving cell indicated by the field servingCellId?

- RAN2 would also like to confirm that qcl-Type is not applicable to reference signals configured in TCI-UL-State as the parameter excel R1-2202759 did not advice to include QCL Type for UL TCI state(row4)?

- If c) is true, it is assumed that QCL related limitations should be deleted from the field description of the servingCellId? That is, whether the restriction i.e. "The RS can be located on a serving cell other than the serving cell in which the TCI-State is configured only if the qcl-Type is configured as typeC or typeD. See TS 38.214 [19] clause 5.1.5." in the field description of servingCellId is applicable or not?

Pending part on offline discussion:

RAN2 also discussed about the configuration flexibility of the UL powercontrol.

In Rel-17 unified TCI framework, TCI-State (joint type) and TCI-UL-State-r17 (UL-only type) can be optionally configured with a set of power control parameters (ul-powerControl-r17). According to TS 38.331 V17.1.0, there are two possible configuration cases: a) ul-powerControl-r17 is present in BWP-UplinkDedicated and it is absent in all joint TCI states and UL TCI states, b) ul-powerControl-r17 is absent in BWP-UplinkDedicated and it is present in all joint TCI states and UL TCI states.

However, RAN1’s agreements do not exclude the case that ul-powerControl-r17 is present in some TCI states and is absent in other TCI states (case c)). In case c), ul-powerControl-r17 can be configured in both BWP-UplinkDedicated and joint TCI-State/TCI-UL-State-r17. When the indicated (currently used) TCI state is not configured with ul-powerControl-r17, the UE uses ul-powerControl-r17 in BWP-UplinkDedicated.

Question 3

RAN2 would like to ask RAN1 whether current specification is sufficient for UL powercontrol or whether further flexibility can be allowed

2. Actions:

To RAN1 group:

ACTION: 	RAN2 respectfully asks RAN1 to provide responses to above questions.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #119-e 	October 2022    Electronic

TSG-RAN WG2 Meeting #120 	November 2022    Europe