3GPP TSG-RAN WG2 Meeting #119 	R2-220

# Electronic August 2022

Title:	DRAFTLS on further questions on feMIMO RRC parameters

Response to: 	-

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	ERICSSON to be replaced by 3GPP TSG-RAN WG2

To:	3GPP TSG-RAN WG1

Contact Person:

Name:	Helka-Liina Määttänen

E-mail Address:	Helka-liina.maattanen@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

DL or joint TCI state configuration

RAN2 discussed the field description of additionalPCI in TCI-state (for Rel-17 joint or DL TCI states) and cell in QCL-Info in TCI-State

TCI-State ::=                       SEQUENCE {

tci-StateId                         TCI-StateId,

qcl-Type1                           QCL-Info,

qcl-Type2                           QCL-Info                             OPTIONAL,   -- Need R

...,

[[

additionalPCI-r17                   AdditionalPCIIndex-r17               OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r17          PUSCH-PathlossReferenceRS-Id         OPTIONAL,   -- Cond JointTCI

ul-powerControl-r17                 Uplink-powerControlId-r17            OPTIONAL    -- Cond JointTCI

]]

}

QCL-Info ::=                        SEQUENCE {

cell                                ServCellIndex                        OPTIONAL,   -- Need R

bwp-Id                              BWP-Id                               OPTIONAL, -- Cond CSI-RS-Indicated

referenceSignal                     CHOICE {

csi-rs                              NZP-CSI-RS-ResourceId,

ssb                                 SSB-Index

},

qcl-Type                            ENUMERATED {typeA, typeB, typeC, typeD},

...

}

The current definition of TCI-state in the RRC specification is so that for each TCI-state, single additionalPCI  and two qcl-Types (i.e., qcl-Type1 and qcl-Type2) may be configured. For a qcl-Type, parameters such as cell, referenceSignal, etc., are configured.

The current field descriptions states:

additionalPCI

Indicates that this TCI state refers to an additional PCI different from serving cell PCI, as configured in ServingCellConfig.

cell

The UE's serving cell in which the referenceSignal is configured. If the field is absent, it applies to the serving cell in which the TCI-State is configured. The RS can be located on a serving cell other than the serving cell in which the TCI-State is configured only if the qcl-Type is configured as typeC or typeD. See TS 38.214 [19] clause 5.1.5.

Question 1

RAN2 would like to ask RAN1

- RAN2 assumes additionalPCI is per TCI-state and refers to the configured reference signal in case of SSB. That is, there is no such case where qcl-Type1 and qcl-Type2 for the same TCI-state associate with different additionalPCI values. Please confirm whether this is also RAN1’s understanding.

- if a) is confirmed, would there be need to state that “cell” cannot be two different values for qcl-Type1 and qcl-Type2?

Question 2

RAN2 considers the case where a serving cell uses the TCI states defined in another cell, i.e. dl-OrJoint-TCIStateList is set to unifiedTCI-StateRef. and would like to ask RAN1:

- When “cell” is absent in QCL-info, is the referenceSignal located in the serving cell where the TCI-state is configured (dl-orJoint-TCI-State-ToAddModList is in IE PDSCH-Config of this serving cell) or in the serving cell where the TCI-state is used ( unifiedTCI-StateRef is in IE PDSCH-Config of this serving cell)? And is the above limited to certain qcl-Type?

- Is the configuration of the TCI state of the serving cell indicated by unifiedTCI-StateRef still applicable for the serving cell configured with unifiedTCI-StateRef when the serving cell (e.g. SCell) indicated by unifiedTCI-StateRef is deactivated?

UL TCI state configuration

RAN2 also discussed the IE TCI-UL-State with respect to the need to update field descriptions of additionalPCI or the servingCellId or ul-powerControl

TCI-UL-State-r17 ::=             SEQUENCE {

tci-UL-State-Id-r17              TCI-UL-State-Id-r17,

servingCellId-r17                ServCellIndex              OPTIONAL,   -- Need R

bwp-Id-r17                       BWP-Id                     OPTIONAL,   -- Cond CSI-RSorSRS-Indicated

referenceSignal-r17              CHOICE {

ssb-Index-r17                    SSB-Index,

csi-RS-Index-r17                 NZP-CSI-RS-ResourceId,

srs-r17                          SRS-ResourceId

},

additionalPCI-r17                AdditionalPCIIndex-r17                        OPTIONAL,   -- Need R

ul-powerControl-r17              Uplink-powerControlId-r17                     OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r17       PUSCH-PathlossReferenceRS-Id-r17              OPTIONAL,   -- Need R

...

}

additionalPCI

Indicates the physical cell IDs (PCI) of the SSBs.

servingCellId

The UE's serving cell in which the referenceSignal-r17 is configured. If the field is absent, it applies to the serving cell in which the TCI-UL-State is configured. The RS can be located on a serving cell other than the serving cell in which the TCI-State is configured only if the qcl-Type is configured as typeC or typeD. See TS 38.214 [19] clause 5.1.5.

ul-powerControl

Configures power control parameters for PUCCH, PUSCH and SRS for this TCI state. The field is present here only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of this serving cell.

Question 3

RAN2 would like to ask RAN1

- in case the servingCellId is present, does the additionalPCI in IE TCI-UL-State refer to one of additional PCIs configured in the serving cell indicated by the field servingCellId?

- is it correct that there is no qcl-Type  field in TCI-UL-State as the parameter list excel file in R1-2202759 did not advice to include QCL Type for UL TCI state(row4)?

- If b) is correct, it is assumed that QCL related limitations should be deleted from the field description of the servingCellId? That is, should. "The RS can be located on a serving cell other than the serving cell in which the TCI-State is configured only if the qcl-Type is configured as typeC or typeD. See TS 38.214 [19] clause 5.1.5." in the field description of servingCellId" be deleted?

Question 4

RAN2 considers the case where a serving cell uses the UL TCI states defined in another cell, i.e. ul-TCIStateList is set to unifiedTCI-StateRef. and would like to ask RAN1:

- When ‘servingCellId’ is absent in TCI-UL-State, is the referenceSignal configured in the serving cell where the TCI-UL-state is configured or in the serving cell where the TCI-ULstate is used (in case this serving cell is not directly configured with UL TCI states but is configured with parameter unifiedTCI-StateRef )?

UL power control

RAN2 also discussed about the configuration flexibility of the UL powercontrol.

In Rel-17 unified TCI framework, TCI-State (joint type) and TCI-UL-State-r17 (UL-only type) can be optionally configured with a set of power control parameters (ul-powerControl-r17). According to TS 38.331 V17.1.0, there are two possible configuration cases: a) ul-powerControl-r17 is present in BWP-UplinkDedicated and it is absent in all joint TCI states used together with this BWP-UplinkDedicated and UL TCI states used in this BWP-UplinkDedicated, b) ul-powerControl-r17 is absent in BWP-UplinkDedicated and it is present in all joint TCI states used together with this BWP-UplinkDedicated and UL TCI states used together with this BWP.

RAN1’s agreements do not exclude the case that ul-powerControl-r17 is present in some TCI states and is absent in other TCI states (case c)). In case c), ul-powerControl-r17 can be configured in both BWP-UplinkDedicated and joint TCI-State/TCI-UL-State-r17 and the UE uses ul-powerControl-r17 in BWP-UplinkDedicated only when the TCI state used is not configured with ul-powerControl-r17. However, this case is currently excluded by RAN2 specifications

Question 5

RAN2 would like to ask RAN1 whether current specification is sufficient for UL powercontrol or whether further flexibility, such as case c), should be supported

PH reporting

In TS 38.321 v17.1.0, the UE reports two Type 1 PH values or (one) Type 3 PH value for a serving cell if the serving cell is configured with mTRP PUSCH repetition and the MAC entity to which this serving cell belongs is configured with twoPHRMode. However, there are diverging understandings in RAN2 on this, so RAN2 would like to ask RAN1:

Question 6

- Does the UE RAN2 have correct understanding for PH report?

  - the UE provides two Type 1 PH value for the serving cell if there is actual or reference PUSCH transmission on both TRP in slot n.

  - the UE provides one Type 3 PH value for the serving cell if there is actual or reference SRS transmission in slot n.

- If a) is correct, in which case will the UE report type 3 PH value for this serving cell?

2. Actions:

To RAN1 group:

ACTION: 	RAN2 respectfully asks RAN1 to provide responses to above questions.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #119-e 	October 2022    Electronic

TSG-RAN WG2 Meeting #120 	November 2022    Europe