3GPP TSG-RAN WG2 Meeting #119 electronic		R2-220xxxx

Online, August 17 – August 26, 2022

Source: 	MediaTek Inc.

Title:  	Report of [AT119-e][004][ePowSav] Subgrouping/PEI (MediaTek)

Agenda Item:	6.9.2

Document for:	Discussion and decision

# Introduction

Companies’ contributions [3] – [21] were summarized in [22]. The summary [22] was treated online and we continue identifying agreeable parts and impacts based on online discussion progress.

This document is for the following offline email discussion:

[AT119-e][004][ePowSav] Subgrouping/PEI (MediaTek)

Scope: Based on online progress, discussion, R2-2208909 and referenced input, continue identify agreeable parts and impacts. No Need to include Stage-2 etc.

Intended outcome: Report (with agreements), offline if possible.

Deadline: W2 Wednesday (can CB W2 Thu if required)

# Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| MediaTek (Rapp) | Mutai Morton Lin | morton.lin@mediatek.com |
| Xiaomi | Yanhua Li | Liyanhua1@xiaomi.com |
| Qualcomm | Linhai He | linhaihe@qti.qualcomm.com |
| Futurewei | Yunsong Yang | yyang1@futurewei.com |
| ZTE | Fei Dong | Dong.fei@zte.com.cn |
| Intel Corporation | Seau Sian Lim | seau.s.lim@intel.com |
| CATT | Pierre Bertrand | pierrebertrand@catt.cn |
| vivo | Chenli | chenli5g@vivo.com |
| Nokia | Chunli Wu | Chunli.wu@nokia-sbell.com |

# Discussion

## UE_ID based subgrouping

It was agreed we go for solution in the contribution [18] in the Subgrouping/PEI online discussion of RAN2#119-e:

R2-2208609 38.304 Clarifications on SubgroupID for UE-ID based subgrouping Xiaomi, ZTE Corporation,Vivo, Ericsson, CATT draftCR Rel-17 38.304 17.1.0 NR_UE_pow_sav_enh-Core - Chair wonder if we can agree this. - Huawei think we should align solution with PO solution already in the TS. Nokia agrees with Huawei and think this proposal is better. - Xiaomi think HW way can also work, but may need to change the 38300 then. - Vivo think that we should not depend on UE capability, and just specify in the TS. Solution in this doc is agreed

Contribution [18] propose to explicitly clarify the same DRX cycle value is used for the subgroupID calculation in both RRC_IDLE and RRC_INACTIVE states.

Q1: Companies are invited to provide other comment (if any) for agreeing TP change in the CR [18].

| Company | Comments |
| --- | --- |

## PEI monitoring

PEI monitoring for RedCap

In RAN2#118-e, the Proposal 4 is discussed in [2] and all companies (10 out of 10) agreed that we need to move pei-SearchSpace-r17, firstPDCCH-MonitoringOccasionOfPEI-O-r17 to PDCCH-ConfigCommon of initialDownlinkBWP-RedCap-r17 and initialDownlinkBWP (already implemented in TS 38.331 v17.1.0).

Contribution [9] proposed to further clarify in description of pei-Config that this configuration is for PEI monitoring on initialDownlinkBWP and/or on initialDownlinkBWP-RedCap.

Q2: Do companies agree with the intention and TP of the CR [9]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Xiaomi | No | In the current spec, these parameters are configured commonly across BWPs not for per BWP. I do not think there is ambiguity here and would rather to keep as it is. Otherwise, if we introduce new initial DL BWP type, like for R18 eRedcap, we need to update the spec to capture the new initial DL BWP type here. |
| Qualcomm | Yes | We are fine with the proposed change. |
| Futurewei | Yes |  |
| ZTE | No | It seems not an essential CR since the presence condition of the pei-ConfigBWP has implied the intention of the CR, please see below: InitialBWP-Paging: his field is optionally present, Need R, if this BWP is the initialDownlinkBWP or initialDownlinkBWP-RedCap, and pei-Config is configured in DownlinkConfigCommonSIB. Otherwise this field is absen |
| Intel | Yes | Agree with the intention |
| CATT | No | It has already been clarified on the Cond InitialBWP-Paging of pei-ConfigBWP-r17, so it is not necessary. |
| vivo | No | Current text has no ambiguity. If further clarification is needed for pei-Config, then the PCCH-Config and BCCH-Config also need similar clarification. We think the change is not essential. |
| Nokia | No | No strong need. The configuration is cell specific anyway, so applicable to whichever BWP configured with PEI. It’s the same for all the other configurations. |

Also in RAN2#118-e, the aspects of paging and PEI monitoring for RedCap and TP were discussed in [2], most companies (7 out of 10) thought field description update could be continued in ePowSav WI and agreed considering the TP as baseline. (i.e. Proposal 5 in [2])

Contributions [11] and the 2nd change of [21] proposed almost identical TP for updating field description of InitialBWP-Paging thus rapporteur suggests treating them together.

Besides, in consideration of a notification from proponent to postpone discussion and to monitor the progress of relevant topic in RadCap WI discussion, rapporteur suggests companies can still gave view here meanwhile please the proponent to bring up-to-date info. from RedCap WI, because our agreement and the conclusion in RedCap session should harmonize. (In any case rapporteur do not want companies to have double effort on the same topic)

Q3: Do companies agree with the intention and TP of the CR [11] and the 2nd change of [21]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Xiaomi | Yes | Ok to postpone and discuss in Redcap WI. |
| Qualcomm | Yes | We are fine with the TP in [11] |
| Futurewei | Yes |  |
| ZTE | Yes | OK to postpone |
| Intel | Yes | The update to the condition looks ok in our view. |
| CATT | Yes | We agree with the intention but prefer to let it treated in RedCap session. |
| vivo | Yes | Proponent. But we are ok to postpone and discuss it in RedCap WI. |
| Nokia | Yes |  |

PEI reception during emergency session

Contribution [17] proposed to add a precondition emergency service is not ongoing (same wording as 38.331) to clarify PEI reception.

Q4: Do companies agree with the intention and TP of the CR [17]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Xiaomi | No? | The cover sheets quoted from SA2's LS that UE should not indicate its support of WUS Assistance Information during an attach for emergency. I think then the NW will not page the UE by PEI, right? And this issue is for R16 LTE. But I checked 36.304, and found there is no description on this issue. Why we need to capture this in NR? |
| Qualcomm | Yes |  |
| Futurewei | Yes |  |
| ZTE | Yes | We think the intention of the issue is valid. As xiaomi comments, maybe 36.304 need to be fixed as well? Anyway, regarding the wording, we think it can be merged with the case of multicast, such as: The UE may use Paging Early Indication (PEI) in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption. If PEI configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting PEI (except for the UEs expecting multicast session activation notification or being aware of an ongoing emergency service) can monitor PEI using PEI parameters in system information according to the procedure described below. |
| Intel |  | It is unclear to us why the PEI with UE-ID based subgrouping should be prohibited when emergency session is ongoing, as long as network and UE are in sync of the UE capability. |
| CATT | Yes | To align with text in 24.501. |
| vivo | Yes |  |
| Nokia | - | Not sure if it needs to be made visible in AS since we have no definition of "emergency service is not ongoing", can AMF take care of it and indicate no subgroup ID and not supporting PEI/subgrouping to gNB? |

## Other issues

### General clarification for subgrouping

In contributions [8][14], general UE subgrouping specification clarifications are proposed for subclause 7.3.0 of TS 38.304, especially for the wording “otherwise” in the paragraph. Since both TPs are for the same paragraph but with different approaches so rapporteur suggests treating them together and let’s come out a final version for agreement if consensus is reached.

Option 1: Agree to use TP in [8] as baseline, provide further comment if any.

Option 2: Agree to use TP in [14] as baseline, provide further comment if any.

Option 3: Others. Provide a different view (included Do Not Agree) or comment.

Q5: With which option do companies agree and any further comment?

| Company | Option 1, 2 or 3 | Comments |
| --- | --- | --- |
| Xiaomi | 1 | Change in [8] is OK. No strong view in [14], as the current spec is clear. |
| Qualcomm | 3 | No strong view. Both [8] and [14] seem to be text optimization. |
| Futurewei | 3 | No strong view on [14] as the change is about formality. Can go with the majority. The leading sentence of the three bullets is currently described from the viewpoint of a single UE. Changes proposed in [8] are from the viewpoint of a group of UEs. Therefore, the changes in [8] create a mismatch. In addition, some changes in [8] are unnecessary. For example, it is unnecessary to add “for the UEs supporting PEI”, because the whole subclause is based on the premise “If PEI and subgrouping are configured, …” However, we support the first editorial change in [8], i.e., changing “formed” to “derived”. We also suggest the following change to the first bullet in the current spec, because it applies to the UE only when the UE has a CN-assigned subgroup ID, while the third paragraph covers the case where the UE doesn’t have a CN-assigned subgroup ID. - If subgroupsNumForUEID is absent in subgroupConfig, the subgroup ID based on CN assigned subgrouping as specified in clause 7.3.1, if available to the UE, is used in the cell. |
| ZTE | 1 | Proponent, we think the option 1 can improve the readability of specification. |
| Intel | 1 | Change in [8] seems sufficient. However, we are fine to also go with [14] |
| CATT | 1 | We agree with the intention and prefer ZTE’s CR as it is clearer. |
| vivo | 1 | Both are not essential. But changes in [8] make it clearer. |
| Nokia | 2 | 38.304 should be written in a UE’s point of view. |

The 2nd change of contribution [20] indicated noLastCellUpdate is applied to PEI-capable UEs only. The condition needs to be added in the field description, as it currently is for the field description of lastUsedCellOnly.

Q6: Do companies agree with the intention and the 2nd change of the CR [20]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Xiaomi | Yes |  |
| Qualcomm | Yes |  |
| Futurewei | Yes |  |
| ZTE | Yes |  |
| Intel | Yes |  |
| CATT | Yes | Proponent. Consistent with wording in field description of lastUsedCellOnly. |
| vivo | Yes |  |
| Nokia | Yes |  |

### Capabilities

Contribution [13] proposed to rephrase field description to avoid possible misleading in TS 38.306.

Q7: Do companies agree with the intention and the 2nd change of the CR [13]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Xiaomi | No | The current spec is Ok since we have agreed supporting PEI and UE-id based subgrouping go together. |
| Qualcomm | Yes | It seems an editorial change with an intention to make the text clearer. |
| Futurewei | - | No strong preference. Can go with the majority. |
| ZTE | No strong view |  |
| Intel | No strong view | The current text seems ok to us. But if companies think that there is ambiguity, we are also fine with the new added text. If the change is agreed, the draftCR should just be endorsed for merging with the mega CR. |
| CATT | Yes |  |
| vivo | Yes | Not essential, but the updated text is more accuracy. |
| Nokia | Yes | Current wording implies PEI is not used for CN-assigned subgrouping which is incorrect. |

# Conclusion

(TBD)

# Reference

- R2-2206458	Report of [AT118-e][072][ePowSav] PEI and Subgrouping (Mediatek)	MediaTek Inc.

- R2-2206775	[DRAFT] Report of [Post118-e][072][ePowSav] PEI and Subgrouping (MediaTek)	MediaTek Inc.

(RAN2#119-e Subgrouping/PEI related tdocs listed in the order of Agenda v2)

- R2-2206932	Reply LS on PEI and UE Subgrouping (R3-224004; contact: ZTE)	RAN3

- R2-2207070	Stage-2 correction on UE-ID based subgrouping		OPPO

- R2-2207742	Miscellaneous CR on TS 38.304 for ePowSav	vivo

- R2-2208015	Stage 2 correction on power saving	Nokia, Nokia Shanghai Bell

- R2-2208227	Corrections for UE power saving enhancements In 38.300	Huawei, HiSilicon

- R2-2208554	CR on 38.304 for PEI and pagingsubgrouping	ZTE Corporation,Sanechips

- R2-2207005	Clarification of PEI monitoring related parameters	Samsung Electronics Co., Ltd

- R2-2207051	Correction to UE ID based subgrouping	OPPO

- R2-2207206	38.331 Corrections on PDCCH-ConfigCommon for PEI	Xiaomi Communications

- R2-2207745	Correction on idle/inactive TRS for ePowSav	vivo

- R2-2208016	Clarification on PEI and subgrouping capability	Nokia, Nokia Shanghai Bell

- R2-2208017	Clarification on subgrouping descriptions	Nokia, Nokia Shanghai Bell

- R2-2208089	PDCCH monitoring adaptation and C-DRX (RIL V146)	Ericsson

- R2-2208226	Correction on the UE_ID based subgrouping	Huawei, HiSilicon

- R2-2208334	Clarification on paging early indication with paging subgrouping during emergency call	MediaTek Inc.

- R2-2208609	38.304 Clarifications on SubgroupID for UE-ID based subgrouping	Xiaomi, ZTE Corporation,Vivo, Ericsson, CATT

- R2-2208090	PDCCH skipping in RAN1 and RAN2 specifications	Ericsson

- R2-2207398	Miscellaneous CR on TS 38.331 for ePowSav	CATT, Xiaomi

- R2-2207744	Correction on RLM/BFD relaxation and PEI configuration		vivo

- R2-2208909	Summary of Subgrouping/PEI contributions (MediaTek)	MediaTek inc.