3GPP TSG-RAN WG2 Meeting #119-electronic	   R2-220xxxx

Online, August 17th - August 29th, 2022

Agenda item:		5.1.3.1.2

Source:	vivo

Title:	Report of [AT119-e][010][NR1516] RRC Other

Document for:		Discussion and Decision

# 1 Introduction

This contribution is aimed at reporting the discussion and results of the following offline discussion:

[AT119-e][010][NR1516] RRC Other (vivo)

Scope: Treat R2-2207547, R2-2207548, R2-2207549, R2-2208265, R2-2207611, R2-2207612, R2-2208337, R2-2208338, R2-2207257, R2-2207615, R2-2207616, R2-2207617, R2-2207618, R2-2207560, R2-2207568, R2-2207574, R2-2208346, R2-2208347, R2-2208348. Determine agreeable parts, For agreeable parts, agree CRs.

Intended outcome: Report, Agreed CRs, LS out if applicable

Deadline: Schedule 1

The discussion scope is to gather companies’ views on the contributions [1]-[19].

# 2 Participants

To facilitate this offline discussion amongst the delegates, would you please fill in your name and the email address in the table below.

| Delegate name | E-mail address |
| --- | --- |
| Yitao Mo (Stephen) | yitao.mo@vivo.com |
| Nokia | amaanat.ali@nokia.com |
| NEC | hisashi.futaki @ nec.com |
| Apple | Zhibin_wu@apple.com |

# 3 Phase-1 Discussion

## 3.1 Correction on SIB1 repetition transmission period

In the RRC spec, it is stated that SIB1 repetition transmission period is 20 ms for SSB and CORESET multiplexing pattern 1. In the contributions [1]-[3], it is pointed out that the current RRC statement is not correct. This is because there is an achieved agreement that the UE assumes that the RMSI CORESET monitoring window corresponding to an SS/PBCH block in the radio frame satisfies the condition mod(SFN,2)=0. In other words, 20ms is just the minimum repetition period for SIB1 for CORESET multiplexing pattern 1. Other larger values (e.g. 40ms) are also feasible for this case. To get rid of the potential misunderstanding, the following text proposal is submitted in [1]-[3],

## 5.2 System information ### 5.2.1 Introduction - the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 minimum repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

Q1: Do companies agree with the intention of CR R2-2207547?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| OPPO | Not sure | Whether LS to RAN1 is needed? |
| Nokia | Yes | As proponent we think the RRC spec should be updated to not create a wrong understanding that only 20 msec SIB1 repetition transmission period is allowed. |
| NEC | Comments | It seems better to ask clarification for RAN1. |
| Apple | See comments | Need RAN1 to clarify first whether there is something wrong |

## 3.2 Discussion on SI-request Period issues

In the contribution [4], two technical issues regarding RA resources configuration for Msg1-based SI request are raised. The first question is the exact starting point of SI-request period is not clear as the parameter si-RequestPeriod only indicates the value of SI-request period (i.e. time offset of SI-request period is unknown). To this end, it was proposed that the SI-request period is started from frame 0, considering that the associate period is started from frame 0 as well.

Q2: Do companies agree that SI-request period is started from frame 0?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| OPPO | No | We think it’s clear that the RA resource association period is started from frame 0, and given that the SI request period is the integer times of the RA resource association period, it’s understood the starting frame is clear. |
| Nokia | No | We are not sure that it is possible that reference point is something else than 0. So to us the change does not seem to be necessary. |
| NEC | No | similar understanding as OPPO. |
| Apple | No strong view |  |

The second question is related to 1024 SFN boundary-crossing. Specifically, taking the following Figure 1 as an example (assuming si-RequestPeriod = n4), SI-request period herein equals 120ms, by which cannot be divided by 1024SFN (i.e. 10240ms). As a result, if ra-AssociationPeriodIndex is configured to 3 for this case, it means the UE cannot initiate the RA procedure for Msg-1 based SI request as there is no next available ROs, when 1024 SFN boundary-crossing issue occurs within the SI-request period. To get rid of this issue, it is suggested that the network ensures the SI-request period is restricted to be a 2μ multiple of 10ms, where μ is a non-negative integer up to the maximum value 4 (i.e. the maximum length of SI-request period is not expected to be larger than the maximum value of RA associate pattern period (i.e. 160ms at most)).

Figure 1: 1024 SFN boundary-crossing issue for SI-request period

Q3: Do companies agree that 1024 SFN boundary-crossing issue may occur within an SI-request period?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| OPPO | No | In the figure, it seems the association period is different, i.e., the RA associate period #0 is 40ms, the second is 20ms. It’s not clear how the si-RequestPeriod is determined in this case. Further, even if in the 1024 SFN boundary, why it’s said there is no available ROs? Thus, in all, we think it’s not clear what the issue is? |
| Nokia | No | Agree with Oppo |
| NEC | No | We could not identify the issue.. |
| Apple | No | It is unclear to us what is the problem. |

Q4: If Yes to Q3, do companies agree that the network ensures the SI-request period is restricted to be a 2μ multiple of 10ms (where μ is a non-negative integer)?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |

## 3.3 Correction on SI change notification due to si-RequestConfig

In the contributions [5]-[8], it is mentioned that the change of si-RequestConfig or si-RequestConfigSUL will result in SI change notifications, which is unnecessary (i.e. the UE which is about to initiate on-demand SI request firstly acquires the latest si-RequestConfig or si-RequestConfigSUL) and power-inefficient to the UE that does not need to request on-demand SI message. Thus, it is proposed that the change of si-RequestConfig or si-RequestConfigSUL should not result in system information change notifications.

Q5: Do companies agree that the change of si-RequestConfig or si-RequestConfigSUL should not result in system information change notifications?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| OPPO | No | In my understanding, the change of si-RequestConfig or si-RequestConfigSUL will not be so often. So it is reasonable to send the change notification due to change. |
| Nokia | No | Agree with Oppo, the basic use case why network will change it often is not clear. Anyway the change will have to be informed and it is already possible for network to do so. So we are not sure why the CR is needed. |
| NEC | No | We do not see any need to change this introduced from Rel-15 frozen long time ago. |
| Apple | See comment | We have no strong view. But for a Rel-15 CR, the change has to be very critical and this seems not meeting that high bar. |

## 3.4 Clarification to the expiry of IDLE mode measurements

In the contribution [9] Based on the current RRC spec, UE does not release stored IDLE mode measurements in VarMeasIdleReport (which is only cleared when UEInformationResponse is sent successfully) upon T331 expiration. As a result, a UE may retain the very “old” measurements across multiple RRC connections, even if T331 has expired.

However, from RAN5 perspective, this creates a problem in that it is difficult to test what happens after T331 expiry. Since UE stops measurements but doesn’t clear them, testing that UE no longer performs the measurements after T331 expiry is difficult since UE may report “old” measurements. Additionally, allowing UE to “continue” the IDLE mode measurements may even mask the behaviour, since it’s not clear when UE has done the measurements it reports, so the network has no way of differentiating those. To correct this problem, there are two main solution alternatives in the discussion contribution [9]:

- Opt 1: Clear VarMeasIdleReport at T331 expiry (simple but may cause UEs to drop some measurements unnecessarily, and may cause issues with legacy UEs)

- Opt 2: Clear VarMeasIdleReport at RRCRelease (to avoid old measurements persisting across multiple RRC connections)

Q6: Which option do companies prefer for the discard of VarMeasIdleReport?

| Company | Opt 1/Opt 2 /Comments | Detailed comments |
| --- | --- | --- |
| OPPO | None Or Opt2 if it is majority | I think it is corner case that UE has available measurement results and does not report after entering RRC_CONNECTED. Furthermore, the “out date” issue was discussed in R16 DCCA WI, but it was not addressed. |
| Nokia | - | Proponent: We think RAN2 should discuss which option to adopt to resolve the problem of VarMeasIdleReport being retained until queried by network. |
| NEC | Opt 2 | We tend to agree with the observations and prefer Opt 2 to use available and valid information as much as possible. |

Further, if either Opt 1 or Opt 2 is agreeable, then, at least for future UEs, a UE capability could also allow RAN5 to differentiate the test cases. Alternatively, a correction without a capability could be considered if no UEs supporting this functionality already exist in the field.

Q7: If either Opt 1 or Opt 2 is agreeable, would companies agree that a new UE capability is needed?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| NEC | Comments | From RAN2 functionality point of view, we do not see need of new UE capability for this. However, we can follow majorty, if it is needed for test case purpose. |

## 3.5 Measurement during RRC connection establishment/resume

Cell re-selection related measurements generally includes intra-frequency, inter-frequency, and inter-RAT frequency measurements. And those measurements shall be continued during the ongoing RRC connection establishment/resume procedure (i.e. the UE shall continue cell re-selection related measurements as well as cell re-selection evaluation), as specified in TS 38.331. However, in the contributions [10]-[13], concerns were raised as it is not feasible to manage inter-frequency or inter-RAT frequency measurements when the physical layer of UE is performing Tx/Rx (e.g. RA procedure with Msg1/3 transmission and Msg2/4 reception) with the serving cell. As a result, to facilitate the Tx/Rx during the RA procedure, the UE may not continue cell re-selection related measurements as well as cell re-selection evaluation during the RRC connection establishment/resume procedure, similar to the RedCap UE behavior (i.e. if the UE is a RedCap UE and the initial DL BWP for RedCap is not associated with CD-SSB, the UE may continue cell re-selection related measurements as well as cell re-selection evaluation).

So the following revision is proposed,

The UE shallmay continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.13.6.

Q8: Do companies agree with the intention of CR R2-2207616?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| OPPO | Not sure with comments | I wonder if it is true and whether LTE spec also needs to change? I also wonder whther the yellow part is up to physical layer implementation and the UE shall continue cell re-selection related measurements from RRC point of view. |
| Nokia | No | UE performs measurements as much as possible in accordance with RAN4 requirements it can do more but if it can’t then it can’t. As long as RAN4 requirements are met we do not see any problem. Hence we think no proposal should be agreed. |
| NEC | Comments | The observation seems valid. However, the propoed change is not fine, because changing “shall” to “may” will remove necessary functionality. As Nokia pointed out, the UE shall continue considering its situation and related RAN4 requirements. If majority agree to have some change, we can follow. |
| Apple | Yes | We think the intention is valid. |

## 3.6 Corrections on s-MeasureConfig in NR

As per section 5.5.2.1 of 38.331, the s-MeasureConfig Threshold (i.e. ssb-RSRP or csi-RSRP) would be set to the “lowest value” of the RSRP ranges indicated by the received value of s-MeasureConfig (e.g. if the IE value is 1, the lowest value of RSRP range is -156 dBm). However, this is not aligned with the description for RSRP-Range, where it’s mentioned that, for thresholds, actual value is derived as (IE value – 156).

Meanwhile, LTE has explicitly specified RSRP_0 (which is invalid codepoint with no lower bound for RSRP measurement) to be used as an indication to disable s-Measure. NR has defined RSRP_127 as threshold value of infinity. In effect, NR can achieve disabling of s-MeasureConfig with RSRP_127, however, it is not clearly specified in 38.331. To solve these issues, the following changes are proposed,

### 5.5.2 Measurement configuration #### 5.5.2.1 General … 1> if the received measConfig includes the s-MeasureConfig: 2> if s-MeasureConfig is set to ssb-RSRP, set parameter ssb-RSRP of s-MeasureConfig within VarMeasConfig to the threshold value of the RSRP ranges indicated by the received value of s-MeasureConfig which is derived as specified in 6.3.2; 2> else, set parameter csi-RSRP of s-MeasureConfig within VarMeasConfig to the threshold value of the RSRP ranges indicated by the received value of s-MeasureConfig which is derived as specified in 6.3.2. …

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS. Value “127” indicates to disable s-MeasureConfig.

Q9: Do companies agree with the intention of CR R2-2207560?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| OPPO | Yes/No with comments | For the frst change, it is still not clear. So the changes are proposed as below if it is majority view: Furthermore, I wonder whether LTE Spec is also needed to change and the LTE spec is changed to “lowest” in CR R2-091696 . For the second change, we think it is not necessary. |
| Nokia | No | If we understand correctly this does not change UE behaviour. Thus we are not sure what is really broken and especially if we extend RSRP range at some point this could break the behaviour? We would like to know what is the real problem? |
| Ericsson (Tony) | Maybe no | Similar comment as Nokia. We think current specification is not really broken. We understand this is mainly to align the behavior on what we already have for LTE, but not sure if this is really needed. |
| NEC | Comments | We do not see strong need but can go with majority |
| Apple | No strong view | We are also fine with the current description |

## 3.7 Corrections on measurement report triggering

Based on the current RRC spec, if there is a cell in cellsTriggeredList also meets the leaving condition, this cell will not be removed from cellsTriggeredList. However, this is not correct. The corresponding change should be as follows,

### 5.5.4 Measurement report triggering #### 5.5.4.1 General … 2> if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event): 3> include a measurement reporting entry within the VarMeasReportList for this measId; 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 3> initiate the measurement reporting procedure, as specified in 5.5.5; 2> else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event): 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 3> initiate the measurement reporting procedure, as specified in 5.5.5; 2> if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event: 3> remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 3> if reportOnLeave is set to true for the corresponding reporting configuration: 4> initiate the measurement reporting procedure, as specified in 5.5.5; 3> if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty: 4> remove the measurement reporting entry within the VarMeasReportList for this measId; 4> stop the periodical reporting timer for this measId, if running …

Q10: Do companies agree with the intention of CR R2-2208346?

| Company | Yes/No/Comments | Detailed comments |
| --- | --- | --- |
| OPPO | Yes | Based on the current RRC spec, if one new cell meets the entry condtion and there is another cell in cellsTriggeredList also meet the leaving condition, this cell will not be removed from cellsTriggeredList becaue the corresponding text is skiped due to “if….else if…else if….”. We also noted that the text for this part is different from LTE and there is no issue in LTE Spec. |
| Nokia | Yes | Maybe OK but on the other hand if we understand correctly this occurs only if same instant (very dependant on UE implementation) both entering and leaving conditions are fulfilled for some cells. Probably not really essential to correct but for completeness we are okay to have CR if other companies agree. |
| NEC | Yes |  |
| Apple | Yes |  |

# 4 Conclusion

# 5 Reference

- R2-2207547, SIB1 transmission period, Nokia, Nokia Shanghai Bell.

- R2-2207548, SIB1 transmission period, Nokia, Nokia Shanghai Bell.

- R2-2207549, SIB1 transmission period, Nokia, Nokia Shanghai Bell.

- R2-2208265, Discussion on SI-request Period Issues, vivo.

- R2-2207611, Disscussion on SI request issue,	vivo.

- R2-2207612, 38331 CR on SI request, vivo	.

- R2-2208337, 38331 CR on SI request, vivo	.

- R2-2208338, 38331 CR on SI request, vivo	.

- R2-2207257, Clarification to expiry of IDLE mode measurements, Nokia, Nokia Shanghai Bell.

- R2-2207615, Discussion on the measurement during RRC connection establishment and RRC connection resume, vivo.

- R2-2207616, Rel-15 331 CR on  the measurement during RRC connection establishment and RRC connection resume, vivo.

- R2-2207617, Rel-16 331 CR on  the measurement during RRC connection establishment and RRC connection resume, vivo.

- R2-2207618, Rel-17 331 CR on  the measurement during RRC connection establishment and RRC connection resume, vivo.

- R2-2207560, Corrections on s-MeasureConfig in NR, Samsung.

- R2-2207568, Corrections on s-MeasureConfig in NR, Samsung.

- R2-2207574, Corrections on s-MeasureConfig in NR, Samsung.

- R2-2208346, Corrections on measurement report triggering-R15, OPPO, ZEKU, ZTE.

- R2-2208347, Corrections on measurement report triggering-R16, OPPO, ZEKU, ZTE.

- R2-2208348, Corrections on measurement report triggering-R17, OPPO, ZEKU, ZTE.