3GPP TSG-RAN WG2 #119 electronic	R2-220xxxx

Electronic Meeting, Aug 17th – 29th, 2021

Agenda Item:	5.1.3.1.2

Source:	Ericsson

Title:	Summary of offline [011][NR1516] RRC LTE Overheating Misc and Idle

Document for:	Discussion, Decision

# 1	Introduction

This contribution summarizes the following discussion:

[AT119-e][011][NR1516] RRC LTE Overheating Misc and Idle (Ericsson)

Scope: Treat R2-2208202, R2-2208203, R2-2207575, R2-2207576, R2-2207577, R2-2208207, R2-2208208, R2-2207357, R2-2207358, R2-2208209, R2-2208210, R2-2208211, R2-2208140, R2-2207540, R2-2207558, R2-2207559 Determine agreeable parts, For agreeable parts, agree CRs.

Intended outcome: Report, Agreed CRs, LS out if applicable

Deadline: Schedule 1

Companies are invited to fill in contact details.

| Company | Contact details |
| --- | --- |
| OPPO | shicong@oppo.com |
| Samsung | sb07.kim@samsung.com (Sangbum Kim) |
| Huawei, HiSilicon | kuangyiru@huawei.com (Yiru Kuang) |
| MediaTek | chun-fan.tsai@mediatek.com |
| Verizon | Vishwanath.ramamurthi@verizonwireless.com |
| Qualcomm Inc | mambriss@qti.qualcomm.com (Mouaffac) |
| Lenovo | hchoi5@lenovo.com |
| CATT | shijie@catt.cn |
| Ericsson | lian.araujo@ericsson.com |
| Intel | sudeep.k.palat@intel.com |
| ZTE | liu.jing@zte.com.cn |
| Apple | sethu@apple.com |

# 2	Discussion

## 2.1	Part 1: Intended to determine agreeable parts

The CRs in [1][2] propose miscellaneous editorial corrections.

Q1 Do companies agree with the intention of the CRs above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes | Can merge with the rapp CR. |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |
| MediaTek | Yes |  |
| Lenovo | Yes | Proponent; ok to merge with the rapporteur CR. |
| Nokia | Yes |  |
| CATT | Yes |  |
| Ericsson | Yes |  |
| Intel | Yes |  |
| ZTE | Yes |  |
| Apple | Yes |  |

The CR in [3] is not provided by the 38331 Rapporteur, and hence not part of this email discussion.

The CRs in [4][5][6] intend to clarify the following:

1.	Clarify in 5.5.5.1 (in 36.331) that the UE includes NR serving frequency results if the purpose field in the associated report configuration is not configured or is set to a value other than reportLocation.

2.	Clarify in 5.5.5.1 (in 36.331) that the UE includes NR serving frequency results only in case of (NG)EN-DC.

Q2 Do companies agree with the intention of the CRs above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes | Proponent |
| Samsung | No | Agree to the intention of this CR, i.e. SN measurement results may be less usefulness to LTE SN for NE-DC, but it seems to be less critical problem. LTE SN could still handle it by implementation i.e. it simply ignores the measurement results. |
| Huawei, HiSilicon | Yes | Proponent |
| MediaTek | Yes |  |
| Nokia | Yes | Proponent, the clarifications are required as the current procedural text seems deficient. |
| CATT | Yes | Proponent |
| Ericsson | Yes | Proponent |
| Intel | Yes |  |
| ZTE | Yes | Proponent |

The CRs in [7][8][9][10] intend to clarify the UE behavior to indicate overheating mitigation for SCG in case of EN-DC. Two options can be outlined:

Option 1: In [7][8], this is clarified in procedural text.

Option 2: In [9][10], this is clarified with a note.

Q3 Which option is preferred by companies?

| Company | Option 1/2 | Comments |
| --- | --- | --- |
| OPPO | Option 2 | The note clarifies the intended behaviour |
| Samsung | Option 2 (if required, possible to merge with Option 1) | Proponent on [9][10]. If required, two options could be merged. Then, UEAssistanceInformation-v1530-Ies should be also considered. Actually, we assume it has been implicitly agreed in R2-2111610 (i.e. in the Reasone for change). On the other hand, for Rel-17, the following intention should be also reflected in RRC specification: not including the overheatingAssistance-v1610 or overheatingAssistance-v1710 signifies that UE can implement it by not including parent IE, e.g. UEAssistanceInformation-v1710-IEs, UEAssistanceInformation-v1700-IEs, UEAssistanceInformation-v1610-IEs or UEAssistanceInformation-v1530-IEs, the NW interpretation in all cases is that “UE did not include overheatingAssistance-v1610 or overheatingAssistance-v1710” |
| Huawei, HiSilicon |  | We understand the issue in [7][8] is the procedural text says “do not include overheatingAssistance-v1610 in OverheatingAssistance IE”, however, the overheatingAssistance-v1610 is included in UEAssistanceInformation-v1610 IE. We prefer to simply the change, e.g. 3> do not include reducedUE-Category, reducedMaxCCs in OverheatingAssistance IE and do not include overheatingAssistance-v1610 (if configured to provide overheating assistance indication for NR SCG) in UEAssistanceInformation-v1610 IE; Besides, the UE cannot send overheatingAssistance-v1610 if overheating assistance for NR SCG is not configured, so NOTE 0 in [7][8] is not needed. |
| MediaTek | See comment | We support the intention to furhter clarify UE beahvior in this scneario as we understand there is real issue in the field. Option 2 (adding note based on previous agreed CR) could be accetable. For CR in option 1, we have several comments. The coversheet should clarify the intended behavior, it is actually unclear whehter the intetion is to revert previous agreed CR R2-2111610 or not. We don’t wnat to revert previous agreement. For newly added NOTE 0, there is no description on why we need this and the wroding is difficult to understand. We are not sure whehter this is really needed. We would like to continue the wording discussion in phase 2 if agreed to have option 1. HW’s wording on procedure text is one alternative. |
| Verizon | Option 1 preferred | It is important to address the issue as it is a real scenario we see in the field. Option 1 provides better clarity for UE implementation and thus minimizes misunderstanding/interoperability issues between network and UE. NOTE 0 helps when UE is recovering from overheating. Some of the text from Option 2 could be combined into Option 1 in phase 2. |
| Qualcomm Inc | Option-1 | Proponent It seems the intention behind Qualcomm CR is missed. it would be great if you can give me the chance to provide more details. Suggested change by Samsung: NOTE 5: not including the overheatingAssistance-v1610 signifies that UE can implement it by not including parent IE, e.g. UEAssistanceInformation-v1610-IEs or UEAssistanceInformation-v1530-IEs, the NW interpretation in both cases is that “UE did not include overheatingAssistance-v1610” With the current suggested changes by Samsung, not including the UEAssistanceInformation-vXYZ can cause backward and forward compatibility issues: - Example of a backward compatibility issue: - By not including the UEAssistanceInformation-v1450, the network will not be able to determine if overheating was alleviated for Rel-14 (MN) or for Rel-16 (SN) or for both, as the UEAssistanceInformation-v1450 contains the overheating information for both MN and SN. UEAssistanceInformation-v1450-IEs ::= SEQUENCE { overheatingAssistance-r14 OverheatingAssistance-r14 OPTIONAL, nonCriticalExtension UEAssistanceInformation-v1530-IEs OPTIONAL } UEAssistanceInformation-v1530-IEs ::= SEQUENCE { sps-AssistanceInformation-v1530 SEQUENCE { trafficPatternInfoListSL-v1530 TrafficPatternInfoList-v1530 } OPTIONAL, nonCriticalExtension UEAssistanceInformation-v1610-IEs OPTIONAL } UEAssistanceInformation-v1610-IEs ::= SEQUENCE { overheatingAssistance-v1610 OverheatingAssistance-v1610 OPTIONAL, nonCriticalExtension UEAssistanceInformation-v1700-IEs OPTIONAL } ***** [Samsung] Thank you for further explanation. We also add some comments for further clarification. The backward compatibility issue introduced above seems unclear to us. Actually, UEAssistanceInformation-v1450 is not related to this topic, i.e. the Reason of change of R2-2111610 mentions UEAssistanceInformation-v1610-IEs or UEAssistanceInformation-v1530-IEs. In our understanding, delta signalling is not supported in LTE UAI, unlike NR UAI, i.e. when UAI message is initiated due to a type of configured assistance information (see 5.6.10.2), UE shall set contents for all types of the configured assistance information (see 5.6.10.3). ***** - Example of a forward compatibility issue: - By not including the UEAssistanceInformation-v1610, we are blocking Rel-17 UEAssistanceInformation-v17xy IE from being reported simultaneously in the same UAI report, as Rel-17 UAI are non-critical extensions of the Rel-16 UAI. UEAssistanceInformation-v1610-IEs ::= SEQUENCE { overheatingAssistance-v1610 OverheatingAssistance-v1610 OPTIONAL, nonCriticalExtension UEAssistanceInformation-v1700-IEs OPTIONAL } UEAssistanceInformation-v1700-IEs ::= SEQUENCE { uplinkData-r17 ENUMERATED { true } OPTIONAL, scg-DeactivationPreference-r17 ENUMERATED { scgDeactivationPreferred, noPreference } OPTIONAL, nonCriticalExtension UEAssistanceInformation-v1710-IEs OPTIONAL } UEAssistanceInformation-v1710-IEs ::= SEQUENCE { overheatingAssistance-v1710 OverheatingAssistance-v1710 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } ***** [Samsung] the coversheet of R2-2111610 doesn’t mention to block other assistance information, i.e. it’s an option for specific case. not including the overheatingAssistance-v1610 signifies that UE can implement it by not including parent IE, e.g. UEAssistanceInformation-v1610-IEs or UEAssistanceInformation-v1530-IEs, the NW interpretation in both cases is that “UE did not include overheatingAssistance-v1610” For instance, if not configured to provide SPS assistance information(Rel-15) and UE no longer experiences overheating, UE is not required to include UEAssistanceInformation-v1530-IEs and UEAssistanceInformation-v1610-IEs according to the coversheet of R2-2111610. On the other hand, if configured to provide SPS assistance information and UE no longer experiences overheating, UE includes UEAssistanceInformation-v1530-IEs for sps-AssistanceIfnromation-v1530. But, UE is still not required to include UEAssistanceInformation-v1610-IEs. UEAssistanceInformation-v1530-IEs ::= SEQUENCE { sps-AssistanceInformation-v1530 SEQUENCE { trafficPatternInfoListSL-v1530 TrafficPatternInfoList-v1530 } OPTIONAL, nonCriticalExtension UEAssistanceInformation-v1610-IEs OPTIONAL } UEAssistanceInformation-v1610-IEs ::= SEQUENCE { overheatingAssistance-v1610 OverheatingAssistance-v1610 OPTIONAL, nonCriticalExtension UEAssistanceInformation-v1700-IEs OPTIONAL } As similar example, if configured to provide its preference for NR SCG deactivation (Rel-17), UEAssistanceInformation-v1610 has to be signalled, while including UEAssistanceInformation-v1700 with scg-DeactivationPreference-r17. But then, UEAssistanceInformation-v1610 doesn’t include overheatingAssistance-v1610, according to current procedural texts. UEAssistanceInformation-v1610-IEs ::= SEQUENCE { overheatingAssistance-v1610 OverheatingAssistance-v1610 OPTIONAL, nonCriticalExtension UEAssistanceInformation-v1700-IEs OPTIONAL } UEAssistanceInformation-v1700-IEs ::= SEQUENCE { uplinkData-r17 ENUMERATED { true } OPTIONAL, scg-DeactivationPreference-r17 ENUMERATED { scgDeactivationPreferred, noPreference } OPTIONAL, nonCriticalExtension UEAssistanceInformation-v1710-IEs OPTIONAL } UEAssistanceInformation-v1710-IEs ::= SEQUENCE { overheatingAssistance-v1710 OverheatingAssistance-v1710 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } ***** QC CR is addressing these issues, by ensuring all the UEAssistanceInformation IEs are independent. more specifically the CR is restricting the UE to only “not include” the overheatingAssistance-v1610 for the NR SCG case, and overheatingAssistance-v1710 for the case of NR SCG FR2-2. This will remove ambiguity at the network, and at the same time ensure forward compatibility with Rel-17 onward. QC CR: 2> else (if the UE no longer experiences an overheating condition): 3> if the UE had a preference for the OverheatingAssistance IE: 4> do not include reducedUE-Category, reducedMaxCCs in OverheatingAssistance IE; 3> if the UE had a preference for the overheatingAssistanceForSCG IE: 4> do not include overheatingAssistance-v1610 in the UEAssistanceInformation-v1610 IE; 4> if configured with serving cells operating on FR2-2 for NR SCG 5> do not include OverheatingAssistance-v1710 in the UEAssistanceInformation-v1710 IE; |
| Nokia | Not sure | Its unclear what eventually UE implementation is. UEAssistanceInformation-v1530-Ies is not a parent IE for overheatingAssistance-v1610, which is specifically for SCG. Thus it should be clear what the UE intends to indicate, by not mixing with -v16 with v15. We think this is unclear interpretation for the network. |
| CATT | See comment. | Agree with the intention. Share the same view that NOTE 0 in [7][8] is not needed. And we are fine with the text in [7][8] except NOTE 0 or the proposed change by Huawei. |
| Ericsson | Option 1 | We think this should be clearly captured in the specifications to avoid inter-operability issues. We can further discuss the detailed wording in phase 2. |
| Intel | Option 1 (with comments) | We prefer to capture in procedural text but more discussion is needed on the wording. |
| ZTE | Option 1 | We also prefer to make it clear in procedural text. So the text proposal from QC can be a startpoint, but the Note 0 is also confused to us, better to remove it. In our understanding, the below 3 cases all imply that the UE no longer experiences overheating issue for SCG (if it had before): - Case 1: the UE includes UEAssistanceInformation-v1610 but without overheatingAssistance-v1610; - Case 2: The UE does not include UEAssistanceInformation-v1610; - Case 3: The UE does not include UEAssistanceInformation-v1530-IEs; |
| Apple | Option 1 | Capturing in procedural text is much clearer. |

The CRs in [11][12][13] propose to update the description of the overheating IEs, to reflect that the UE sends the overheating UAI for active CCs.

Q4 Do companies agree with the intention of the CRs above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Samsung | No | We have a strong concern on this change because it results in a critical impact on current UE implementation. Furthermore, RRC specification clearly mentions 'configured CCs' in #### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message (skipped) 1> if transmission of the UEAssistanceInformation message is initiated to provide maxCC-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3: 2> include maxCC-Preference in the UEAssistanceInformation message; 2> if the UE has a preference on the maximum number of secondary component carriers for the cell group: 3> include reducedMaxCCs in the MaxCC-Preference IE; 3> set reducedCCsDL to the number of maximum SCells the UE desires to have configured in downlink in the cell group; 3> set reducedCCsUL to the number of maximum SCells the UE desires to have configured in uplink in the cell group; Finally, we see no problem with the current specification. [Qualcomm] configured CC could be configured active CC and/or configured inactive CC. In addition the same IE for the power saving feature, it was explicitly mentioned ”active" CC, so clearly there is a misalignment between feature that needs to be rectified. [Samsung] In the procedural texts in 5.7.4.3 (power saving) and 5.7.4.3a (overheating), ‘UE desires to have configured‘ is described. We cannot support this change since we can see no critical problem with the current specification. Rather, this change may just confuse the network, especially in the case of mixing of legacy UEs and new UEs implementing this change. |
| Huawei, HiSilicon | No | These are NBC changes. Based on previous discussion and conclusion, the intention of reducedCCs is about “configured CC” instead of “active CC” which can be hinted from the description “Indicates the UE's preference on reduced configuration corresponding... ”. [Qualcomm] in the previous discussion it was agreed to be left to Network implementation how to handle the UAI report, by either deactivating or deconfiguring the designating CC. |
| MediaTek | Prefer No |  |
| Qualcom Inc | Yes | Proponent Just to make a couple of comments: - As mentioned in the CR ”reduced configuration” doesn’t necessarily mean only configured CCs, as configured CCs can be active and/or non-active. - Reporting configured non-active CCs is useless, as configured CCs are not the source of overheating. - The CR doesn’t address what would be the network expected behavior upon reception of the UAI report, i.e., deconfiguring or deactivating a specific CC to reduce overheating, as this has been agreed in previous meeting that it’s left to network implementation to handle the report. The CR is just clarifying the significance of the transmitted value in the UAI report. |
| CATT | No | As it is NBC changes, we prefer to keep it as it is. And please note the above text in 5.7.4.3 is for power saving, not for overheating. |
| Ericsson | No | We think there may still be some issues. For instance, does it mean that the NW may keep the current number of MIMO layers if it deactivates the SCells? Or does it mean that the UE is happy with the number of MIMO layers that are currently being deactivated and will continue to be happy with those even if the NW activates the SCells? |
| ZTE | No | After checking internally, this change is indeed NBC to us. |
| Apple | No | This is a NBC change. Prefer to keep it as is. From the very beginning this was meant to be for the configured CCs. |

The CRs in [14][15][16] intend to clarify that, when the cell is identified to not support IMS emergency calls when camped on Any Cell state, the UE shall deprioritize the pertinent frequency to be the lowest priority frequency.

Q5 Do companies agree with the intention of the CRs above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No | Not sure how frequent the case happens in the field. BTW, even if it happens, is it possible that the UE igores the previous cell and reselect to another cell for camping by UE implementation. |
| Samsung | Yes | Once UE reselects an acceptable cell that supports IMS energency calls, it is very likely with the existing cell reselection criteria that UE reverts to the previous cell not supporting IMS emergency calls. This ping-pong of reselection is undesirable. @OPPO, It will not be consistent with present specification. We think a clear behaviour should be provided in the specification to support UE implementation. |
| Huawei, HiSilicon | No | The spec already allowes the UE ignores priorities provided in SI in order to reselect to an acceptable cell that supports emergency calls, this change is not necessary. Besides, some other cells in the same frequency will be impacted according to the CR. |
| MediaTek | No | We prefer leave this to UE implmentation. |
| Qualcomm Inc | May be not | Changing Rel-15 procedural text is too much; a Chair Note may be sufficient, |
| Lenovo | No | Not clear what the issue is with the current behaviour since it was adopted from LTE (where it is specified since Rel-9). Was such ping-pong effect observed in the field? |
| Nokia | No, avoided by proper UE implementation | Problem is possible with stupid implementation i.e. UE will reselect out of IMS not supporting cell/frequency and on new frequency/cell it will perfrom regular reselection which may cause reselection back to this cell/frequency not supporting IMS emergency. Proper UE will handle this and if we want to specify solution it would need much smarter approach i.e. merely UE is allowed to consider IMS not supporting frequency as lower priority for X seconds. It cannot last forever as IMS support may not be whole PLMN wide issue! |
| CATT | No | Share the same view with Huawei that it has already clarified that the UE shall perform cell selection/reselection to an acceptable cell that supports emergency calls in any supported RAT regardless of priorities provided in system information from current cell, if no suitable cell is found in this case. No need to deprioritize the pertinent frequency. |
| Ericsson | No | The intention of the specification text is clear (since LTE), and the UE should not re-select back to a cell which does not support IMS emergency if an acceptable cell with support is found. We do not think the change is essential or necessary. Further, we agree with Huawei that with the proposed change there can be further implications with unwanted consequences: Downprioritizing the whole frequency may affect other cells as well. |
| Intel | No | We don’t see this as an essential correction. It assumes a certain UE behaviour but UEs may/can implement mechanisms to prevent such pingpong. |
| ZTE | No | We think this can be handled by smart UE implementation. It is inappropriate to deprioritize the whole frequency, as other cells on the same frequency may be suitable for the UE. |
| Apple | No | Can be left to UE implementation. |

## 2.2	Part 2: Intended to progress discussion on agreeable parts

- To be updated after discussion on part 1 -

# 3	Conclusion

- To be updated after discussion on part 1 -

# 4	References

- R2-2208202	Miscellaneous corrections	Lenovo	draftCR	Rel-16	38.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2208203	Miscellaneous corrections	Lenovo	draftCR	Rel-17	38.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2208140	Miscellaneous non-controversial corrections Set XV	Ericsson	CR	Rel-16	38.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2207575	Correction on NR serving frequency results reporting for event-triggered measurement (R15)	Huawei, HiSilicon, OPPO, MediaTek Inc., vivo, Nokia, Nokia Shanghai Bell, CATT, Ericsson, NTT DOCOMO, Lenovo, ZTE Corporation, Apple, NEC, China Telecom	CR	Rel-15	36.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2207576	Correction on NR serving frequency results reporting for event-triggered measurement (R16)	Huawei, HiSilicon, OPPO, MediaTek Inc., vivo, Nokia, Nokia Shanghai Bell, CATT, Ericsson, NTT DOCOMO, Lenovo, ZTE Corporation, Apple, NEC, China Telecom	CR	Rel-16	36.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2207577	Correction on NR serving frequency results reporting for event-triggered measurement (R17)	Huawei, HiSilicon, OPPO, MediaTek Inc., vivo, Nokia, Nokia Shanghai Bell, CATT, Ericsson, NTT DOCOMO, Lenovo, ZTE Corporation, Apple, NEC, China Telecom	CR	Rel-17	36.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2208207	Rel-16 Correction of overheating for NR SCG	Qualcomm Incorporated, Ericsson	CR	Rel-16	36.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2208208	Rel-17 Correction of overheating for NR SCG	Qualcomm Incorporated, Ericsson	CR	Rel-17	36.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2207357	SCG Overheating termination in EN-DC	Samsung	CR	Rel-16	36.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2207358	SCG Overheating termination in EN-DC	Samsung	CR	Rel-17	36.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2208209	Rel-15 Clarification on the overheating UAI	Qualcomm Incorporated	CR	Rel-15	38.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2208210	Rel-16 Clarification on the overheating UAI	Qualcomm Incorporated	CR	Rel-16	38.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2208211	Rel-17 Clarification on the overheating UAI	Qualcomm Incorporated	CR	Rel-17	38.331, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2207540	UE behavior when IMS emergency call is not supported in cell	Samsung	CR	Rel-15	38.304, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2207558	UE behavior when IMS emergency call is not supported in cell	Samsung	CR	Rel-16	38.304, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2207559	UE behavior when IMS emergency call is not supported in cell	Samsung	CR	Rel-17	38.304, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022