3GPP TSG-RAN WG2 #119e                                                                                        R2-220xxxx

eMeeting, 17th – 26th August, 2022

Agenda Item:	    5.1.3.3

Source: 	    MediaTek Inc.

Title:		    [Draft] Report of [AT119-e][012][NR1516] UE capabilities (MediaTek)

Document for:	    Discussion and decision

1 Introduction

This is report for the following AT119-e mail discussion.

[AT119-e][012][NR1516] UE capabilities (MediaTek)

Scope: Treat R2-2206911, R2-2208501, R2-2208502, R2-2208503, R2-2208504, R2-2207640, R2-2207641, R2-2207049, R2-2207085, R2-2207086, R2-2207094, R2-2207095, R2-2207113, R2-2207114, R2-2208027, R2-2208028, R2-2207331, R2-2207332, R2-2208505, R2-2208506. Determine agreeable parts, For agreeable parts, agree CRs.

Intended outcome: Report, Agreed CRs, LS out if applicable

Deadline: Schedule 1

This discussion follows Schedule 1, which is organized as follow:

Discussions with Deadline Schedule 1:

A first round with Deadline for comments W1 Friday Aug 19th 1400 UTC to settle scope what is agreeable etc

A Final round with Final deadline W2 Thursday Aug 25th 1200 UTC to settle details / agree CRs etc.

For all discussions: Additional deadlines check points etc if needed are defined by the Rapporteur of each discussion respectively. In case some parts of an email discussion need more time, doesn’t converge, need on-line treatment, then please contact the chair.

2 Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| MediaTek (Rapp) | Mutai Morton Lin | morton.lin@mediatek.com |
| Huawei, HiSilicon | Tong Sha | shatong3@hisilicon.com |
| Qualcomm Incorporated | Masato Kitazoe | mkitazoe@qti.qualcomm.com |
| Ericsson | Lian Araujo | lian.araujo@ericsson.com |
| Apple | Naveen Palle | naveen.palle@apple.com |
| OPPO | Qianxi Lu Zhe Fu | qianxi.lu@oppo.com fuzhe@oppo.com |
| China Telecom | Pei Lin | linp@chinatelecom.cn |
| Samsung | Sangbum Kim | sb07.kim@samsung.com |
| Nokia |  | amaanat.ali@nokia.com |
| vivo | Jing Liang | liangjing@vivo.com |

3 Discussion

3.1 First round: Intended to determine agreeable parts

Power Class 1.5

[1] R2-2207049	On early implementation and capability signaling of Power Class 1.5	MediaTek Inc.	discussion	Rel-16	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

[2] R2-2207094	Make PC1.5 an early implementation candidate	MediaTek Inc.	CR	Rel-16	38.331	16.9.0	3454	-	F	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

[3] R2-2207095	Make PC1.5 an early implementation candidate	MediaTek Inc.	CR	Rel-17	38.331	17.1.0	3455	-	A	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

The discussion paper [1] indicates the necessity in the early implementation of PC1.5 and discloses a special power class reporting strategy which is unclear how would network treat such a UE capability. It proposed:

- Add PC 1.5 in the Table C-1 of TS 38.331 as the earliest implementable release is Rel-15.

- The principle “Network regards the highest supported power class among all advertised ones as the supported capability” shall be captured in RAN2 specification.

Therefore CRs [2][3] propose to make PC1.5 an early implementable feature to resolve prerequisite absence issue.

Q1: Do companies agree with the intention of the CRs [2][3]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm Incorporated | Yes, but | Why is the proposed text only limit the early implementability to intra-band EN-DC, while the PC1.5 was introduced also for n41 NR SA? [MTK@v12] Text was kept unchanged and it was the title of the original draftCR R2-2006360. (Seems like 29dBm for DC_(n)41 went earlier than n41 SA in RAN4 somehow) |
| Ericsson | See Comments | We are fine with P1. On P2, we prefer to capture the principle as a UE requirement, e.g. that “UE indicates all power classes it supports”. We typically avoid requirements on Nw impl in UE capabilities. We should keep that principle. [MTK@v12] As per we indicated in subclause 2.4 in [1], it is not viable to report “all power classes UE supports” by ENUMERATED type IE. We are fine to follow RAN2 principle. |
| Apple | OK to P1, no to P2 | Power-class handling has been different to other capabilities. The philosophy of p-max is that UE uses it’s power-class unless NW specifically informs otherwise. We are not sure if anything needs to be captured. UE report the capability and based on presence/absence of power config, the UE uses the relevant Tx power. [MTK@v12] Not sure if we read the comment correctly. So far, we learnt from previous PC1.5 discussion, some infra vendors had concern to commit setting P-max for proper Tx power restriction by NW side. |
| OPPO | Yes | Same comment as Qualcomm |
| Samsung | Fine for P1, but need to clarify P2 | Regarding P2, we would like to ask any inter-operability issue between gNB and UE, i.e. for legacy gNB not supporting Power Class 1.5, the field ue-PowerClass should be also set together with ue-PowerClass-v1610? [MTK@v12] In our understanding UE does not know if serving gNB is legacy one or up to which power class the gNB supports so there are different cases like we summarized in the Table 2-1 in [1]. The scenario you mentioned is also included. The intention of P2 is exact for interoperability problem avoidance. |
| Nokia | Yes | We think the title could be just "powerClass" as well (i.e., no suffixes), but what is being proposed is also fine. |
| vivo | Yes | We think P1 is OK to us. For P2, no strong view and we can follow the majority. |
| MediaTek | Yes (Proponent) |  |

[4] R2-2207085	PC1.5 and legacy power class capability reporting clarification	MediaTek Inc.	CR	Rel-16	38.306	16.9.0	0795	-	F	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

[5] R2-2207086	PC1.5 and legacy power class capability reporting clarification	MediaTek Inc.	CR	Rel-17	38.306	17.1.0	0796	-	A	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

Based on the proposal in discussion paper [1], CRs [4][5] propose to clarify network interpretation when UE reports more than one PC (to gain better UL coverage in legacy NW).

Q2: Do companies agree with the intention of the CRs [4][5]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm Incorporated | Yes |  |
| Ericsson | No |  |
| Apple | No |  |
| OPPO | Yes |  |
| Samsung | pending | Need to clarify. See above |
| Nokia | Yes | We think the title could be just "powerClass" as well (i.e., no suffixes), but what is being proposed is also fine. |
| vivo | No strong view |  |
| MediaTek | Yes (Proponent) |  |

CSI-RS capability signaling

[6] R2-2207113	Clarification on codebookParametersPerBC parameter for extension of CSI-RS capabilities reporting	MediaTek Inc.	CR	Rel-16	38.331	16.9.0	3452	-	F	NR_newRAT-Core, TEI16

[7] R2-2207114	Clarification on codebookParametersPerBC parameter for extension of CSI-RS capabilities reporting	MediaTek Inc.	CR	Rel-17	38.331	17.1.0	3453	-	A	NR_newRAT-Core, TEI16

CRs [6][7] point out a conflict that UE is required to report codebookParametersPerBC parameter under CA-ParametersNR for non-CA band combination and propose to clarify reporting condition.

Q3: Do companies agree with the intention of the CRs [6][7]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | This is NBC. In current specification, it is required that both codebookParametersPerBC and codebookParametersPerBand should be reported together by UE if supported, regardless whether the BC is for CA or not. Then for the legacy NW, the NW would consider a UE not reporting codebookParametersPerBC as not supporting the corresponding enhanced codebook capability. If a UE is implemented according to the CR, then the enhanced codebooks can never be configured for a non-CA BC. We should avoid such a NBC change for a Rel-16 capability at this stage. |
| Qualcomm Incorporated | Yes | But backward compatibility to legacy network must be assessed by infra-vendors. |
| Ericsson | Yes | The RAN1 LS stated that the per BC parameter is to be used to limit the CSI-RS resources that can be used across carriers, it was not meant for single CC case. How exactly would the NW use the per BC parameter in case of single CC case? |
| OPPO | Yes | But agree that the inter-operability issue needs to be considered by NW vendors. |
| Samsung | Yes | For the case of non-CA band combination, it is unclear what codebookParametersPerBC means. It seems less reasonable to include codebookParametersPerBC even for the case of non-CA band combination, due to the reason that non-CA band combination is also in BandCombinationList. |
| Nokia | Open to discuss | We agree with Huawei the CR seems NBC as it is written now. Although we agree that it seems counter-intuitive how to reconcile the non-CA BC i.e., single CC case with CA-ParametersNR. At least let’s first have a common understanding how the network is supposed to interpret the existing signalling and then we can maybe clarify. |
| vivo | Yes | The intention is OK to us but agree with companies that backward compatibility needs to be further evaluated. |
| MediaTek | Yes (Proponent) | Possible interoperability problem was mentioned in the coversheet. But we need to clarify and fix this issue by considering more release-independent NR bands are going to be introduced. |

Beam management

[8] R2-2207331	Correction on beamManagementSSB-CSI-RS	Qualcomm Incorporated	CR	Rel-16	38.306	16.9.0	0765	-	F	TEI16

[9] R2-2207332	Correction on beamManagementSSB-CSI-RS	Qualcomm Incorporated	CR	Rel-17	38.306	17.1.0	0766	-	A	TEI16

CRs [8][9] indicate that the current R2 specification does not correctly capture the RAN1 conclusion in RAN1#101-e regarding to the beam management capability and propose corresponding corrections.

Q4: Do companies agree with the intention of the CRs [8][9]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | We do not see the description in the CR reflected in the RAN1 feature list nor in the LS to RAN2, thus it is not necessary to be captured in the 38.306. Besides, for FR2, it is confused how the UE indicates the capability according to the smallest SCS configured for PDSCH, since UE has no idea what is the smallest SCS of the serving cell(s) configured by the NW when reporting the capability information. Assuming that UE reports the capability according to the supported smallest SCS for FR2 band (e.g. 60kHz), then it is contradictory with the conclusion of the smallest SCS configured for PDSCH. We suggest to send RAN1 a LS, asking to clarify what does it mean by “For FR2, the parameter indicates the total number of resources across serving cells within 1 slot of the smallest subcarrier spacing configured for PDSCH in FR2”. |
| Qualcomm Incorporated | Yes | At least the current text is incorrect and must be corrected. As for the FR2 capability (Huawei’s question), it is our understanding that the UE capability is semi-static regardless of the size of the smallest SCS that may be configured. We are OK sending an LS to RAN1, if majority prefer. |
| Ericsson |  | Fine to go with Huawei suggestion and ask clarifications to RAN1. |
| Apple | Yes | Same view as Qualcomm. |
| OPPO | Yes | Same view as Qualcomm |
| Samsung | Yes | Assume to reflect RAN1 conslusion |
| Nokia | Yes | Looks okay |
| vivo | Yes |  |
| MediaTek | No | By current specification, the number of CSI-RS resources are only counted per FR without SCS constraint. The TP reverses current specification without actual RAN1 agreement, thus it is a NBC which is unacceptable to us. |

MMSE-IRC

[10] R2-2207640	CR to TS 38.306 on UE capability of MMSE-IRC receiver	China Telecom	CR	Rel-15	38.306	15.17.0	0775	-	F	NR_newRAT-Core

[11] R2-2207641	CR to TS 38.306 on UE capability of MMSE-IRC receiver	China Telecom	CR	Rel-16	38.306	16.9.0	0776	-	A	NR_newRAT-Core

CRs [10][11] propose new MMSE-IRC receiver capability shall be specified in TS 38.306 for Rel-15 based on RAN4 LS.

Q5: Do companies agree with the intention of the CRs [10][11]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes, but | Fine in principle. We do not see the need to talk about FDD/TDD differentiation for mandatory feature without signalling. Ideally, reference to RAN4 specification / section should be added so that it is clear what the corresponding feature is. |
| Ericsson | No | If the feature is mandatory for Rel-17, it seems odd to include an optional feature on 38.306 for Rel-15/16 that will later disappear from the specifications. Nothing would be broken if we do not capture anything. |
| Huawei, HiSilicon | Yes | Agree with the intention. |
| China Telecom | Yes | We think it is better to add the new MMSE-IRC receiver capability in R15 and R16 to make it clear. And we are also fine with Qualcomm’s suggestion on the detailed CR wording. |
| Samsung | Yes |  |
| Nokia | Yes, but the CR needs revisions | The CR is not totally ok although the intent may ok. The CR text talks about capability although this is feature without capability signaling. Also, the CR cover pages talks about new UE capability although this should be 'Optional features without UE radio access capability parameters'. The actual spec change seems to be correct but the cover page should be changed to indicate optional feature without UE capability to make sure that other companies won't propose a new capability as correction later. RAN4 had decided that "Optional without capability signalling for Rel-15 and Rel-16. Mandatory without capability signalling from Rel-17". Thus, no new UE capability should be specified for this as indicated in RAN4's UE feature list attached to the LS. This same feature list also says that gNB does not need to know about this. Also, in our view this feature does not impact Radio Network as the RAN4 UE feature list already mentions that the network does not need to know it. This means that the network does not need to implement anything due to this CR. 1st change: Cover page changes 2nd change: Description updated |
| vivo | Yes | Agree with the intention. |
| China Telecom | Yes | Thanks Nokia for the kind suggestion on the cover page and description update. We are fine to go for the revision suggested by Nokia. |
| MediaTek | No strong view | We are fine to capture RAN4 agreement in their feature list but in this case, we think RAN2 do not even mention the new hardware related capability which is optional without signaling in the Rel-15/16 specification in such a late stage. |

Aperiodic CSI-RS

[12] R2-2208027	Correction on crossCarrierA-CSI-trigDiffSCS-r16 (38.306)	Ericsson	CR	Rel-17	38.306	17.1.0	0786	-	A	LTE_NR_DC_CA_enh-Core

[13] R2-2208028	Correction on crossCarrierA-CSI-trigDiffSCS-r16 (38.306)	Ericsson	CR	Rel-16	38.306	16.9.0	0787	-	F	LTE_NR_DC_CA_enh-Core

CRs [12][13] proposed to clarify the meaning of the term. "A-CSI trigger" and to remove prerequisite capability description because the prerequisite had been already mandated to report.

Q6: Do companies agree with the intention of the CRs [12][13]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes |  |
| Ericsson | Yes (Proponent) |  |
| Apple | Ok |  |
| Huawei, HiSilicon | Yes |  |
| China Telecom | Yes |  |
| Samsung | Yes |  |
| Nokia | Yes |  |
| vivo | Yes |  |
| MediaTek | Yes |  |

PDCCH blind detection

[14] R2-2206911	Reply LS on PDCCH Blind Detection in CA (R1-2205320; contact: Huawei)	RAN1	LS in	Rel-17	NR_L1enh_URLLC-Core	To:RAN2

[15] R2-2208501	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-16	38.306	16.9.0	0789	-	F	NR_L1enh_URLLC-Core

[16] R2-2208502	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-17	38.306	17.1.0	0790	-	A	NR_L1enh_URLLC-Core

[17] R2-2208503	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-16	38.331	16.9.0	3429	-	F	NR_L1enh_URLLC-Core

[18] R2-2208504	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-17	38.331	17.1.0	3430	-	A	NR_L1enh_URLLC-Core

The TP in CRs [15][16][17][18] to capture agreements from RAN1 LS R2-2200079 and R2-2206911 [14]. Late non-critical extensions are added to avoid NBC problem.

Q6: Do companies agree with the intention of the CRs [15][16][17][18]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes |  |
| Ericsson |  | For 38.331, shouldn’t it be a choice structure between pdcch-BlindDetectionCA-MixedExt-r16 and pdcch-BlindDetectionCG-UE-MixedExt-r16? So the UE could report the CA value on the CA branch and the NR-DC value on the NR-DC branch. For 38.306: We don’t need to capture the aspects 2) and 3) from the coversheet (also pasted below) because they are already clear from ASN.1 2) Clarify that only one between pdcch-BlindDetectionCA-Mixed-r16 and pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16 can be reported by UE, only one between pdcch-MonitoringCA-r16 and pdcch-MonitoringCA-NonAlignedSpan-r16 can be reported by UE. 3) Clarify that pdcch-BlindDetectionMCG-UE-Mixed-r16 for MCG and pdcch-BlindDetectionSCG-UE-Mixed-r16 for SCG should be reported together if supported by UE. This sentence is awfully long. Possible to increase readability by splitting to more sentences?: “If a UE supports pdcch-BlindDetectionCA-Mixed-r16 or pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16, then the capability defined by pdcch-BlindDetectionCA-Mixed-r16 or pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16 is applied to the combination of pdcch-BlindDetectionMCG-UE-Mixed-r16 and pdcch-BlindDetectionSCG-UE-Mixed-r16, if a UE supports pdcch-BlindDetectionCA-Mixed-v16xy or pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-v16xy, then the capability defined by pdcch-BlindDetectionCA-Mixed-v16xy or pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-v16xy is applied to the combination of pdcch-BlindDetectionMCG-UE-Mixed-v16xy and pdcch-BlindDetectionSCG-UE-Mixed-v16xy correspondingly as defined in clause 10 in TS 38.213.” The formatting of the RRC CRs seems wrong. The spacing between lines is larger than it should be and some indentations are incorrect. If/when merged these should be fixed. |
| Apple | Yes, we agree with the intent. |  |
| OPPO (Zhe) | Yes |  |
| Huawei, HiSilicon | Yes | We share our understanding according to the comments from Ericsson, For 38.331, According to RAN1 LS, one combination of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) corresponds to one combination of (pdcch-BlindDetectionCA-r15, pdcch-BlindDetectionCA-r16) reported for FG 11-2c or FG 11-2g, that means pdcch-BlindDetectionCA-MixedExt-r16 and pdcch-BlindDetectionCG-UE-MixedExt-r16 can be reported simultaneously and they are one-to-one mapping. That’s why we put them together within one entry of a SEQUENCE type of list. For 38.306, The clarification of 2) 3) is for the legacy Rel-16 capability fields. For 2), in current spec, the pdcch-BlindDetectionCA-Mixed-r16 and pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16 are two separate capability fields both defined as OPTIONAL, then according to RAN1 LS, it should be clarified that only one between them can be reported. There is similar issue for 3). As Ericsson mentioned, to avoid confusion, for the extended combinations added in the CR, ASN.1 signalling has been improved to reflect such restrictions. But it is still necessary to make some clarification for the legacy fields in 38.306 since we use a backward-compatible way to introduce the extended combinations in addition to the existing ones. For the long sentence, perhaps we can have a combined description for legacy field and the extended capability fields, like “If a UE supports pdcch-BlindDetectionCA-Mixed or pdcch-BlindDetectionCA-Mixed-NonAlignedSpan, then the capability defined by pdcch-BlindDetectionCA-Mixed or pdcch-BlindDetectionCA-Mixed-NonAlignedSpan is applied to the combination of pdcch-BlindDetectionMCG-UE-Mixed and pdcch-BlindDetectionSCG-UE-Mixed, as defined in clause 10 in TS 38.213.” |
| China Telecom | Yes |  |
| Samsung | Yes |  |
| Nokia | Yes | Some feedback from our side: - For this change pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16, pdcch-BlindDetectoinCA-Mixed-NonAlignedSpan-v16xy we should have this note here again: “Only one between pdcch-BlindDetectionCA-Mixed-r16 and pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16 can be reported by UE.” - pdcch-MonitoringCA-NonAlignedSpan-r16 Similar comment, repeat the note of the capability above here: “Only one between pdcch-MonitoringCA-r16 and pdcch-MonitoringCA-NonAlignedSpan-r16 can be reported by UE.” |
| vivo | Yes |  |
| MediaTek | Yes | We agree with the intent. |

PUSCH repetition

[19] R2-2208505	Clarification on pusch-RepetitionTypeA-r16 capability	Huawei, HiSilicon	CR	Rel-16	38.306	16.9.0	0791	-	F	NR_L1enh_URLLC-Core

[20] R2-2208506	Clarification on pusch-RepetitionTypeA-r16 capability	Huawei, HiSilicon	CR	Rel-17	38.306	17.1.0	0792	-	A	NR_L1enh_URLLC-Core

CRs [19][20] propose to capture prerequisites for FG11-6 mentioned in RAN1 feature list, and also to clarify corresponding parameters for unlicensed band case.

Q6: Do companies agree with the intention of the CRs [19][20]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes, but | The first change in pusch-RepetitionTypeA-r16 does not seem necessary. It is merely explaining how the feature works. Usually, we only provide a pointer to the feature itself, and do not explain the feature itself in detail. |
| Ericsson | Yes | Agree with QC. |
| Apple | Yes but | While we agree with the intention and think that the CRs are ok in general, we think that the capability description needs an update (not directly related but should be corrected). The statement "Indicates whether the UE supports PUSCH transmission with or without slot aggregation” does not make sense since PUSCH transmission without slot aggregation is basic functionality and does not need to be covered by a UE capability. The feature is not about whether the UE supports slot aggregation or not (which is provided by of type2-PUSCH-RepetitionMultiSlots and pusch-RepetitionMultiSlots). Instead, it is about whether the UE supports the dynamic indication of the number of repetitions. In the RAN1 UE feature list, “with or without aggregation” was used because the dynamic indication could indicate a value of 1, meaning no slot aggregation, or a value of larger than 1, meaning with slot aggregation. We would suggest correcting the above statement by replacing the text with: “Indicates whether the UE supports the dynamic indication of the number of repetitions for PUSCH transmission, as specified in TS 38.214 [12], clause 6.1.2.1.” |
| OPPO (Zhe) | Yes, but | We suggest removing the first change because there is usually no detailed explanation of one feature in 306 spec. |
| Huawei, HiSilicon | Yes | We agree with Apple that the current field description doesn’t capture the actual meaning for the Rel-16 capability introduced by RAN1, thus it is necessary to make some correction to differentiate it from the Rel-15 capability. The text proposed by Apple is fine for us. |
| China Telecom | Yes |  |
| Samsung | Yes |  |
| Nokia | Yes, but | Agree with Qualcomm |
| vivo | Yes |  |
| MediaTek | Yes |  |

3.2 Second round: Further discussion

(TBD)

4 Conclusions

(TBD)

5 References

- R2-2207049	On early implementation and capability signaling of Power Class 1.5	MediaTek Inc.	discussion	Rel-16	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

- R2-2207094	Make PC1.5 an early implementation candidate	MediaTek Inc.	CR	Rel-16	38.331	16.9.0	3454	-	F	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

- R2-2207095	Make PC1.5 an early implementation candidate	MediaTek Inc.	CR	Rel-17	38.331	17.1.0	3455	-	A	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

- R2-2207085	PC1.5 and legacy power class capability reporting clarification	MediaTek Inc.	CR	Rel-16	38.306	16.9.0	0795	-	F	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

- R2-2207086	PC1.5 and legacy power class capability reporting clarification	MediaTek Inc.	CR	Rel-17	38.306	17.1.0	0796	-	A	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

- R2-2207113	Clarification on codebookParametersPerBC parameter for extension of CSI-RS capabilities reporting	MediaTek Inc.	CR	Rel-16	38.331	16.9.0	3452	-	F	NR_newRAT-Core, TEI16

- R2-2207114	Clarification on codebookParametersPerBC parameter for extension of CSI-RS capabilities reporting	MediaTek Inc.	CR	Rel-17	38.331	17.1.0	3453	-	A	NR_newRAT-Core, TEI16

- R2-2207331	Correction on beamManagementSSB-CSI-RS	Qualcomm Incorporated	CR	Rel-16	38.306	16.9.0	0765	-	F	TEI16

- R2-2207332	Correction on beamManagementSSB-CSI-RS	Qualcomm Incorporated	CR	Rel-17	38.306	17.1.0	0766	-	A	TEI16

- R2-2207640	CR to TS 38.306 on UE capability of MMSE-IRC receiver	China Telecom	CR	Rel-15	38.306	15.17.0	0775	-	F	NR_newRAT-Core

- R2-2207641	CR to TS 38.306 on UE capability of MMSE-IRC receiver	China Telecom	CR	Rel-16	38.306	16.9.0	0776	-	A	NR_newRAT-Core

- R2-2208027	Correction on crossCarrierA-CSI-trigDiffSCS-r16 (38.306)	Ericsson	CR	Rel-17	38.306	17.1.0	0786	-	A	LTE_NR_DC_CA_enh-Core

- R2-2208028	Correction on crossCarrierA-CSI-trigDiffSCS-r16 (38.306)	Ericsson	CR	Rel-16	38.306	16.9.0	0787	-	F	LTE_NR_DC_CA_enh-Core

- R2-2206911	Reply LS on PDCCH Blind Detection in CA (R1-2205320; contact: Huawei)	RAN1	LS in	Rel-17	NR_L1enh_URLLC-Core	To:RAN2

- R2-2208501	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-16	38.306	16.9.0	0789	-	F	NR_L1enh_URLLC-Core

- R2-2208502	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-17	38.306	17.1.0	0790	-	A	NR_L1enh_URLLC-Core

- R2-2208503	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-16	38.331	16.9.0	3429	-	F	NR_L1enh_URLLC-Core

- R2-2208504	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-17	38.331	17.1.0	3430	-	A	NR_L1enh_URLLC-Core

- R2-2208505	Clarification on pusch-RepetitionTypeA-r16 capability	Huawei, HiSilicon	CR	Rel-16	38.306	16.9.0	0791	-	F	NR_L1enh_URLLC-Core

- R2-2208506	Clarification on pusch-RepetitionTypeA-r16 capability	Huawei, HiSilicon	CR	Rel-17	38.306	17.1.0	0792	-	A	NR_L1enh_URLLC-Core