3GPP TSG-RAN WG2 #119 electronic	R2-220xxxx

Electronic Meeting, Aug 17th – 29th, 2022

Agenda Item:	5.1.3.1.2

Source:	Ericsson

Title:	Summary of offline [232][MUSIM] Potential clarifications to MUSIM (Ericsson)

Document for:	Discussion, Decision

1	Introduction

This contribution summarizes the following discussion:

[AT119-e][232][MUSIM] Potential clarifications to MUSIM (Ericsson)

Scope: Discuss the corrections for MUSIM marked for this discussion.

Intended outcome: Report in in R2-2208768.

Deadline: Deadline 1 (report)

Companies are invited to fill in contact details.

| Company | Contact details |
| --- | --- |
| Qualcomm | Ozcan Ozturk, oozturk@qti.qualcomm.com |
| ASUSTeK | Roger_Guo@asus.com |
| ZTE | Li.wenting@zte.com.cn |
| MediaTek | Felix Tsai, chun-fan.tsai@mediatek.com |
| OPPO | Jiangsheng Fan, fanjiangsheng@oppo.com |

2	Discussion

The paper in [1] contains the following proposal:

Proposal: Define the field musim-GapLength as Mandatory IE.

Since the CR related to this proposal is discussed in e-mail discussion 231, there is no need to repeat the discussion here, hence, the paper above will not be treated on this discussion.

The paper in [2] contains the following proposal:

Proposal: Update the procedure text such that how to perform the MUSIM gap configuration procedure is specified in a new clause. The draft TP in Annex can be considered as baseline.

Q1 Do companies agree with the proposal above? Please also provide comments, if any, to the draft TP in [2].

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | No strong preference since it is editorial |
| ASUSTeK | Yes | We are fine with the TP. |
| ZTE | Yes |  |
| MediaTek | Yes |  |
| OPPO | Yes |  |

Since [3] was revied into [4], only [4] will be treated in this document.

The latter paper contains the following proposals:

Proposal 1: RAN2 to confirm that MUSIM assistance information and signaling procedure for switching notifications are only carried out as MCG Configuration change for Rel-17.

Proposal 2: The gap configurations signalled from Master cell-group is used by UE to switch from NTWK-A completely including MCG and SCG operations for Rel-17.

Proposal 3: Cell-Group specific MUSIM Gap configuration and leave notification should be considered in Rel-18 WID.

Proposal 4:  Uplink transmission for SPS and CG are allowed during MUSIM Gap based on network control.

Proposal 5:  UE may indicate the support for uplink transmission during MUSIM Gap as optional capability

Proposal 6: RAN2 to consider inclusion of absence time or preferred return time to minimise the user plane data interruption and packet loss due to release of RRC connection for MUSIM switching for short absence.

Q2 Do companies agree with the proposals above? In the Yes/No column, please state whether you agree/disagree with each of the proposals above.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | None of these are essential corrections. P3 can be discussedin Rel-18 WI directly. We do not want to introduce any new signaling or UE behavior for MUSIM unless something is critcally broken, which is not the case here. If SN sends a MUSIM gap applicable to SCG configuration only (e.g. for FR1 MN and FR2 SN case) via SRB3, then it has no impact on MCG and also UE implementation. Such decision can be left to NW implementation. Our RAN2 agreement was not to introduce any additional signaling for DC so better leave at that. Rel-18 can of course introduce optimizations if there is support. |
| ASUSTeK | Agree: 1, 2, 3, 6 Disagree: 4, 5 | NW could prevent the collisions between SPS/CG and MUSIM Gap. |
| ZTE | No | Similar view as Qualcomm |
| MeidaTek | P1/P2 could discusse Disagree: P3 to P6 | P1/P2, prefer to discuss based on solid TP. P3 to P6 is clearly further optimization and should not be discussed in correction phase. |
| OPPO | See comments | For P1, P1 can be handled by UE implementation, if majority wants, we can follow. For P2, only per UE gap is introduced, so no doubt to cover both MCG and SCG operation, as for the limitation for MCG signaling, this can also be handled by UE implementation. For the rest proposals, it’s totally optimization for R17, not essential from our side. |

The CR in [5] intends to clarify that UE start or re-start of timer when UE assistance information triggering corresponds to MUSIM assistance information, upon receiving the field reconfigurationwithsync, is applicable only for MCG.

Q3 Do companies agree with the intention of the CR above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | As in Q2, nothing is broken. No new functionality is needed. |
| ASUSTeK | Yes |  |
| MediaTek | Yes |  |
| OPPO | No | The proposed change may also impact R16 UE behaviour as the change is not only added for MUSIM operation. |

The paper in [6] contains the following proposals:

Proposal 1: The PTW related parameters shall be included in GAP configuration.

Proposal 2: Three new parameters shall be added in UAI for GAP configuration:

- Timer 1: the length of PTW

- Timer 2: the length of non-PTW

- The SFN and sub frame of the beginning of PTW

Q4 Do companies agree with the proposals above? In the Yes/No column, please state whether you agree/disagree with each of the proposals above.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | This is against the very basic principle of MUSIM WI that NW A does not coordinate or optimize the UE behavior on NW B. It is a very specific optimization and thus can not be considered in essential correction phase. |
| ASUSTeK | No | The UE may request periodic gap not only for paging monitoring but also for other purposes, e.g. SSB detection, measurement. MUSIM gap in non-PTW may not always be a waste. |
| ZTE | No |  |
| MediaTek | No | Looks like an optimization |
| OPPO | No | Seems like optimization, not essential for R17. |

The paper in [7] contains the following proposal:

Proposal: The UE shall release the MUSIM gap upon initiating the RRC re-establishment procedure if the UE is not configured with conditionalReconfiguration, and the UE releases the MUSIM gap upon selecting a suitable NR cell and the selected cell is not one of the candidate cells for conditional handover, instead of upon receiving the RRC re-establishment message from the gNB.

Q5 Do companies agree with the proposal above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | The UE does release MUSIM gap configuation upon re-establishment initiation. We don’t need to optimize this for CHO recovery. |
| ASUSTeK | Yes |  |
| ZTE | No | Agree with Qualcomm |
| MediaTek | No |  |
| OPPO | No | Seems like optimization |

The paper in [8] contains the following proposals:

Proposal 1: Upon initiation of re-establishment procedure, the UE stops timer T346g, if running.

Proposal 2: RAN2 to discuss whether to add the following NOTE in clause 5.3.7.2 as follows:

NOTE: It is up to UE implementation whether to initiate the procedure while T346g timer is running.

Q6 Do companies agree with the proposals above? In the Yes/No column, please state whether you agree/disagree with each of the proposals above.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | It wouldn’t make sense for a UE to initiate re-establishment and then leave NW connection due to T346g. So this can be left to the UE implementation. Even if a UE does this, nothing is broken as a UE can always abort re-establisment for other reasons. |
| ASUSTeK | Agree: 1 Disagree: 2 | Our understanding on the previous agreement is that the UE would initiate the RRC re-establishment regardless of T346g. |
| ZTE | Agree with P1 |  |
| MediaTek | P1: Disagree P2: No strong view | Similar view as QC, it makes no sense to trigger re-estabilishment while T346g is running. |
| OPPO | No | As mentioned by Qualcomm, we also think a good UE implementation will aviod this bad scenario. |

3	Conclusion

- To be updated after discussion on section 2 -

4	References

- R2-2208032	Discussion on gap length IE optionality	Ericsson	discussion, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2208344	Clarification on performing MUSIM gap configuration procedure	Samsung Electronics Co., Ltd	discussion	Rel-17, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2208035	On Remaining Issues ofr MUSIM Switching Procedures	Nokia, Nokia Shanghai Bell	discussion	Rel-18, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2208683	On Remaining Issues ofr MUSIM Switching Procedures	Nokia, Nokia Shanghai Bell	discussion	Rel-17, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2207994	Clarification for MUSIM Assistance Information in DC for reconfiguration with Sync 	Nokia, Nokia Shanghai Bell	CR	Rel-17, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2207670	Support eDRX in Multi-SIM scenario	Spreadtrum Communications	discussion	Rel-17, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2207961	Discussion on the MUSIM gap release during RRC reestablishment	Huawei, HiSilicon	discussion	Rel-17, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022

- R2-2208369	Further discussion on re-establishment handling while T346g timer is running	Samsung Electronics Co., Ltd	discussion	Rel-17, RAN2#119-e, Eletronic Meeting, Aug 17th – 29th, 2022