3GPP TSG-RAN WG2 #119bis-e	R2-2209378

E-meeting, October 2022

Agenda Item:	6.7.2.2

Source:	OPPO

Title:	Discussion on left issues for CP

Document for:	Discussion, Decision

Introduction

This is to discuss the left issue for control plane.

Discussion

In 119, the following change has been agreed,

For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the Uu Relay RLC Channel of the LCID from which the SRAP Data PDU is received

I.e., the UE receives the SRAP PDU, will decide whether it is for SRB or DRB, by checking the ingress LCID, and comparing it with the egress RLC channel configuration, to see whether it matches the RLC channel for SRB or DRB.

SL-MappingToAddMod-r17 ::=              SEQUENCE {

sl-RemoteUE-RB-Identity-r17             SL-RemoteUE-RB-Identity-r17,

sl-EgressRLC-ChannelUu-r17              Uu-RelayRLC-ChannelID-r17                                        OPTIONAL, -- Cond L2RelayUE

sl-EgressRLC-ChannelPC5-r17             SL-RLC-ChannelID-r17                                             OPTIONAL, -- Need N

...

}

SL-RemoteUE-RB-Identity-r17 ::=         CHOICE {

srb-Identity-r17                        INTEGER (0..3),

drb-Identity-r17                        DRB-Identity,

...

}

This SRB/DRB differentiation operation is needed at 3 stages

- When Relay-UE receives a SRAP PDU from DL

- When Remote-UE receives a SRAP PDU from SL

- When Relay-UE receives a SRAP PDU from SL

Where for A, it is the network's responsibility to ensure the DL SRAP PDU comes to Relay-UE via the corresponding RLC channel / LCID. So the issue is only for B and C.

For B and C, if we take B as an example, what Remote-UE gets is

- A SRAP configuration from network, including the egress RLC channel configuration, i.e., RLC channel ID and LCID

- A list of RLC channel configurations from Relay-UE, for the RLC channel where Relay-UE as Tx and Remote-UE as Rx, i.e., for the ingress RLC channel configuration, including RLC channel and LCID.

SL-RLC-ChannelConfigPC5-r17::=          SEQUENCE {

sl-RLC-ChannelID-PC5-r17                SL-RLC-ChannelID-r17,

sl-RLC-ConfigPC5-r17                    SL-RLC-ConfigPC5-r16                                                OPTIONAL, -- Need M

sl-MAC-LogicalChannelConfigPC5-r17      SL-LogicalChannelConfigPC5-r16                                      OPTIONAL, -- Need M

...

}

I.e., essentially, the checking operation above is for remote UE to compare 1) the egress RLC channel configured by the network and 2) the ingress RLC channel configured by Relay-UE. Surely we can assume network will ensure for a same bearer (either SRBx or DRBx), it provides compatible configuration to Remote-UE and Relay-UE, i.e., configuring egress RLC channel ID X (of IE SL-RLC-ChannelID-r17) to both Remote-UE and Relay-UE. However, since the LCID allocation is done by UE, remote-UE cannot check correspondence based on LCID, but can only base on RLC channel ID.

Then it ends up with that, it requires Relay-UE, when sending RRCReconfigurationSidelink message to Remote-UE, to configure the ingress RLC channel, the field setting of sl-RLC-ChannelID-PC5-r17, has to be aligned with the sl-EgressRLC-ChannelPC5-r17, which is obtained from network.

Similarly, for C above, the requirement is on Remote-UE side.

- R2 confirms Tx-UE ensures alignment between sl-RLC-ChannelID-PC5-r17 (in PC5-RRC) and sl-EgressRLC-ChannelPC5-r17 (in Uu-RRC).

NOTE 3:	For L2 U2N Remote UE in RRC_IDLE, the cell (re)selection procedure as specified in TS 38.304 [20] and relay (re)selection procedure as specified in 5.8.15.3 are performed independently and up to UE implementation to select either a cell or a L2 U2N Relay UE.

Based on the NOTE above, UE should perform the two procedures independently

- Cell selection and reselection,

- Relay selection and reselection

While there is common understanding between companies on reselection, it seems beneficial to confirm this NOTE covers initial cell selection and relay selection as well. E.g., when a UE power-on in out-of-coverage scenario (out of direct path coverage), it is allowed to camp on a relay, i.e., no need to require the UE to firstly go to direct path coverage to camp on a direct Uu coverage, and then back to indirect path coverage to benefit from extended coverage from R17 Relay.

- Based on NOTE3 in TS 38.331 clause 5.3.3.3, 5.3.13.3, 5.8.15.3, R2 confirms UE can camp on a relay UE upon power-on, and it is up to UE implementation to camp on either cell or relay when both are available.

And considering cell selection is specified in 304, where so far there is no description of the feasibility of selecting a relay instead of a cell, it would be helpful to clarify it explicitly in 304, e.g., using a NOTE as follows in

NOTE:	If both suitable cells and suitable NR sidelink U2N Relay UEs (as specified in TS 38.331 [3]) are available, it is up to NR sidelink U2N Remote UE implementation to select either a cell or a NR sidelink U2N Relay UE.

- Add a NOTE “If both suitable cells and suitable NR sidelink U2N Relay UEs (as specified in TS 38.331 [3]) are available, it is up to NR sidelink U2N Remote UE implementation to select either a cell or a NR sidelink U2N Relay UE.” Into TS 38.304 clause 5.2.1.

Conclusion

We have the following proposals:

Proposal 1	R2 confirms Tx-UE ensures alignment between sl-RLC-ChannelID-PC5-r17 (in PC5-RRC) and sl-EgressRLC-ChannelPC5-r17 (in Uu-RRC).

Proposal 2	Based on NOTE3 in TS 38.331 clause 5.3.3.3, 5.3.13.3, 5.8.15.3, R2 confirms UE can camp on a relay UE upon power-on, and it is up to UE implementation to camp on either cell or relay when both are available.

Proposal 3	Add a NOTE “If both suitable cells and suitable NR sidelink U2N Relay UEs (as specified in TS 38.331 [3]) are available, it is up to NR sidelink U2N Remote UE implementation to select either a cell or a NR sidelink U2N Relay UE.” Into TS 38.304 clause 5.2.1.