3GPP TSG-RAN WG2 #119bis-e	R2-2209526

Electronic meeting, October 2022

Agenda Item:	6.10.4.1

Source:	Ericsson

Title:	On neighbour cell SI

Document for:	Discussion

# 1	Discussion

The neigh cell SI design was done in a very last minute of Rel-17 and proper discussions did not take place. In previous meeting it was agreed to make  NBC change to increase the number of elements in the list from 4 to 8 by adding an extension of 4 elements. As such, this is not a very nice to extend lists within a release like this as it is not clear if all UEs will support also the extended list.

However, it provides opportunity to have enhanced signalling with the yellow highlighted sentence which is what included in the RAN2 approved RRC CR. That is, if the extended list does not include ntn-Config, the same ntn-Config applies to this PCI as for the PCI in the first list in the corresponding location in the list.

In our view, since release 17 is closed and intention is to do bug fixes only, not even editorial corrections, it is not according to the spirit of bug fixes to spend time on further revising this sentence for further enhancements.

#### –	SIB19

SIB19 contains satellite assistance information for NTN access.

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

... ,

[[

ntn-NeighCellConfigListExt-v17xx              NTN-NeighCellConfigList-r17                     OPTIONAL       -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

-- TAG-SIB19-STOP

-- ASN1STOP

SIB19 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m.

ntn-Config Provides parameters needed for the UE to access NR via NTN access such as Ephemeris data, common TA parameters, k_offset, validity duration for UL sync information and epoch.

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList (without suffix) and all elements of ntn-NeighCellConfigList-v17xx. If ntn-Config is absent for an entry in ntn-NeighCellConfigListExt, the ntn-Config provided in the entry at the same position in ntn-NeighCellConfigList applies.

referenceLocation Reference location of the serving cell provided via NTN quasi-Earth fixed system and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]..

t-Service Indicates the time information on when a cell provided via NTN quasi-Earth fixed system is going to stop serving the area it is currently covering. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).. The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field.

RAN2 does not enhance further the release 17 neighbour cell SI broadcasting

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 does not enhance further the release 17 neighbour cell SI broadcasting