3GPP TSG RAN WG2#119bis electronic                                                R2-2209570

Electronic, 10th - 19th October, 2022

Source:	CATT

Title:	Discussion on Inter-RAT Signaling Based Logged MDT Override Protection

Agenda Item:	8.13.3

Document for:	Discussion and Decision

Introduction

In RAN2#119e meeting, signalling based logged MDT override protection in inter-RAT scenario was discussed and the following agreements were achieved [1].

Agreement:

1	RAN2 confirms the valid scenario for Rel-18 inter-RAT scenario for signalling based logged MDT override protection is set by the WID:

a.	Logged MDT is configured in E-UTRAN, the UE reselects to NR.

2	Rel-17 mechanism for signalling based logged MDT override protection in intra-NR scenario is the baseline for Rel-18 inter-RAT scenario.

In this contribution, we will continue to discuss signalling based logged MDT override protection in inter-RAT scenario in section 2.

Discussion

- Scenario to be addressed in R18

In current R18 SON MDT WID [2], it is indicated we will address the case that the UE is configured sig-based logged MDT configuration in E-UTRAN cell, but UE reselects to NR cell while logged measurements are collected or after logged measurements are collected and before uploading the logged MDT report in E-UTRAN. But, whether to address the reverse scenario i.e. when the UE is configured sig-based logged MDT in NR and the UE reselects to E-UTRAN cell while logged measurements are collected or after logged measurements are collected and before uploading the logged MDT report in NR has no consensus. In our understanding, if the reverse scenario is agreed to be addressed in R18, then may introduce many eNB impacts which do not we want to see. Therefore, we suggest that only the scenario in current WID is addressed in R18, the reverse scenario is excluded in R18 scope.

Proposal 1: Exclude the scenario i.e. the UE is configured with NR sig-based logged MDT measurement configuration/result reselects to E-UTRAN in R18 scope.

- Solutions to address the case which is set by the WID

In last meeting, it was agreed that Rel-17 mechanism for signalling based logged MDT override protection in intra-NR scenario is the baseline for Rel-18 inter-RAT scenario. In R17, the signaling based logged MDT override protection mechanism is that the network configures the logged MDT measurement configuration and provides an indicator in the logged MDT configuration to UE to indicate there is signaling based logged MDT configuration. When the UE has stored the signaling based logged MDT configuration or signaling based logged MDT measurement results, the UE will provide the available indicator in the UL complete message to the network to avoid network sending management based logged MDT measurement configuration to UE which will override the existing configuration or results.

For inter-RAT scenario, if a UE is configured with signaling based logged MDT in E-UTRAN, UE reselects to NR cell while logged measurements are collected or after logged measurements are collected and before uploading the logged MDT report in E-UTRAN, in order to avoid the UE is configured with NR management based logged MDT configuration which will override the E-UTRAN signaling based logged MDT, there are two options to address the issue:

Option 1: UE provides the available indicator to NR network to avoid NR override configuration

Option 2: UE provides the available indicator to NR network and NR can request the E-UTRAN results which is not retrieved

For Option 1, it seems to be a simpler way which only needs to introduce a signaling based logged MDT indicator in E-UTRAN logged MDT measurement configuration, the UE could report the available indicator to NR network if there is E-UTRAN signaling based logged MDT measurement configuration or results in UE as current R17 mechanism. But it could cause the NR network cannot configure the management logged MDT configuration to UE during the UE connected to the NR network. For Option 2, it will allow the inter-RAT logged MDT measurement results reporting which will change the current mechanism and introduce some complexity.

We prefer to Option 1 to provide a simpler way to address the inter-RAT scenario mentioned above.

Proposal 2 RAN2 to discuss the following two options to address the inter-RAT scenario in R18 WID for signaling based logged MDT override protection.

Option 1: UE provides the available indicator to NR network to avoid NR override configuration

Option 2: UE provides the available indicator to NR network and NR network can request the E-UTRAN results which is not retrieved

Conclusion

According to the discussion in section 2, we propose:

Proposal 1: Exclude the scenario i.e. the UE is configured with NR sig-based logged MDT measurement configuration/result reselects to E-UTRAN in R18 scope.

Proposal 2: RAN2 to discuss the following two options to address the inter-RAT scenario in R18 WID for signaling based logged MDT override protection.

Option 1: UE provides the available indicator to NR network to avoid NR override configuration

Option 2: UE provides the available indicator to NR network and NR network can request the E-UTRAN results which is not retrieved

Reference

- R2_119-e_meeting_report_v1

- RP-221825 Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity)