3GPP TSG-RAN WG2 Meeting #119bis electronic	R2-2209847

Online, October 10 – 19, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3512 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SL DRX operation for U2N Remote UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Rel-17, SL DRX is supported for L2 U2N Relay. The current RRC specification should be allowed for a L2 relay UE to configure a L2 remote UE in RRC_INACTIVE/IDLE with a SL DRX configuration for e.g. receiving paging. However, since the L2 remote UE may also receive and accept another SL DRX configuration provided by its peer UE for ProSe unicast communication and/or also apply the other SL DRX configuration for ProSe broadcast/groupcast communication, the active time of the SL DRX configuration for ProSe relay communication may not be aligned with the active time of the SL DRX configuration(s) for ProSe unicast/broadcast/groupcast communication. Therefore, it should be reasonable for the L2 remote UE to send its accepted or applied SL DRX configuration(s) for ProSe unicast/broadcast /groupcast communication as assistance information to its L2 relay UE for aligning overall active time of SL monitoring in the L2 remote UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 5.8.9.6.1, the statement of “For NR sidelink L2 U2N relay communication, the L2 U2N Remote UE may determine its desired sidelink DRX configurations taking its accepted sidelink DRX configurations for other peer UEs or its applied sidelink DRX configurations for sidelink groupcast/broadcast communication into account.” is added in the NOTE. Impact analysis Impacted functionality(ies): Sidelink UE assistance information. Inter-operability: - If the UE is implemented according to this CR but the peer UE is not, there is no inter-operability issue foreseen. - If the UE is implemented according to this CR but the network is not, there is no inter-operability issue foreseen. - If the network is implemented according to this CR but the UE is not, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The L2 remote UE may consume more power if the L2 remote UE cannot provide the accepted or applied SL DRX configuration(s) for ProSe unicast/broadcast /groupcast communication to its L2 relay UE for aligning overall active time of SL monitoring in the L2 remote UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of 1st Change * * * *

##### 5.8.9.6.1	General

Figure 5.8.9.6.1-1: Sidelink UE assistance information

The purpose of this procedure is for a UE to inform its peer UE of the sidelink DRX assistance information used to determine the sidelink DRX configuration for unicast communication.

For sidelink unicast, a UE may include its desired sidelink DRX configurations in the UEAssistanceInformationSidelink as the sidelink DRX assistance information which is transmitted to its peer UE.

NOTE:	It is up to UE implementation to determine its desired sidelink DRX configurations for unicast communication. For NR sidelink L2 U2N relay communication, the L2 U2N Remote UE may determine its desired sidelink DRX configurations taking its accepted sidelink DRX configurations for other peer UEs or its applied sidelink DRX configurations for sidelink groupcast/broadcast communication into account.

* * * End of Changes * * * *