3GPP TSG-RAN WG2 Meeting #119-bis Electronic	R2-2210348

# Elbonia, 10 – 19 October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0289 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of enhanced and relaxed cell reselection for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce the support for enhanced and relaxed NTN cell reselection, as requested in R4-2214472. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 5.2.4.2 the description is added regarding how the UE performs measurements for cell reselection if enhancedNTN-Reselection or relaxedNTN-Reselection is signalled in SIB2. Impact analysis Impacted functionality: Cell reselection in NTN Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE cannot use enhanced and relaxed cell reselection for that cell and will not comprehend the corresponding fields in SIB2. - If the UE is implemented according to the CR and the network is not, there is no enhanced/relaxed cell reselection signalled in SI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhanced and relaxed NTN cell reselection is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 CR 3540 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

#### 5.2.4.2	Measurement rules for cell re-selection

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation and has obtained its location information:

-	If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

-	Else, the UE shall perform intra-frequency measurements;

-	Else, the UE may not perform intra-frequency measurements;

-	Else, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8].

-	For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation and has obtained its UE location information:

-	If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8];

-	Else, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8].

-	If the UE supports relaxed measurement and relaxedMeasurement is present in SIB2, the UE may further relax the needed measurements, as specified in clause 5.2.4.9.

If the t-Service of the serving cell is present in SIB19, and if UE supports time-based measurement initiation, the UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of the distance between UE and the serving cell reference location or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, The exact time to start measurement before t-Service is up to UE implementation. UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8] regardless of the remaining service time of the serving cell (i.e. time remaining until t-Service).

NOTE:	When evaluating the distance between UE and the serving cell reference location, it's up to UE implementation to obtain UE location information.

If the enhancedNTN-Reselection is present in SIB2 and the UE supports NTN LEO, the UE shall perform enhanced cell reselection measurements according to TS 38.133 [8]. If the relaxedNTN-Reselection is present in SIB2 and the UE supports NTN GEO, the UE shall perform relaxed cell reselection measurements according to TS 38.133 [8].

End of Modified Subclause