3GPP TSG-RAN WG2 Meeting #119bis-e	R2-2210722

Online, 10 - 19 October, 2022

Agenda item:	8.4.2.1

Source: 	Huawei, HiSilicon

Title: 	Target Performance Enhancements and supported scenarios

Document for:	Discussion and Decision

- Introduction

L1/L2 based inter-cell mobility is a direction of Rel-18 Mobility Enhancement, which aims at reducing the latency during handover [1]. In RAN2 #119-e meeting, RAN2 discussed the latency model and defined the handover interruption time for L1/L2 mobility. In this contribution, we analyse which components in the latency model can be enhanced to reduce the overall handover latency.

- Discussion

## 2.1 Components reducing latency

In a post email discussion of RAN2 #119-e [2], the rapporteur suggests a latency model for L1/L2 mobility, as shown below.

Component; Meaning; Value. TRRC; Processing time for RRCReconfiguration carrying candidate configurations; Up to [10] ms. Tprocessing,1 / Tprocessing,2; Time for UE processing, before and after cell switch command, respectively. This may include L2/3 reconfiguration, RF retuning, baseband retuning, security update if needed, etc.; Up to [20] ms for same FR Up to [40] ms for different FR. Tmeas; Measurement delay (from target appears to cell switch command); -. Tcmd; Time for processing L1/L2-command (HARQ and parsing); Up to [5] ms. Tsearch; Time required to search the target cell; 0ms (if cell is known) Up to [60] ms (if cell is unknown). TΔ; Time for fine tracking and acquiring full timing information; SMTC periodicity (typ. [20] ms). Tmargin; Time for SSB or CSI-RS post-processing; Up to [2] ms. TIU; interruption uncertainty in acquiring the first available PRACH occasion in the new cell; Typ. [15] ms. TRAR; Time for RAR delay; Typ. [4] ms. Tfirst-data; Time for UE performs the first DL/UL reception/ transmission on the indicated beam of the target cell, after RAR; -.

In this latency model, there are three main steps: 1) UE reconfiguration, 2) DL synchronization and 3) UL synchronization. In the following text, we discuss based on these three steps.

### 2.3.1 UE reconfiguration processing

In the proposed latency model, before receiving a L1/L2 handover command, the UE needs to perform RRC processing and UE processing (part 1). During this time period, the UE receives the pre-configuration of candidate cells via RRCReconfiguration message. The pre-configuration includes the L1 measurement resources of candidate cells, if the UE only stores the pre-configuration but does not process it, the UE is not able to obtain the configuration of RSs to be measured. Therefore, the UE should decode ASN.1 upon receiving the RRC pre-configuration message. This also helps to reduce the handover interruption time, because the UE can apply the target cell’s configuration faster if the ASN.1 has been decoded.

Proposal 1: Confirm that the UE is expected to decode the ASN.1 upon receiving the RRC pre-configuration message. The UE applies the indicated target configuration upon receiving L1/L2 handover command.

After a L1/L2 handover is executed (e.g., from cell 1 to cell 2 as shown in the figure below), it may be useful execute a subsequent handover (e.g., from cell 2 to cell 3) if another cell has become better. If there would be the need for an RRC reconfiguration before the next L1/L2 handover, this would increase the signalling and might delay that L1/L2 handover.

- Cell1 to cell2 and then to cell3;      2) Cell1 to Cell2 and back to Cell1

Figure 1: Two examples of subsequent L1/L2 handover.

Proposal 2: Support the execution of multiple subsequent L1/L2 handovers without the need for an RRC reconfiguration, e.g. to reduce the latency of RRC processing and UE processing for the subsequent mobility scenario (UE moving fast or moving back and forth).

### 2.3.2 L2 reset

UE processing (part 2) in the latency model is the UE processing time after receiving the L1/L2 handover command. Within this time period, the UE applies the target configuration and performs corresponding L2 behaviours (PDCP/RLC/MAC). In RAN2 #119-e meeting, RAN2 had the following assumption on L2 behaviours.

RAN2 #119-e agreement R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery.

PDCP

As shown in the WID [1], intra-CU is the scenario we consider for L1/L2 mobility, which indicates the PDCP anchor is not changed. Since the PDCP anchor remains unchanged, the security key can remain unchanged as well. Therefore, PDCP re-establishment can be avoided, which helps to reduce the handover interruption time.

Proposal 3: PDCP anchor is not changed in Rel-18 L1/L2 handover and security key remains unchanged.

Proposal 4: There is no need of PDCP re-establishment in Rel-18 L1/L2 handover.

Regarding AM DRB, RLC SDUs and RLC PDUs are discarded if RLC is re-established (e.g., in inter-DU L1/L2 handover). To avoid data loss, PDCP data recovery is needed to re-transmit some PDCP Data PDUs.

Regarding SRB, in L3 handover, the network can set discardOnPDCP to true in L3 handover command. In this case, the PDCP entity of the UE shall discard all stored PDCP SDUs and PDUs. The benefit is that the UE does not need to transmit some signalling (e.g., the L3 measurement results of the source cell) which is not valid for the target cell. In L1/L2 handover, the SRB discard behaviour should also be considered.

Proposal 5: Support the use of PDCP data recovery for AM DRB in Rel-18 L1/L2 handover.

Proposal 6: Support the use of PDCP discard for SRB in Rel-18 L1/L2 handover (e.g., to discard invalid L3 measurement report).

RLC

In the network side, RLC function locates at the DU in CU-DU split architecture. In inter-DU handover, if the UE continues to use the variables of the source cell to transmit to the target cell, the SN of the RLC PDU is very likely to fall outside of the receiving window of RLC receiving entity of the target DU. This mismatch will lead to wrong behaviour in RLC layer. Therefore, RLC re-establishment is needed in inter-DU handover. However, when we consider the intra-DU case, the UE and the DU can still align the running situation of the timers and the values of the state variables, RLC function can continue in this case. If the RLC transmission and reception procedures are continued, some unnecessary re-transmission can be avoided, and the interruption of data transfer is reduced.

Proposal 7: The network should re-establish RLC in inter-DU case, and can choose not to re-establish RLC in intra-DU case.

MAC

Some functions of the MAC layer is closely related to “cell”. For instance, the HARQ procedure is for each serving cell, and the beam failure detection is for each serving cell as well. Consider a scenario that the UE’s BFI_COUNTER is accumulated to a not low number in the source cell, then if the UE continues this counter in the target cell, it is likely that when the target cell beam condition suffers degradation for a short period of time, the counter reaches the threshold and the UE declares beam failure in the target cell. Then the UE initiates RACH to recover the beam. This is not reasonable because the target cell beam may become good very soon.

Observation 1: No action at all on MAC is not suitable even in intra-DU case, because there are cell level MAC variables that should not be kept after moving to the target cell.

Since some action on MAC are necessary, we consider whether a full or a partial MAC reset should be performed. We understand that the processing time difference for performing complete MAC reset or partial MAC reset is minor, because in both cases the MAC entity needs to stop some timers and initialize some variables. From the UE processing time perspective, we do not see the motivation to do only a partial MAC reset.

Observation 2: From UE processing time perspective, the benefit of performing partial MAC reset rather than full MAC reset is minor.

### 2.3.3 UL synchronization

In the legacy NR L3 mobility, the UE obtains the TA towards the target cell and completes the UL synchronization via the RACH procedure. With CFRA in 4-step RACH or CBRA in 2-step RACH, the UE can complete the RACH procedure by two steps (Msg1+Msg2 or MsgA+MsgB), which is faster than CBRA in 4-step RACH (Msg1 to Msg4). However, even with the CFRA in 4-step RACH or CBRA in 2-step RACH, the UE has to wait for the available PRACH occasion, send the preamble and wait for the RAR. The time for the RACH procedure is around 10~20 ms.

“Timing Advance management” is one objective in Rel-18 L1/L2 mobility, as shown in the WID [1]. We understand the TA management here refers to TA management on the candidate/target cell, in order to avoid RACH during handover and reduce handover latency.

Proposal 8: Consider procedures/solutions to avoid RACH during Rel-18 L1/L2 handover, at least in case the UL TA towards the target cell is available before mobility.

In LTE, RACH-less solutions were studied and specified to reduce the interruption time during handover. Two cases were agreed: one is the TA of the target cell equals to 0, and the other one is the target cell has the same TA with the source cell. In Rel-18 L1/L2 mobility, we can also consider these two cases to avoid RACH. In addition, since “TA management” is in the scope and lead by RAN1, we expect RAN1 will work on the solution to obtain TA of the candidate/target cell, which can enable RACH-less handover.

Proposal 9: Ask RAN1 to work on solutions to obtain the TA towards the candidate target cell before L1/L2 handover.

## 2.4 Supported CA scenarios

In RAN2 119-e meeting, there is an agreement related to the applicable scenarios of Rel-18 L1/L2 mobility.

RAN2 #119-e agreement R2 assumption: Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell). This includes the following cases - a) the target PCell/target SCell(s) is not a current serving cell (CA CA scenario with PCell change) - b) FFS the target PCell is a current SCell - c) FFS the target SCell is the current PCell.

Regarding to the FFS b) in the above agreement, the following figure is this scenario: the target PCell is a current SCell.

Figure 2: An example of mobility with CA.

When the UE applies CA in the source side, the UE performs L1 measurement on both PCell (Cell 1) and SCell (Cell 2) for beam management and for channel state information reporting. In this case, the network has already obtained the L1 report of Cell 2. When the UE moves to the target side, it is possible for the network to indicate the UE handover to the target PCell (Cell 2). Though the centre frequency of the source PCell and the target PCell is different, this does not require the UE to perform inter-frequency neighbour cell L1 measurement.

Observation 3: The network can decide to change the PCell to a cell on a current SCell frequency using the intra-frequency L1 measurement results configured for that SCell (as can be configured in Rel-15/16/17).

Proposal 10: L1/L2 mobility support inter-frequency PCell change to a PCell frequency that is the frequency of an SCell before mobility, based on Rel-15/16/17 L1 intra-frequency measurements configured on the SCell before mobility.

For the above CA scenario, if the UE is the target PCell of if the L1 measurements performed on the SCell before mobility imply that the UE knows the full DL timing information of the target PCell, the DL synchronization latency could be reduced to close to 0 ms. In addition, if the target PCell is the SCell and it has UL, the network and the UE know the TA towards the target PCell, so RACH-less handover is possible.

DL synchronization is the procedure in which the UE measures the downlink signals of the target cell and acquires the DL timing information of the target cell. RAN4 defined three latency components for DL synchronization in L3 handover, which are the time for cell search (Tsearch), the time for fine tracking (TΔ), and the time for post processing (Tmargin). The time for cell search can be saved if the target cell is a “known” cell and the time for fine tracking is related to the SMTC periodicity of the target cell.

According to the latency model, the typical delay for DL synchronization procedure is large (around 20 ms, depends on SMTC periodicity) even the target cell is a known cell to the UE. The solution on reducing DL synchronization is worth discussing in RAN4.

Proposal 11: Ask RAN4 to study the DL synchronization delay for the above scenario i.e. target PCell frequency is the frequency of an SCell before mobility and handover is triggered by L1 intra-frequency measurements configured on the SCell before mobility.

Proposal 12: If RAN2 designs RACH-less L1/L2 mobility when the TA of the target PCell is already known to the UE before L1/L2 handover (as in proposal 8), include the case when the TA is known because it is the same as a previous serving cell.

## 2.5 Support inter-frequency scenarios

Based on the WI, the inter-frequency L1/L2 mobility is clearly in the scope. Considering the L1 measurement is agreed for the L1/L2 mobility, RAN1/RAN4 should discussion the details on inter-frequency L1 measurement.

Proposal 12: Ask RAN1 and RAN4 to work on the inter-frequency L1 measurement.

- Conclusion

In this contribution, we have the following observations and proposals:

Latency to reduce:

Proposal 1: Confirm that the UE is expected to decode the ASN.1 upon receiving the RRC pre-configuration message. The UE applies the indicated target configuration upon receiving L1/L2 handover command.

Proposal 2: Support the execution of multiple subsequent L1/L2 handovers without the need for an RRC reconfiguration, e.g. to reduce the latency of RRC processing and UE processing for the subsequent mobility scenario (UE moving fast or moving back and forth).

Proposal 3: PDCP anchor is not changed in Rel-18 L1/L2 handover and security key remains unchanged.

Proposal 4: There is no need of PDCP re-establishment in Rel-18 L1/L2 handover.

Proposal 5: Support the use of f PDCP data recovery for AM DRB in Rel-18 L1/L2 handover.

Proposal 6: Support the use of PDCP discard for SRB in Rel-18 L1/L2 handover (e.g., to discard invalid L3 measurement report).

Proposal 7: The network should re-establish RLC in inter-DU case, and can choose not to re-establish RLC in intra-DU case.

Proposal 8: Consider procedures/solutions to avoid RACH during Rel-18 L1/L2 handover, at least in case the UL TA towards the target cell is available before mobility.

Proposal 9: Ask RAN1 to work on solutions to obtain the TA towards the candidate target cell before L1/L2 handover.

Supported scenario:

Proposal 10: L1/L2 mobility support inter-frequency PCell change to a PCell frequency that  is the frequency of an SCell before mobility, based on Rel-15/16/17 L1 intra-frequency measurements configured on the SCell before mobility.

Proposal 11: Ask RAN4 to study the DL synchronization delay for the above scenario i.e. target PCell frequency is the frequency of an SCell before mobility and handover is triggered by L1 intra-frequency measurements configured on the SCell before mobility.

Proposal 12: If RAN2 designs RACH-less L1/L2 mobility when the TA of the target PCell is already known to the UE before L1/L2 handover (as in proposal 8), include the case when the TA is known because it is the same as a previous serving cell.

- References

- RP-221799, Revised WID on Further NR mobility enhancements.

- R2-2209256, Report of [Post119-e][036][feMob] Time Chart.