3GPP TSG-RAN WG2 Meeting #119bis-e	R2-2210747

# Electronic meeting, 10th Oct. – 19th Oct. 2022

Agenda item:	7.2.4

Source: 	CATT

Title: 	Discussion on the NTN configuration at CHO

Document for:	Discussion and Decision

Introduction

In [offline-105] RRC corrections of last meeting, the NTN configuration of SIB31 in CHO was discussed. And the discussion was postponed based on the companies’ comments in the discussion. In this contribution, we give our view on this issue.

Discussion

In the 36.331 v17.2.0 version, the SystemInformationBlockType31 is still provided in RRCConnectionReconfiguration message, as a mandatory field with the condition of handover to an NTN cell. The main concern is, for the CHO case, the NW may have no knowledge of when the actual handover will actually take place, and then it is possible that the configurations of SIB31 have become out-of-date, when the UE performs the CHO. This will result in unnecessary signaling overhead.

However, if we try to relax the conditionally mandatory requirement, it is also difficult for the network to determine whether to include SIB31 when providing CHO configuration to UE, if the network has no knowledge of when the actual handover will actually take place. It is simple for the network just providing, maybe trying its best, SIB31 in CHO configuration for information, and leaving whether using the SIB31 in CHO configuration or not to UE. Or leave it to NW implementation, when the SIB31 is out of date, the NW update the CHO configuration with new SIB31.

And if the conditionally mandatory requirement is modified, it may have non-backwards issue. Because the UE following the version before or of 36.331 v17.2.0 will assume SIB31 always configured in CHO configuration.

So we think we should keep current version unchanged for the SIB31 in CHO configuration.

Proposal 1: In case of CHO, SystemInformationBlockType31 is always provided by the NW when handover to NTN cell.

Conclusion

In this contribution, we make the following proposal:

Proposal 1: In case of CHO, SystemInformationBlockType31 is always provided by the NW when handover to NTN cell.