3GPP TSG-RAN WG2 Meeting#119bis-e		       					   	R2-2210749

Electronic, 10th – 19th October 2022

Source: 			ZTE corporation, Sanechips

Title: 	Discussion on SA2 LS on slice list and priority information for cell reselection and Random Access

Agenda item:		6.8

Document for: 	Discussion and Decision

- Introduction

An LS [1] with attached CR has been sent from SA2 to share some further progress on the slice list and priority information for cell reselection and Random Access.

In this paper, we analyze the impact on RAN2 specs based on the latest SA2 and CT1 progress.

- Discussion

In the attached CR [2], the following modification has been made:

--------------------------------------------------------------From CR S2-2207692---------------------------------------------------

The UE NAS provides to the UE AS the NSAG Information  as received from the AMF and  the S-NSSAIs in the Allowed NSSAI as input to cell reselection, except when the UE intends to register with a new set of S-NSSAIs with a Requested NSSAI different from the current Allowed NSSAI, in which case the UE NAS provides to the UE AS layer the NSAG Information as received from the AMF and the S-NSSAIs in the Requested NSSAI, and this may trigger a cell reselection, before sending the Registration Request including the new Requested NSSAI.

--------------------------------------------------------------From CR S2-2207692---------------------------------------------------

With the above modification, we can find that the information provided from NAS layer to AS layer for cell reselection has been changed from “the NSAG information for the S-NSSAIs in the Allowed NSSAI or the S-NSSAIs in the Requested NSSAI” into “the NSAG information and the S-NSSAIs in the Allowed NSSAI or the S-NSSAIs in the Requested NSSAI”, which means that the NAS layer will not sort out the NSAG associated with the allowed S-NSSAIs or the requested S-NSSAIs. Instead, the AS layer is expected to do such filtering based on the NSAG information and the allowed/requested S-NSSAIs provided.

Based on the following highlighted NOTE in CT1 spec [3], we understand CT1 shares the same understanding and would expect AS layer to filter the NSAG based on the allowed/requested S-NSSAIs in cell reselection.

-------------------------------------------------------------------From 24.501-i01---------------------------------------------------------

#### 4.6.2.6	Provision of NSAG information to lower layers

NSAG information provided by the network and stored in the UE includes a list of NSAGs each of which contains:

a)	an NSAG ID;

b)	a list of S-NSSAI(s), which are associated with the NSAG and shall be part of the configured NSSAI;

c)	optionally a list of TAIs in which the NSAG is valid. If it is not provided by the network, the NSAG is valid in the PLMN which has sent the NSAG information; and

d)	a priority value that is associated with the NSAG.

The UE NAS layer shall provide the lower layers with the most recent NSAG information stored in the UE (see subclause 4.6.2.2) to lower layers.

NOTE:	Along with the NSAG information, the UE provides to the lower layers with allowed NSSAI and requested NSSAI for the purpose of NSAG-aware cell reselection.

-------------------------------------------------------------------From 24.501-i01---------------------------------------------------------

To align with the understanding in SA2 and CT1, 38.304 needs to be updated as the allowed S-NSSAIs or the requested S-NSSAIs are not considered in cell reselection in the latest version.

Proposal 1: Based on the latest progress in SA2 and CT1, RAN2 understand AS layer is expected to filer the NSAG information based on the allowed/requested S-NSSAIs and only the NSAG information associated with the allowed/requested S-NSSAIs will be considered in slice based cell reselection.

Proposal 2: Agree the 38.304 CR [4] to align with SA2 and CT1 understanding on the NAS-AS interaction in slice based cell reselection.

Proposal 3: Approve the draft reply LS [5] to confirm RAN2 understanding with SA2 and CT1 and inform the update of 38.304.

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Proposal 1: Based on the latest progress in SA2 and CT1, RAN2 understand AS layer is expected to filer the NSAG information based on the allowed/requested S-NSSAIs and only the NSAG information associated with the allowed/requested S-NSSAIs will be considered in slice based cell reselection.

Proposal 2: Agree the 38.304 CR [4] to align with SA2 and CT1 understanding on the NAS-AS interaction in slice based cell reselection.

Proposal 3: Approve the draft reply LS [5] to confirm RAN2 understanding with SA2 and CT1 and inform the update of 38.304.

- References

[1] R2-2209358 LS Out on LS on slice list and priority information for cell reselection and Random Access (S2-2207698; contact: ZTE)

[2] S2-2207692_ Closing open issues on RAN slicing

[3] TS 24.501-i01

[4] R2-2210750 _REL-17_38.304_CR 0295 _Correction on handling of the NSAG information in cell reselection

[5] R2-2210751_[Draft] Reply LS on slice list and priority information for cell reselection and Random Access