3GPP TSG-RAN WG2 Meeting #119bis-e	R2-22xxxxx

Online, October 2022

Source: 	Session Chair (Apple)

Title:	Report from session on NCR

# Status of At-Meeting Email Discussions

This subclause is not an Agenda Item. It contains a running summary of the email discussions assigned to take place during the meeting weeks.  This section will be moved to an appendix in the final version of the report.

[AT119-e][700][NCR] Organisational Sasha – NCR (Apple)

Scope: Organisational discussions and announcements, as needed throughout the meeting weeks

Intended outcome: Well-informed participants

Deadline:  Wednesday 2022-10-19 1000 UTC

## 8.1	NR network-controlled repeaters

(NR_NetConRepeater; leading WG: RAN1; REL-18; WID: RP-222673)

Time budget: 0.5 TU

Tdoc Limitation: 1 tdocs

### 8.1.1	Organizational

Including LSs and any rapporteur inputs.

R2-2209328	LS on NCR Solutions (R3-225253; contact: ZTE)	RAN3	LS in	Rel-18	FS_NR_netcon_repeater	To:SA3, SA5	Cc:RAN2, SA2

R2-2209329	Progress on NCR identification and authorization (R3-225254; contact: ZTE)	RAN3	LS in	Rel-18	FS_NR_netcon_repeater	To:RAN1	Cc:RAN2

R2-2210294	Work plan for Network-controlled repeaters	ZTE Corporation, Sanechips	Work Plan	Rel-18	FS_NR_netcon_repeater

### 8.1.2	Signalling for side control information

Signalling and procedures for for side control information, based on RAN1 agreements. Additionally, any other RAN2 reletated aspects, if needed.

R2-2210920  Report of [AT119bis-e][NCR] NCR open issues (ZTE)

Proposal 1	RAN2 confirms to use RRC signalling to configure NCR-MT to receive side control information. How the side control information itself is transmitted (e.g. via RRC or DCI or MAC CE) is up to RAN1.

Proposal 2	NCR-MT supports RRC_CONNECTED and RRC_IDLE states, FFS on RRC_INACTIVE state (e.g. optional support or not support).

Proposal 3	NCR-MT supports SRB0/1/2 and DRB. FFS whether DRB is optional feature for NCR-MT.

Proposal 4	The association between RRC states of NCR-MT and NCR-Fwd ON/OFF is pending RAN1 progress.

Proposal 5	NCR-MT should ignore cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse，cellReservedForOtherUse, intraFreqReselection indications and UAC configuration if broadcast in system information.

Proposal 6	From RAN2 perspective, the following RRM functions are applicable to NCR-MT:

- Cell (re)selection;

- RLM;

- BFD, BFR;

- FFS the applicable features are mandatory or optional

Proposal 7	To discuss whether the following RRM functions are applicable to NCR-MT:

- RRM measurements in RRC_IDLE and RRC_INACTIVE (if supported);

- RRM measurements in RRC_CONNECTED;

- handover

R2-2210155	Discussion on signalling for side control information	CMCC	discussion	Rel-18	NR_netcon_repeater

R2-2210279	Signalling for NCR side control information	MediaTek Inc.	discussion	Rel-18

R2-2210334	Discussion on RAN2 topics for NCR	Ericsson	discussion	Rel-18	FS_NR_netcon_repeater

R2-2210295	Consideration on NCR signalling and RRM functions	ZTE Corporation, Sanechips	discussion	Rel-18	FS_NR_netcon_repeater

R2-2210454	Discussion on NCR capability framework	Philips International B.V.	discussion	Rel-18	FS_NR_netcon_repeater

R2-2209367	Signaling for side control information and RRM functions	CATT	discussion	Rel-18	FS_NR_netcon_repeater

R2-2209630	Discussion on C-plane aspects for NCR-MT	Fujitsu	discussion	Rel-18	NR_netcon_repeater

R2-2209639	Signalling of side control information for NCR	Intel Corporation	discussion	Rel-18	NR_netcon_repeater

R2-2209667	Discussion on NCR configuration signaling and RRM functions	Huawei, HiSilicon	discussion	Rel-18	NR_netcon_repeater

R2-2209680	NCR side control signalling and other RRC and RRM aspects	Nokia, Nokia Shanghai Bell	discussion	NR_netcon_repeater

R2-2209697	Signalling for side control information to support NR network-controlled repeaters	AT&T, FirstNet	discussion

R2-2209705	Configuration of signaling for side control information	Qualcomm Inc.	discussion	Rel-18	FS_NR_netcon_repeater

R2-2209773	Discussion on Signaling for Side Control Information	Apple	discussion	Rel-18	DUMMY

R2-2209933	Discussion on Signaling and procedures for side control information	Lenovo	discussion	Rel-18

R2-2210135	Control plane signaling and procedures of network-controlled repeater	NEC Corporation	discussion

R2-2210155	Discussion on signalling for side control information	CMCC	discussion	Rel-18	NR_netcon_repeater

R2-2210200	Network-controlled repeaters - key issues	Samsung R&D Institute UK	discussion

R2-2210207	Considerations on NCR fwd link config	Sony	discussion	Rel-18	NR_netcon_repeater

R2-2210386	Discussion on NCR Related Procedures	vivo	discussion	Rel-18

R2-2210431	Consideration of network-controlled repeaters 	Kyocera 	discussion	Rel-18	R2-2208293

R2-2210563	Discussion on RAN2 issues for NCR	LG Electronics	discussion	Rel-18	NR_netcon_repeater

R2-2210572	On RAN2 impact of Network-Controlled Repeaters	China Telecom Corporation Ltd.	discussion

### 8.1.3	Repeater management

Including Identification and authorization of network-controlled repeaters, taking into accout feedback from SA3.

Note: we will wait for SA3 reply, so no contributions are expected to be treated in RAN2#119-bis.

R2-2209706	Management of Network-Controlled Repeater	Qualcomm Inc.	discussion	Rel-18	FS_NR_netcon_repeater