3GPP TSG-RAN WG2 #119bis-e	Tdoc R2-22xxxxx

Electronic meeting, 2022-10-10 - 2022-10-19

Agenda Item:	6.24.3 Other

Source:	Ericsson

Title:	Report from [AT119bis-e][012][NR17] MINT (Ericsson)

Document for:	Discussion, Decision

1	Introduction

This document provides the report from the following discussion:

[AT119bis-e][012][NR17] MINT (Ericsson)

Scope: Treat R2-2209305, R2-2210657, R2-2210658. Determine agreeable parts, Based on agreeable parts, progress CRs

Intended outcome: Report, Agreed-in-principle CRs.

Deadline: In time for CB W2 Mon (if CB is needed)

These papers are treated:

R2-2209305	Reply LS on system information extensions for minimization of service interruption (MINT) (C1-225386; contact: Ericsson)	CT1	LS in	Rel-17	MINT	To:RAN2	Cc:SA2

R2-2210657	Correction to disasterRoamingFromAnyPLMN	Ericsson, Lenovo	CR	Rel-17	36.331	17.2.0	4878	-	F	TEI17

R2-2210658	Correction to disasterRoamingFromAnyPLMN	Ericsson, Lenovo	CR	Rel-17	38.331	17.2.0	3557	-	F	TEI17

Deadline round 1: Please provide input by Thursday 06:00 UTC.

2	Contact information

Respondents are requested to provide their contact information in this table:

| Company | Name | E-mail |
| --- | --- | --- |
| Ericsson | Mattias Bergström | mattias.a.bergstrom@ericsson.com |
| Huawei, HiSilicon | Yang Zhao | zhaoyang@huawei.com |
| OPPO | Qianxi Lu | qianxi.lu@oppo.com |

3	Discussion

In the LS in R2-2209305, CT1 writes:

1. Overall Description: CT1 would like to thank RAN2 for LS R2-2206480. CT1 would like to inform RAN2 that CT1 agreed attached CR updating definition of the "disaster related indication" for the RAN sharing scenario. In order to avoid inconsistency between TS 23.122 and TS 38.331, CT1 would like to ask RAN2 to consider replacing description of disasterRoamingFromAnyPLMN in TS 38.331 with a reference to the definition of the "disaster related indication" in TS 23.122. 2. Actions: To RAN2 group. ACTION: CT1 would like to ask RAN2 to consider replacing description of disasterRoamingFromAnyPLMN in TS 38.331 with a reference to the definition of the "disaster related indication" in TS 23.122.

In response to this LS, the following two CRs are provided

R2-2210657	Correction to disasterRoamingFromAnyPLMN	Ericsson, Lenovo	CR	Rel-17	36.331	17.2.0	4878	-	F	TEI17

R2-2210658	Correction to disasterRoamingFromAnyPLMN	Ericsson, Lenovo	CR	Rel-17	38.331	17.2.0	3557	-	F	TEI17

The rapporteur notes:

- as indicated on the cover-page: the CRs needs to be lifted to the latest version of the spec

- some of the added text has red font which should be black

Assuming the proponent addresses this when submitting the final versions to RAN2#120, can the CRs be in-principle-agreed? If your reply is “No” please explain why or what changes you find necessary for the CRs to be agreeable.

| Company | Yes / No | Comments |
| --- | --- | --- |
| Ericsson | Yes | We want to highlight that the name of the new field is “disasterRelatedIndication” as also used in CT1 specifications. It is perhaps not a very descriptive name, but this is the term CT1 have used and there is benefit in using the same name also in RAN2 specifications to make sure the reader sees the connection. Changing the field name is backwards compatible as the encoded bits do not change. |
| Huawei, HiSilicon | Yes |  |
| OPPO | Yes |  |

Any other comments?

| Company | Comments |
| --- | --- |

4	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	A Proposal with automatic numbering. Assign this type by pressing Alt-P. A list of all Proposals can be found in the Conclusion section.