3GPP TSG RAN WG2 #119bis-e	draft R2-2210850

Online, 10 - 19 Oct, 2022

Source: 	Huawei, HiSilicon

Title: 	Report of [Offline-109][NR NTN] cell reselection requirements (Huawei)

Agenda Item:	6.10.1.1

Document for:	Discussion and decision

Introduction

This document is the report of the following offline discussion, which is triggered by RAN4 LS [1].

[AT119bis-e][109][NR NTN] cell reselection requirements (Huawei)

Initial scope: Discuss the proposals for enhanced cell reselection requirements for NTN

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

- List of proposals that should not be pursued (if any)

Deadline (for companies' feedback):  Thursday 2022-10-13 14:00 UTC

Deadline (for rapporteur's summary in R2-2210850):  Thursday 2022-10-13 16:00 UTC

Proposals marked "for agreement" in R2-2210850 not challenged until Friday 2022-10-14 10:00 UTC will be declared as agreed via email by the session chair (for the rest the discussion might continue online).

During online discussion on 10th October, the following was agreed:

Agreements:

- Introduce one indication for cell reselection requirement enhancement for LEO. FFS if in SIB1 or SIB19

In this offline discussion, we will discuss:

1) Whether the indication for LEO is in SIB1 or SIB 19;

2) Whether the relaxed monitoring of GEO can reuse the existing configuration;

And try to attempt some stage-3 details and a draft reply LS.

- Contact Information

To make it easier to find the contact delegate for potential follow-up questions, delegates are encouraged to provide their contact information in the following table:

| Company | Name | Email |
| --- | --- | --- |
| Huawei, HiSilicon | Lili Zheng | zhenglili4@huawei.com |
| vivo | Xiao XIAO | xiao.xiao@vivo.com |

Discussion

It was raised online by OPPO that the indication should be in SIB19 rather than SIB1, so that all NTN-related information can be grouped into one SIB.

Q1: Please share your views on whether the indication for enhanced requirements for LEO should be in SIB1 or SIB19.

| Company | SIB1/SIB19 | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | SIB1 | Firstly we think 1-bit will not take much space and can be included in SIB1. Secondly, there are other NTN related configuration located in SIB2/4 (e.g. multiple SMTCs). There is no need to restrict all NTN configuration to SIB19. If we follow this principle, the relaxed monitoring configuration will be located in SIB19 as well, while R16 relaxed monitoring configuration for UE power saving and R17 relaxed monitoring configuration for RedCap are in SIB2. |
| vivo |  | We can follow the majority. |

With respect to GSO relaxed monitoring, it was raised in [2][3] to reuse the exiting relaxedMeasurement-r16 field, but several companies expressed online that a separate bit is preferred. One of the argument is that it is unclear whether the requirements are exactly the same with the R16 requirements.

In the current 38.133 v17.7.0, the intention is to simply refer to the R16 requirements:

- 4.2C.2.7 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion The requriements in this clause 4.2.2.7 apply provided that UE is GEO.

However, there is a mistake that the clause number should be 4.2.2.9 (Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion) instead of 4.2.2.7 (General requirements). This is an obvious mistake, and likely to be fixed by RAN4 shortly. The following is a CR proposed by CATT in RAN4 (R4-2215431):

#### 4.2C.2.7 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion The requriements in clause 4.2.2.9 apply provided that UE is GEO. #### 4.2C.2.8 Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion The requriements in clause 4.2.2.10 apply provided that UE is GEO.

Based on the above, the requirements for GSO relaxed monitoring are the same with R16 requirements.

Companies may notice that Rel-17 also introduced relaxed monitoring specific to RedCap UEs in SIB2 (cited from 38.331 v17.2.0):

relaxedMeasurement-r16              SEQUENCE {

lowMobilityEvaluation-r16           SEQUENCE {

s-SearchDeltaP-r16                  ENUMERATED {

dB3, dB6, dB9, dB12, dB15,

spare3, spare2, spare1},

t-SearchDeltaP-r16                  ENUMERATED {

s5, s10, s20, s30, s60, s120, s180,

s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                   OPTIONAL,       -- Need R

cellEdgeEvaluation-r16              SEQUENCE {

s-SearchThresholdP-r16              ReselectionThreshold,

s-SearchThresholdQ-r16              ReselectionThresholdQ                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition-r16     ENUMERATED {true}                               OPTIONAL,       -- Need R

highPriorityMeasRelax-r16           ENUMERATED {true}                               OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]],

[[

cellEquivalentSize-r17                  INTEGER(2..16)                                  OPTIONAL,       -- Cond HSDN

relaxedMeasurement-r17                  SEQUENCE {

stationaryMobilityEvaluation-r17        SEQUENCE {

s-SearchDeltaP-Stationary-r17           ENUMERATED {dB2, dB3, dB6, dB9, dB12, dB15, spare2, spare1},

t-SearchDeltaP-Stationary-r17           ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

},

cellEdgeEvaluationWhileStationary-r17   SEQUENCE {

s-SearchThresholdP2-r17                 ReselectionThreshold,

s-SearchThresholdQ2-r17                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition2-r17        ENUMERATED {true}                           OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]]

But the case for RedCap is different with NTN:

For RedCap, the criteria for relaxed measurements are different with R16: R16 uses 1) low mobility and 2) not-at-cell-edge, R17 RedCap uses 1) stationery and 2) not-at-cell-edge while stationery. RAN4 also defined separate requirements in 38.133 (clause 4.2B.2.9 and 4.2B.2.10). Besides, it is allowed that both R16 relaxed monitoring and R17 relaxed monitoring are configured simultaneously to RedCap UEs, and the UE behaviour for different combinations of configuration is specified in 38.133.

For NTN, as previously mentioned, RAN4 decided to reuse the current requirements. From RAN2 perspective, we don’t see the need to configure R16 relaxed monitoring and configure a new R17 NTN relaxed monitoring at the same time, so it is straightforward to reuse the R16 configuration.

Q2: Which of the following is preferred:

- Option 1: Reuse the existing relaxedMeasurement-r16 field;

- Option 2: Add a separate flag, but reuse the configuration in relaxedMeasurement-r16;

- Option 3: Add separate configuration, which is similar to relaxedMeasurement-r16.

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option 1 | The field description can be modified, as in R2-2210409: relaxedMeasurement Configuration to allow relaxation of RRM measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9). In NTN, this field is only present in GSO. |
| vivo |  | We can follow the majority. |

It was proposed in [6] to add the following description to 38.304 Section 5.2.4.2	Measurement rules for cell re-selection:

If the enhancedNTN-Reselection is present in SIB2 and the UE supports NTN LEO, the UE shall perform enhanced cell reselection measurements according to TS 38.133 [8]. If the relaxedNTN-Reselection is present in SIB2 and the UE supports NTN GEO, the UE shall perform relaxed cell reselection measurements according to TS 38.133 [8].

Even though the exact description depends on the 38.331 signalling details, the moderator would like to collect companies’ initial opinions on whether 38.304 changes are required.

Q3: Do you think changes to 38.304 are needed?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | As long as the field description in 38.331 is made clear and refers to the correct RAN4 spec (i.e., 38.331), we don’t see the necessity to refer to RAN4 spec again in 38.304. |
| vivo | No | According to current Spec, TS 38.304 specifies that UE performs measurement according to TS 38.133 and TS 38.331 specifies which requirements to be applied by UE under what circumstances. The measurement behavior of UE is clear and no additional description in TS 38.304 is needed. |

The draft LS and 38.331 details will wait for the conclusion of the previous questions.

Conclusion

To be completed

Reference

- R2-2209337	LS to RAN2 on Network indication for applying enhanced cell reselection requirements (R4-2214472; contact: Huawei), RAN4

- R2-2210408, Discussion on enhanced cell reselection requirements for NTN, Huawei, HiSilicon

- R2-2210409, CR on enhanced cell reselection requirements for NTN, Huawei, HiSilicon

- R2-2210044, On LS Network indication for applying enhanced cell reselection requirements, Ericsson

- R2-2210347, NR RRC CR: Introduction of enhanced and relaxed cell reselection for NTN, Nokia, Nokia Shanghai Bell

- R2-2210348, NR IDLE-mode CR: Introduction of enhanced and relaxed cell reselection for NTN, Nokia, Nokia Shanghai Bell