3GPP TSG RAN2 Meeting #120	 R2-2211175

# 14th – 18th November 2022, Toulouse, France

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR |  | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous Corrections for SDT Operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Issue 1: The procedure in section 5.27.1 uses phrase ‘the Serving Cell for SDT’. The meaning of this is not clear. It seems to suggest that there are multiple serving cells and one of them is for SDT. However in RRC_INACTIVE there is only one serving cell i.e. cell on which UE is currently camped. Suggest to replace the phase with either ‘current Serving Cell’ or ‘Serving Cell’. Issue 2: The procedure in section 5.27.1 uses the phrase ‘TA of the configured grant Type 1 resource’. In our understanding TA is maintained for a cell in RRC_INACTIVE for CG-SDT procedure. It is not per CG resource. So suggest to replace the phrase with ‘TA for CG-SDT’ Issue 3: The procedure in section 5.27.1 specifies that “If RA-SDT is selected above and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI until the RA-SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI and CS-RNTI until the CG-SDT procedure is terminated.” It’s not clear which RNTI is used as C-RNTI during the above operation. In our understanding the C-RNTI stored in Inactive UE AS context should be used for CG-SDT and C-RNTI received in random access response should be used for RA-SDT. This should be clarified. Issue 4: The procedure in section 5.27.1 checks for validtity of TA at the first available CG occasion. However, in our understanding it should be the first available CG occasion corresponding to SSB associated with CG-SDT resources with SS-RSRP above cg-SDT-RSRP-ThresholdSSB as this CG occasion is used for initial UL transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Phrase ‘the Serving Cell for SDT’ is changed to ‘Serving Cell’; Phrase ‘TA of the configured grant Type 1 resource’ is changed to ‘TA for CG-SDT’; Clarified the C-RNTI’s used during the SDT procedure. Clarified that UE checks for validtity of TA in the first available CG occasion corresponding to SSB associated with CG-SDT resources with SS-RSRP above cg-SDT-RSRP-ThresholdSSB Impacted functionality: SDT Inter-operability: - If the network is implemented according to the CR and the UE is not, there are no interoperability issues. - If the UE is implemented according to the CR and the network is not, there are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Issue 1, 2: Ambigous and misleading text for Serving Cell related to SDT and TA for CG-SDT. Issue 3: C-RNTI used during SDT procedure (RA-SDT, CG-SDT) is unclear. Issue 4: CG-SDT is unnecessarily selected for the case TA is invalid at the first available CG occasion selected for first UL transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <Start of Change>

### 5.27.1	General

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer. The SDT procedure can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT).

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT.

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold; and

NOTE:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than sdt-RSRP-Threshold; or

1>	if sdt-RSRP-Threshold is not configured:

2>	if Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid (according to clause 5.27.2) in the first available CG occasion corresponding to SSB in cg-SDT-Configuration with SS-RSRP above cg-SDT-RSRP-ThresholdSSB; and

2>	if, for each RB having data available for transmission, configuredGrantType1Allowed, if configured, is configured with value true for the corresponding logical channel; and

2>	if at least one SSB configured in cg-SDT-Configuration with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if a set of Random Access resources for performing RA-SDT are selected according to clause 5.1.1b on the selected UL carrier:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

If RA-SDT is selected above and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI (received in random access response) until the RA-SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI (as stored in UE Inactive AS context as specified in TS 38.331 [5] )and CS-RNTI (configured by cg-SDT-CS-RNTI) until the CG-SDT procedure is terminated.

#### <End of Change>