3GPP TSG-RAN WG2 Meeting #120                                    R2-2211246

Toulouse, France, November 14-18, 2022

Agenda item:		6.3

Source:				Samsung

Title:					Views on NAS busy indication in RRC_INACTIVE

Document for:		Discussion & Decision

1	Introduction

This contribution provides information on how SA2 and CT1 specification are updated for NAS busy indication in RRC_INACTIVE and proposes to conclude this issue.

2	Discussion

In RAN2#118-e meeting, the following agreements on NAS busy indication in RRC_INACTIVE were made:

[234] 1: From RAN2 perspective, confirm that INACTIVE UE may not be able to send a Service Request message to the network including the Reject Paging indication as a response to the RAN paging due to UE implementation constraints.

[234] 2: From RAN2 perspective, agree to add the following NOTEs in TS 38.331 and TS 36.331 respectively as follows:

- NOTE in TS 38.331: A MUSIM UE may not initiate the RRC connection resumption procedure, e.g. when it decides not to respond to the Paging message due to UE implementation constraints as specified in TS 24.501 [23].

- NOTE in TS 36.331: A MUSIM UE may not initiate the RRC connection resumption procedure, e.g. when it decides not to respond to the Paging message due to UE implementation constraints as specified in TS 24.501 [95].

[234] 4: Send LS to CT1/SA2 to inform them of our decision and to ask them to update their specifications accordingly if needed.

Accordingly, the reply SA2 LS [1] is received with the following update in TS 23.501 (see below):

### 5.38.4	Reject Paging Request

A Multi-USIM UE may set up a connection to respond to a page with a Reject Paging Indication to the network indicating that the UE does not accept the paging and requests to return to CM-IDLE state after sending this response, if both UE and network indicate the Reject Paging Request feature is supported to each other.

Upon being paged by the network, the Multi-USIM UE in CM-IDLE state attempts to send a Service Request message to the paging network including the Reject Paging Indication as the response to the paging, unless it is unable to do so, e.g. due to UE implementation constraints. In addition to the Reject Paging Indication, the UE may include Paging Restriction Information as specified in clause 5.38.5 in the Service Request message, if supported by UE and network.

NOTE:	A Multi-USIM UE in RRC_INACTIVE state can decide to not initiate the RRC connection resumption procedure, e.g. when it decides not to respond to the paging message due to UE implementation constraints as specified in TS 24.501 [47] and TS 38.331 [28].

Observation 1: SA2 updated their specification to align with RAN2 on NAS busy indication in RRC_INACTIVE i.e. see TS 23.501, clause 5.38.4.

Though RAN2 didn't receive the reply CT1 LS, the following update in TS 24.501 [2] has been implemented (see below):

5.3.1.4	5GMM-CONNECTED mode with RRC inactive indication

<Only relevant procedure texts are copied below>

If the UE in 5GMM-CONNECTED mode with RRC inactive indication receives an indication from the lower layers about RAN paging and the MUSIM UE decides not to initiate the service request procedure with service type set to “mobile terminated services” to respond to the RAN paging, the UE may initiate the service request procedure and set request type to "NAS signalling connection release" in the UE request type IE and service type to "signalling" in the SERVICE REQUEST message to reject the RAN paging as specified in subclause 5.6.1.2 for case o of subclause 5.6.1.1. The UE may include its paging restriction preferences in the Paging restriction IE in the SERVICE REQUEST message as specified in subclause 5.6.1.2 for case o of subclause 5.6.1.1.

NOTE 5:	The interworking between the NAS layer and the AS layer triggered by RAN paging is up to UE implementation.

NOTE 6: As an implementation option, the MUSIM UE is allowed to not respond to RAN paging based on the information available in the paging message, e.g. voice service indication.

Observation 2: CT1 updated their specification to align with RAN2 on NAS busy indication in RRC_INACTIVE i.e. see TS 24.501, clause 5.3.1.4.

According to observations 1 and 2, we think that there is no further work in RAN2 as UE behavior on NAS busy indication in RRC_INACTIVE is already implemented in TS 36.331/TS 38.331 before sending a LS to SA2/CT1. Hence, we suggest the chair to note the reply SA2 LS [1]".

Proposal: Suggest to note R2-2211119 and confirm that no further action in RAN2 is required.

3	Conclusion

In section 2, the following observations are made:

Observation 1: SA2 updated their specification to align with RAN2 on NAS busy indication in RRC_INACTIVE i.e. see TS 23.501, clause 5.38.4.

Observation 2: CT1 updated their specification to align with RAN2 on NAS busy indication in RRC_INACTIVE i.e. see TS 24.501, clause 5.3.1.4.

Based on the above, RAN2 is requested to discuss and agree on the following proposal:

Proposal: Suggest to note R2-2211119 and confirm that no further action in RAN2 is required.

4	Reference

[1] R2-2211119	Reply LS on NAS busy indication in RRC_INACTIVE (S2-2207029; contact: Samsung)

[2] 3GPP TS 24.501 Non-Access-Stratum (NAS) protocol for 5G System (5GS) v17.8.0