3GPP TSG-RAN WG2 #120	R2-2211266

Toulouse, France, November 2022

Agenda Item:	6.21.1

Source:	OPPO

Title:	Discussion on per FR gap UE capability

Document for:	Discussion, Decision

Introduction

The current granularity of UE capability on per FR measurement gap i.e. independentGapConfig is a per UE capability. As explained in [1] the assumption that FR2 always has its independent hardware could be not valid anymore due to more and more hardware sharing. So it is possible that UE may not be able to support it in high order CA/DC combination, but could support it in low order CA/DC combination.

There are 3 alternatives on the table at RAN2#119bis meeting:

Alt1.1: The per FR gap is introduced per BC level additional to per UE level

Alt 1.3: To introduce a threshold i.e. number of carriers, under which per FR gap is feasible

Alt 2: To report per FR gap on top of current needForGap scheme

And here is the conclusion:

Exclude Alt 1.1 for now.

On the table: Alt 1.3, Alt 1.3 per BC, Alt 2 (add info, based on current config as today, FFS excl/incl DC)

RAN2 excluded alt1.1 due to its high signalling overhead. Meanwhile alt1.3 per BC is added i.e. the threshold is configured per BC instead per UE. Another alternative was discussed but not documented i.e. the combination of Alt1.3(or Alt1.3 per BC)+Alt2 :

Alt3: Network only request per FR gap when configured carriers is lower or equal to reported threshold

This paper discuss further detail of the pros and cons of the listed alternatives and propose to go for Alt3.

Discussion on alternatives

As indicated by the conclusion in [1] the main solutions on the table are:

One of the key aspects that impacts the UE’s per FR capability is how much hardware resource is consumed for current serving cells. It is related to the number of carriers for sure, but it is also relevant to other aspects e.g. channel bandwidth of the carriers, total configured aggregated bandwidth, configured MIMO layers etc. That’s why only take number of carriers as sole criteria to determine whether UE can support per FR gap is not sufficient for both Alt1.3 and Alt1.3 per BC.

On the other hand companies complain that Alt2 is over flexible i.e. whenever RRC configuration is modified, network may need check with UE once again. So the combo of Alt1.3 per BC and Alt2 could help to reduce signalling overhead. Here is description of Alt3:

Step 0: UE indicate that it support new per FR gap report scheme

Step 1: UE report a threshold per BC i.e. maximum number of carriers, under which per FR gap is feasible

Step 2: network may request UE to report support of per FR gap only when number of configured carriers is lower than threshold

Step 3: UE report whether it support per FR gap for requested bands or cells

Table 1 Alt3

Proposal 1: RAN2 agree to go for Alt3 i.e. UE report per FR gap on top of current needForGap scheme when configured number of carriers is lower than reported threshold for concerned band combination.

Open issues of Alt 3

The additional signaling is only necessary when UE set independentGapConfig to be False. So legacy gNB will interpret that UE can’t support per FR gap. Hence there is no backwards compatibility issue.

Observation 1: Alt3 is a backwards compatible solution

Current needForGap scheme only support stand alone architecture i.e. MRDC and NR-DC scenarios are not supported. The main concern of MRDC case is the potential additional coordination between MN and SN. For (NG)ENDC case it also means modification of LTE spec. Considering the motivation of finer granularity is mainly because of possible hardware sharing between carriers, it is more possible to occur within same RAT. For NRDC case, currently MN node decide on per UE or per FR gap configuration. In case of per FR gap configuration, SN’s gap configuration is also decided by MN. A finer level per FR gap report help MN understand better UE’s capability, but it doesn’t impact the coordination scheme itself i.e. current coordination scheme can be reused.

Since the number of configured carrier will influence on MN’s request behaviour, any change of configured carriers in SN should be known by MN instantly. Again this is already supported in current spec for SN to send CG-Config to MN.

Proposal 2: Apart from SA scenario, NRDC is also supported for per FR gap report

In Rel17 UE answer NeedForGapNCSG-ConfigNR-r17 in RRCReconfiguration/RRCResume message with NeedForGapNCSG-InfoNR-r17, where 3 values i.e. { gap, ncsg, nogap-noncsg } is reported per cell for intra-frequency measurement and per band for inter-frequency measurement. It only makes sense for UE to report per FR gap when it will report either {gap} or {ncsg} in current NeedForGapNCSG-ConfigNR-r17 since both imply either per UE gap and/or per FR gap could be configured. So naturally the new indication need be reported on top of existing scheme.

Proposal 3: UE could report per FR gap indication when UE report either value “gap” or “ncsg” in NeedForGapNCSG-ConfigNR-r17

If UE can support NRDC for per FR gap report already it will not be difficult for UE to do it for legacy gap report since legacy gap report covers both per UE and per FR gap report.

Proposal 4: NRDC is also supported for Rel17 needForGap scheme

In current Rel17 needForGap scheme, UE report gap support per target band for inter-frequency case and per target cell per intra-frequency case. Paper [2] believes for per FR gap report, such granularity is not necessary because it is only related to serving cells configuration. Take inter-frequency measurement as example, if UE report {gap} for band A and {nogap-noncsg} for band B, then what does UE report for per FR gap? If UE report per FR gap is needed for all bands including band A and band B, then network will configure per FR gap also when measuring frequency of band B, which is not necessary and contradict with reported legacy value for band B i.e. {nogap-noncsg}.The consequence is that UE can only report to support per FR gap when UE report {gap} for all concerned bands. In order to avoid such backwards issue, per FR gap should be report as the same granularity as current Rel17 i.e. per target band for inter-frequency and per target cell for intra-frequency case.

Proposal 5: per FR gap is reported the same granularity as current Rel17 i.e. per target band for inter-frequency and per target cell for intra-frequency case

Since per FR gap is reported based on current detail RRC configuration, it is feasible for UE to report whether it can support per-FR1 gap or per-FR2 gap or both. In case UE can only support either per-FR1 or per-FR2 gap and if UE can only report 1 bit i.e. either support or not support both per-FR1 gap and per-FR2 gap, then it can only report NOT to support per FR gap.

Proposal 6: the reported value are {perFR1, perFR2, both}

For this new feature an UE capability is needed. Since a threshold will be reported per band combination, no additional signaling is necessary. For one specific band combination, UE doesn’t report per FR gap for that band combination if no such threshold is reported from Rel17 on. In case a UE doesn’t support such feature at all, no threshold will be reported for any band combination.

Proposal 7: Apart for per BC threshold, no additional UE capability is reported for supporting per FR gap.

UE also answer NeedForGapNCSG-ConfigEUTRA-r17 with NeedForGapNCSG-InfoEUTRA-r1, where gapIndication-r17 covers {gap, ncsg, nogap-noncsg} per band. There is no such difference between the inter-frequency measurement and inter-RAT measurement w.r.t. per FR gap i.e. the same scheme can be applied.

Proposal 8: proposal 1~6 are also applicable for needForGap for inter-RAT measurement

The proposals are captured in the draft CR [3][4]

Conclusion

Proposal 1: RAN2 agree to go for Alt3 i.e. UE report per FR gap on top of current needForGap scheme when configured number of carriers is lower than reported threshold for concerned band combination.

Observation 1: Alt3 is a backwards compatible solution

Proposal 2: Apart from SA scenario, NRDC is also supported for per FR gap report

Proposal 3: UE could report per FR gap indication when UE report either value “gap” or “ncsg” in NeedForGapNCSG-ConfigNR-r17

Proposal 4: NRDC is also supported for Rel17 needForGap scheme

Proposal 5: per FR gap is reported the same granularity as current Rel17 i.e. per target band for inter-frequency and per target cell for intra-frequency case

Proposal 6: the reported value are {perFR1, perFR2, both}

Proposal 7: Apart for per BC threshold, no additional UE capability is reported for supporting per FR gap.

Proposal 8: proposal 1~6 are also applicable for needForGap for inter-RAT measurement

Reference

[1] RP-222276	UE Capability for per-FR Measurement Gap Qualcomm Incorporated

[2] R2-2210237	Discussion on per-FR gaps	Nokia, Nokia Shanghai Bell	discussion	Rel-17	TEI17

[3] R2-2211267 Draft 38.331 CR on per FR gap report

[4] R2-2211268 Draft 38.306 CR on UE capability for per FR gap report