3GPP TSG-RAN WG2 Meeting #120	                                               R2-2211312

Toulouse, France, Nov14th-18th, 2022

Source:	CATT

Title:	Enhancements for Neighbor Cell Measurements

Agenda Item:	8.6.3.1

Document for:	Discussion and Decision

Introduction

In RAN2#119e meeting, the following agreements about enhancement for neighbor measurement for IoT-NTN were made:

Agreements:

- IoT NTN can use the mechanism for neighbour cell measurements in connected mode (specified in Rel-17 for NB-IoT). FFS if any enhancements are needed (e.g. triggers) for both NB-IoT and eMTC.

- RAN2 to continue working on a new time-based trigger for triggering intra and inter frequency measurements in connected mode, e.g. the serving cell is going to stop covering the current area, for both earth-moving and earth-fixed cell (FFS on distance-based trigger)

In RAN2#119bis-e, even though an offline discussion was held, but no further agreement was made. In this contribution, we will further discuss the enhancements for neighbor measurement.

Discussion

Time-based trigger mechanism for NB-IoT

During the offline, the start/stop time of performing neighbor cell measurement was discussed. The following proposals were made during the offline discussion, without conclusion.

Proposal 3a: For quasi-earth fixed cell, NB-IoT UE may start intra/inter frequency measurement in connected mode before the t-Service, if present in SIB3-NB. Proposal3b: For earth moving cell, it’s FFS whether UE may start intra/inter frequency measurement in connected mode before the time when the serving cell is going to stop serving the area it is currently covering. And if yes, RAN2 further discuss how NB-IoT NTN UE determines this time and what assistance information needs to be provided to UE. Proposal4: RAN2 would not specify the condition of stopping UE measurement in connected mode.

In the following we continue discussion based on the above discussion.

Quasi-earth fixed cell

For quasi-earth fixed cell, the NW will broadcast the stop serving time of the serving cell t-Service in SIB3-NB for NB-IoT. Before the end of the serving time of the NTN cell, it is very likely the serving quality of the NTN cell for the UE is stable and reliable. So there is no need to trigger the neighbor cell measurement too early before the end of the serving time for the UE considering unnecessary power consumption. This is common understanding among companies.

Observation 1: The channel quality is stable during the serving time for a UE, so it is not necessary to trigger the neighbor cell measurement too early before the end of the serving time for the UE.

Besides, the serving time of the neighbor cell is also needed to be taken into consideration. Discontinues coverage prediction is supported in R17 IoT NTN, so the UE could be aware of when the incoming satellite will cover the area where the UE locates. Before the satellite covers the area, the UE couldn’t receive any signaling from the satellite. So it is not needed to trigger the measurement on the neighbor cell too early before the cell covers the area of the UE located which will lead in unnecessary power consumption too, i.e. the neighbor cell measurement should be triggered after the neighbor cell covers the area where UE located.

Observation 2: It is not necessary to trigger the neighbor cell measurement before the neighbor cell covers the area where the UE locates.

Based on the above observation, we propose to take the serving time information of the serving cell and neighbor cell into consideration to trigger the neighbor cell measurement. In order to avoid extra power consumption, too early neighbor cell measurement should be avoided. And in order to trigger handover or fast RLF recovery, the neighbor cell measurement shouldn’t be triggered too late. So we propose that the connected UE should trigger the neighbor cell measurement before the end of the serving time of serving cell and after the starting serving time of the neighbor cell which is late arrival.

Proposal 1: For quasi-earth fixed cell, at least for NB-IoT UE, the connected UE should trigger the neighbor cell measurement:

- before the t-Service of serving cell, if configured in SIB3-NB, or

- after the start serving time of the neighbor cell, if configured in SIB32-NB

If both configured, he late time is selected.

Earth-moving cell

For the earth-moving NTN cell, even though the stop serving time is different for different UE based on UE location, it is also predictable for each UE. The start serving time is also predictable for neighbor cells. In theory, the time-based trigger is also feasible for earth moving cell. The connected UE could trigger the neighbor cell measurement before the calculated end serving time of serving cell or after the calculated start serving time of the neighbor cell for the UE.

Observation 3: For earth-moving NTN cell, the stop serving time of the serving cell and the start serving time of the neighbor cell is predictable for each UE. Hence, time-based trigger for neighbor measurement is feasible.

Proposal 2: For earth-moving cell, at least for NB-IoT UE, the connected UE should trigger the neighbor cell measurement:

- before the end serving time of serving cell calculated by the UE, if configured, or

- after the start serving time of the neighbor cell calculated by the UE, if configured

If both configured, the late one is selected. .

But according to current information broadcasted in SIB31-NB, the UE couldn’t calculate the stop serving time of the earth-moving cell. In order to support the time-based trigger mechanism for neighbor measurement, some enhancement needs to be studied. To make the stop serving time of the serving cell is calculated for each UE. As for the neighbor cell, RAN2 should evaluate whether the start serving time of the neighbor is calculated for each UE.

Proposal 3: RAN2 to discuss enhancement for SIB to support time-based trigger for neighbor measurement for earth-moving NTN cell, including:

- calculating the stop serving time of the serving cell;

- Evaluating whether it is feasible to calculate the start serving time for neighbor cell according to current SIB32-NB

Distance-based trigger mechanism for NB-IoT

New time-based trigger for triggering intra and inter frequency measurements in connected model was agreed to be supported. It is obvious that, for the earth fixed cell, the RLF for a UE is mainly due to the movement of the satellite. In this case, the time-based mechanism proposed above can work well. In addition, if the RLF occurs due to UE moving across the edge of the cell, we think the RSRP based mechanism can also work well, but this case is minority we think. So for earth-fixed scenario, the time-based and RSRP based trigger mechanism are enough, it is not necessary to introduce the distance-based trigger mechanism.

Observation 4: Time-based and RSRP-based trigger for neighbor cell measurement can work well for earth-fixed NTN cell.

For earth-moving cell, the RSRP based mechanism can work for the case that RLF occurs due to UE movement as mentioned above. For the case RLF led by satellite movement, considering the UE can predict the serving time of the incoming satellite based on the SI, and the end serving time of the serving for the UE is ascertainable for the UE. So the time-based mechanism proposed above can work too in theory.

Observation 5: Time-based and RSRP-based trigger for neighbor cell measurement can work for earth-moving NTN cell.

Based on the above discussion, the distance-based trigger mechanism is not necessary for connected UE. Besides, distance-based mechanism needs the UE to perform GNSS measurement to evaluate whether the distance-based mechanism is fulfilled, which will increase the UE power consumption simultaneous GNSS and NTN NB-IoT operation is not supported. So we propose:

Proposal 4: For NB-IoT UE, distance-based trigger for triggering intra and inter frequency measurement in connected mode is not supported in IoT-NTN.

Applicability for eMTC

Based on the offline discussion, it seems the enhancement for neighbor cell measurement for NB-IoT is supported, but there is concern for eMTC UE. The main concern is that for eMTC, the measurement is supported for HO, and the enhancement for neighbor measurement for NB-IoT aims to enhance for RLF recovery. It may too late for eMTC to trigger the neighbor measurement for HO if the same enhanced mechanism for neighbor measurement used for NB-IoT applied to eMTC.

Considering the time-based trigger method discussed above, it takes the serving time of the serving cell and neighbor cell into consideration. It makes the UE perform neighbor cell measurement effectively, i.e. UE doesn’t need to perform measurement on neighbor cell before the neighbor cell coming, and not need to perform the measurement too early before the stop serving time of the serving cell. Anyway, currently the UE perform neighbor cell measurement is before the stop time of the serving time, and even the UE perform the neighbor cell measurement before the start serving time of the neighbor cell, the UE couldn’t receive any signaling from this neighbor cell which is in vain. Furthermore, even though the UE trigger the neighbor measurement according the time-based mechanism, the actual time to trigger the neighbor cell measurement depends on UE, the UE could trigger the measurement early if necessary. So at least for the time-based trigger mechanism can be used for eMTC UE, which can at least help the UE for power saving, avoiding perform neighbor cell measurement in vain.

Proposal 5: At least the time-based trigger mechanism for triggering neighbor cell in connected adopted for NB-IoT can be used for eMTC in IoT-NTN.

Conclusion

In this contribution, we give some discussion on neighbour cell measurement enhancements for IoT-NTN, the following proposal and observations are given:

Observation 1: The channel quality is stable during the serving time for a UE, so it is not necessary to trigger the neighbor cell measurement too early before the end of the serving time for the UE.

Observation 2: It is not necessary to trigger the neighbor cell measurement before the neighbor cell covers the area where the UE locates.

Observation 3: For earth-moving NTN cell, the stop serving time of the serving cell and the start serving time of the neighbor cell is predictable for each UE. Hence, time-based trigger for neighbor measurement is feasible.

Observation 4: Time-based and RSRP-based trigger for neighbor cell measurement can work well for earth-fixed NTN cell.

Observation 5: Time-based and RSRP-based trigger for neighbor cell measurement can work for earth-moving NTN cell.

Proposal 1: For quasi-earth fixed cell, at least for NB-IoT UE, the connected UE should trigger the neighbor cell measurement:

- before the t-Service of serving cell, if configured in SIB3-NB, or

- after the start serving time of the neighbor cell, if configured in SIB32-NB

If both configured, he late time is selected.

Proposal 2: For earth-moving cell, at least for NB-IoT UE, the connected UE should trigger the neighbor cell measurement:

- before the end serving time of serving cell calculated by the UE, if configured, or

- after the start serving time of the neighbor cell calculated by the UE, if configured

If both configured, the late one is selected. .

Proposal 3: RAN2 to discuss enhancement for SIB to support time-based trigger for neighbor measurement for earth-moving NTN cell, including:

- calculating the stop serving time of the serving cell;

- Evaluating whether it is feasible to calculate the start serving time for neighbor cell according to current SIB32-NB

Proposal 4: For NB-IoT UE, distance-based trigger for triggering intra and inter frequency measurement in connected mode is not supported in IoT-NTN.

Proposal 5: At least the time-based trigger mechanism for triggering neighbor cell in connected adopted for NB-IoT can be used for eMTC in IoT-NTN.

Reference

- R2_119-e_meeting_report_v1

- R2_119bis-e_meeting_report_v1