3GPP TSG-RAN WG2 Meeting #120		R2-2211313

Toulouse, France, 14th – 18th November, 2022

Source:	CATT

Title:	Discussion on Location Based CHO Mechanism

Agenda Item:	8.6.3.2

Document for:	Discussion and Decision

Introduction

In RAN2#119bis-e meeting, the mobility enhancements were discussed and the following agreement was achieved.

Agreements:

- For eMTC over NTN, for both earth-moving and earth-fixed cell scenarios, we introduce location based CHO triggering events

In this contribution, the location based CHO will be further discussed for both earth-moving and earth-fixed cell scenarios. And observations as well as proposals will be provided.

Discussion

  - Quasi-earth-fixed cell scenario

In Rel-17, the location based CHO CondEvent D1 was introduced for quasi-earth-fixed cell scenario in NR NTN WI. For CondEvent D1, it was agreed that, the event is met when the distance between UE and the PCell’s reference location becomes larger than absolute threshold1 and the distance between UE and the candidate cell becomes shorter than absolute threshold2. The CondEvent D1 was specified as follow [1].

#### 5.5.4.15 Event D1 The UE shall: 1> consider the entering condition for this event to be satisfied when both condition D1-1 and condition D1-2, as specified below, are fulfilled; 1> consider the leaving condition for this event to be satisfied when condition D1-3 or condition D1-4, i.e. at least one of the two, as specified below, are fulfilled; Inequality D1-1 (Entering condition 1) Ml1-Hys>Thresh1 Inequality D1-2 (Entering condition 2) Ml2+Hys<Thresh2 Inequality D1-3 (Leaving condition 1) Ml1+Hys<Thresh1 Inequality D1-4 (Leaving condition 2) Ml2-Hys>Thresh2 The variables in the formula are defined as follows: Ml1 is the UE location, represented by the distance between UE and a reference location parameter for this event (i.e. referenceLocation1 as defined within reportConfigNR for this event), not taking into account any offsets. Ml2 is the UE location, represented by the distance between UE and a reference location parameter for this event (i.e. referenceLocation2 as defined within reportConfigNR for this event), not taking into account any offsets. Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event). Thresh1 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference1, from a reference location configured with parameter referenceLocation1 within reportConfigNR for this event. Thresh2 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference2, from a reference location configured with parameter referenceLocation2 within reportConfigNR for this event. Ml1 is expressed in meters. Ml2 is expressed in the same unit as Ml1. Hys is expressed in the same unit as Ml1. Thresh1 is expressed in the same unit as Ml1. Thresh2 is expressed in the same unit as Ml1. NOTE: The definition of Event D1 also applies to CondEvent D1.

Actually, there is no different between NR NTN and eMTC over NTN in aspect of UE capability, scenario and requirement, etc. For quasi-earth-fixed scenario, the location based CHO introduced for NR NTN in Rel-17 is also applicable for eMTC over NTN.

Proposal 1: For quasi-earth-fixed cell scenario of eMTC over NTN, reuse the location based CHO mechanism introduced in Rel-17 NR NTN.

  - Earth-moving cell scenario

For earth-moving cell scenario, the coverage area of cell slides over the earth surface. Under the coverage of the satellite, some new UEs move in, while some move out. For example, as shown in Figure 1, the cell is moving with a speed V on earth surface, and “△” and “□” are represent two UEs, i.e., UE1 and UE2, in coverage of cell#0 at T1. Without considering the movement of UE, it can be predicted these two UEs will lose coverage of cell#0 after the cell#0 moves distance D2 for UE2 and D1 for UE1 successively. In this case, there is no unified time of losing coverage for UE with different locations in the cell.

Figure 1: Earth moving cell scenario

Observation 1: For earth-moving cell scenario, the time of losing coverage is different for UEs with different locations in the cell.

To realize location based CHO in earth moving scenario, the CondEvent D1 which considers the distance between UE and the reference location of serving cell/candidate cells is also applicable for eMTC over NTN. For earth moving cells, the reference location keeps changing, that the reference location used in CondEvent D1 should be real-time position.

Proposal 2: For location based CHO in earth-moving cell scenario, CondEvent D1 is also applicable, and the reference location used in CondEvent D1 should be real-time position.

To support location based CHO for earth-moving cell, the main issue is how to determine the real-time cell position. Obviously, it will bring large energy consumption to make the network keep updating the cell reference location via dedicated signaling or UE frequently read the system information. To solve this problem, Three UE-assisted solutions are given as follow:

- Option A: Provide UE the coordinate of reference location with timestamp and the moving speed of cells. UE can calculate the real time reference location based on the reference location with the timestamp and the moving speed. One example is shown as the following figure.

Figure 2: calculate the real time reference location by coordinate with timestamp.

- Option B: Provide UE the relative coordinate of the cell center, the relative coordinate is reference to the coordinate of sub-satellite point. The coordinate of sub-satellite point can be calculated by the ephemeris of satellite. Additionally, the cell radius or a threshold is also needed for UE to calculate the cell edge.

Figure 3: Relative coordinates reference to the coordinate of sub-satellite point.

- Option C: Provide UE the beam antenna angle of the cell center. The beam angle of the cell center is reference to the direction of sub-satellite point. With the information of beam antenna angle, UE can calculate the coordinate of cell center. Additionally, the cell radius or a threshold is also needed for UE to calculate the cell edge.

Figure 4: Beam antenna angle reference to the direction of sub-satellite point.

For option A, after receiving this assistance information, UE can calculate the real time reference location in certain time duration. If the UE does not update the assistance information, this may impact the accuracy for CHO evaluation. However, if the UE updates the assistance information timely, this definitely brings negative impacts on power consumption.

For option B and option C, the relative coordinate reference to sub-satellite point or the beam antenna angle is unchanged for earth moving cell. There is no need to update this assistance information during the whole serving time. These two options make full use of the time information contained in the ephemeris, which is already supported in Rel-17, and have advantages on the signalling aspect than option A.

Hence, we have a preference on option B and C.

Proposal 3: RAN2 to discuss the following two options on how to determine the real-time reference location of earth-moving cell.

- Provide UE the relative coordinate of the cell center. The relative coordinate of cell center is reference to the coordinate of sub-satellite point.

- Provide UE the beam antenna angle of the cell center. The beam angle of the cell center is reference to the direction of sub-satellite point.

Additionally, there is no different between NR NTN and eMTC over NTN in aspect of UE capability, scenario and requirement, etc. For earth-moving cell scenario, the above discussion can also apply to NR NTN.

Proposal 4: For earth-moving cell scenario, the enhanced mechanism of location-based CHO is also applicable for NR NTN.

- Conclusion

In this document, we discuss the location based CHO mechanism. According to the analysis in section 2, the following observations and proposals are provided:

Quasi-earth-fixed cell scenario:

Proposal 1: For quasi-earth-fixed cell scenario of eMTC over NTN, reuse the location based CHO mechanism introduced in Rel-17 NR NTN.

Earth-moving cell scenario:

Observation 1: For earth-moving cell scenario, the time of losing coverage is different for UEs with different locations in the cell.

Proposal 2: For location based CHO in earth-moving cell scenario, CondEvent D1 is also applicable, and the reference location used in CondEvent D1 should be real-time position.

Proposal 3: RAN2 to discuss the following two options on how to determine the real-time reference location of earth-moving cell.

- Provide UE the relative coordinate of the cell center. The relative coordinate of cell center is reference to the coordinate of sub-satellite point.

- Provide UE the beam antenna angle of the cell center. The beam angle of the cell center is reference to the direction of sub-satellite point.

Proposal 4: For earth-moving cell scenario, the enhanced mechanism of location-based CHO is also applicable for NR NTN.

- Reference

- 38.331, V17.1.0;