3GPP TSG-RAN WG2 Meeting #120		R2-2211317

Toulouse, France, Nov14th – Nov 18th, 2022

Source:	CATT

Title:	Discussion on NTN HO Enhancements

Agenda Item:	8.7.4.2

Document for:	Discussion and Decision

Introduction

In RAN2#119bis-e meeting, the following agreements were made on HO enhancements for NTN[1]:

Agreements

- RAN2 can further consider whether some information in the handover command that can be common to all UEs, can be delivered to UEs in common signalling and if there is real benefit (in terms of signalling overhead reduction) in this

- Send an LS to RAN1 (cc RAN4) listing the scenarios (intra-satellite, inter-satellite with same or different feeder links) and check with RAN1 in which scenarios RACH-less is possible (with no indication of RAN2 preference)

Agreements:

- Continue the discussion (in future meeting) on group HO / “UE specific pre-configuration of the target cell + group HO” indication in the next meeting, also on the possible real benefits

Agreements:

- RAN2 confirms that at least for the moving cell case the next serving cells can be largely predicted in NTN (at least for UEs not at the cell edge) thanks to the existence of predefined satellite orbits and negligible UE’s mobility in comparison to satellite’s motion (we can further discuss at the next meeting whether this applies to idle mode UEs as well)

It can be seen that solutions addressing mobility enhancements in NTN have been discussed and provided by companies. In this contribution, we further discuss the solutions for HO enhancements in NTN and our proposals are provided.

Discussion

2.1	Earth moving scenario

In R17, the quasi-earth-fixed cell scenario was studied. Since the UE is served by beam covering one geographic area for a limited period in quasi-earth-fixed cell, the stop serving time of the serving cell is unified and the coverage area of the serving cell is fixed. Therefore, these information are always valid for all the UEs under the quasi-earth-fixed cell. As for mobility management, NW can determine CHO execution conditions based on the stop serving time and the coverage area of the serving cell. Hence, CondEvent T1 was defined in Rel-17, i.e., UE can perform CHO execution if the RRM condition is fulfiled[2], during the time [t1-Threshold, t1-Threshold + duration], considering the execution condition determained by the network is mainly based on the stop serving time of the serving cell and the start serving time of candidate cells. And event D1 was specified for the UE to consider the distance between the UE location and fixed reference loaction.

However, in earth moving cell scenario, the coverage area of cell slides over the earth surface, under the coverage of the satellite, some new UEs move in, while some move out. In this case, there is no unified cell stop serving time for UE with different locations in the cell, and the cell center or cell edge keeps moving together with the cell, i.e., the stop time of serving cell and start time of candidate cells for each UE are various.

To support time based CHO, gNB need to calculate the CHO execute times based on the current location reported by UE, i.e. calcuating the stop serving time of the serving cell and the start serving time of the neighbor cell according to the UE position, and the serving cell and neighbor cell movement trajectory. And configure the CHO execution conditions to UE. In this case, if the UE is moving and the reported location information is invalid, the stop serving time of the serving cell and the start serving time of the candidate cell will change, these execution conditions may become invalid. Reconfiuration of the excution condition from NW and location information report from UE may needed.

Observation 1: Currently, the time-based CHO can work for earth-moving scenario, if the UE can report the location information to NW, but it may lead in signaling overhead on update the excution condition.

For location based solution for earth moving cell, the reference location (e.g. center of the cell or edge of the cell) varies with timing in earth moving scenario. If the UE is not aware of the timing information for the reference location, the UE cannot make proper decision. The current location-based CHO is not applicable for earth moving scenario.

Observation 2: Current distance-based CHO is not applicable to earth moving cell scenario.

Solutions are needed  for CHO in earth moving scenario. For time-based CHO, the network may be not able to get the UE location considering the UE does support reporting location or the network does not have the user consent. So, time-based CHO solution is not applicable to all the UEs. While for distance-based CHO, UE location report is not one prerequisite condition. So it  can be applied to all UEs. Therefore, enhanced distance-based CHO can be studied for earth-moving scenario.

One straightforward solution is that the network keeps updating the cell reference location via dedicated or common signaling. Obviously, this will bring great burden on signaling overhead and large power consumption for frequent signaling updating which is not acceptable. In order to faciliate the procedure, assitance information from network are needed and 3 solutions are provided:

- Solution 1: Network providing the coordinate of reference location with timestamp and the moving speed of cells.

In Solution 1, the UE calculates the real timer reference location based on the information including reference location with timestamp and the moving speed. One example is shown in Figure 1:

Figure 1: Calculating the real time reference location by coordinate with timestamp

- Solution 2: The network providing UE the relative coordinate of cells center, the relative coordinate is reference to the coordinate of sub-satellite point.

In Solution 2, the coordinate of sub-satellite point can be calculated by the ephemeris of satellite. UE could get the center of the cell according to the sub-satellite point and the relative position of other cell in the satellite. So it can acquire the real-time reference location for this cell in the satellite. This is illustrated in Figure 2.

Figure 2: Relative coordinates reference to the coordinate of sub-satellite point.

- Solution 3: The network providing UE the beam antenna angle of cells center.

In Solution 3, the beam angle is reference to the direction of sub-satellite point which is calculated by the ephemeris of satellite. With the information of beam antenna angle, UE can calculate the cell center. The UE can decide whether the CHO condition is satisfied or not by comparing configured threshold and the distance between UE and cell center. .

Figure 4: Beam antenna angle reference to the direction of sub-satellite point.

For Solution 1, the network may update the assistance information periodically considering there may be some UEs may enter into the coverage of the satellite. If the UE does not update the assistance information, the accuracy for estimation on the reference location may be impacted. If the UE updates the assistance information, this definitely brings extra power consumption.

For Solution 2 and 3, the relative coordinate reference to sub-satellite point or the beam antenna angle is unchanged for earth moving cell. There is no need to update this relative information frequently. These two options make full use of the time information contained in the ephemeris, which is already supported in Rel-17.

Hence, we have a preference on Solution 2 and 3 and propose that:

Proposal 1: RAN2 to discuss the following two solutions on how to determine the real-time reference location of earth-moving cell:

- providing UE the relative coordinate of cells center, the relative coordinate is reference to the coordinate of sub-satellite point;

- providing UE the beam antenna angle of cells center.

2.2	Broadcast the common configuration for HO/CHO

In NTN, the HO is mainly caused by the movement of satellite due to the large coverage of NTN cell. And the satellite topology as well as the trajectory of the satellite does not change frequently. Therefore, the target cell can be predicated among coming cells.

Observation 3: For handover in NTN scenario, the target cell can be predicted due to the stable satellite trajectory and topology.

In accordance to Observation 3, it can be seen that the common configuration is same for most of the UEs and the size for the common configuration signalling is limited. Hence, it is not efficient to provide the same common configuration of the same cell to different UEs by dedicated signalling separately. And it is feasible to provide the common configuration via broadcasting which is beneficial to reduce signalling overhead and power consumption.

Proposal 2: Broadcasting common configuaration of the neigbour cells for HO/CHO command can be supported in NTN in Rel-18.

If the mechanism of broadcasting common configuration of HO/CHO command is supported, some open issues need to be further analyzed. For example:

- Which parameters to be transmitted in the broadcast common configuration for neighbor cells for HO/CHO;

- What mechanism e.g. using current SI mechanism based on SIB1 scheduled and periodically broadcast, is applied to the broadcast of common part of HO/CHO command;

- Whether and how to handle the difference of broadcasting common part of HO/CHO command in different scenarios, e.g. earth moving cell and quasi-earth fixed cell.

- Whether feedback for the broadcasted common part of HO/CHO command is needed.

2.3	Group HO

Some companies propose to support group HO in NTN. In this scheme, the UEs can be divided into some groups so that the network can manage the whole group of the UEs instead of single UE.

To support group HO, the assistance information is needed to be reported to the network which can be utilized to improve the accuracy of the subgrouping. For example, based on the location information, the network can divide the UEs with close distance into the same group. When the satellite can’t provide the service for this group, it can indicate the UEs in this subgroup to perform HO. It can be seen that the procedure for assistance information reporting is similar to measurement reporting. And likely, when the UE’s location changes, the UE needs to report its update location. Otherwise, the group may be not accurate and the group HO may fail.

Observation 4: The group management for group HO requires frequent assistance information reported from UE.

After grouping, the common information can be sent by broadcast/multicast message, e.g. RACH common information of the target cell which is similar with the mechanism broadcasting common configuration of HO/CHO in Section 2.3. From this point of the view, signaling overhead reduction via broadcasting common configuration of HO/CHO can be achieved independent of group HO.

Observation 5: Signalling reduction via broadcasting common configuration of HO/CHO can be achieved independent of group HO.

Some UE specific information, e.g. C-RNTI for the target cell, dedicated preamble, dedicated radio resource configuration and group ID needs to be sent to the UE by dedicated signaling separately, considering the security issue, resource collision and other reasons.

Meanwhile, the HO indication can be sent by group signaling in order to reduce the signaling overhead, i.e. the UEs in the same group can be indicated by single HO indication. It should be noted that the UE may perform HO based on the Group configuration or Group indication. But this requires special handling on how to design the group HO indication, e.g. special RNTI configuration or DCI format.

Observation 6: Special handling is needed on group HO indication, e.g. RNTI configuration or signalling design.

Hence, it seems no outstanding benefits on signaling overhead are achieved by group HO in NTN. So, it is suggested study group HO with low priority.

Proposal 3: RAN2 to study group HO when great benefits of group HO on signalling overhead are identified in NTN.

Conclusion

In this document, we analyse issues for HO enhancement in NTN, and we find the observations and proposls as following:

Earth moving scenario:

Observation 1: Currently, the time-based CHO can work for earth-moving scenario, if the UE can report the location information to NW, but it may lead in signaling overhead on update the excution condition.

Observation 2: Current distance-based CHO is not applicable to earth moving cell scenario.

Proposal 1: RAN2 to discuss the following two solutions on how to determine the real-time reference location of earth-moving cell:

- providing UE the relative coordinate of cells center, the relative coordinate is reference to the coordinate of sub-satellite point;

- providing UE the beam antenna angle of cells center.

Broadcasting the common configuration for HO/CHO:

Observation 3: For HO in NTN scenario, the target cell can be predicted due to the stable satellite trajectory and topology.

Proposal 2: Broadcasting common configuaration of the neigbour cells for HO/CHO command can be supported in NTN in Rel-18.

Group HO:

Observation 4: The group management for group HO requires frequent assistance information reported from UE.

Observation 5: Signalling reduction via broadcasting common configuration of HO/CHO can be achieved independent of group HO.

Observation 6: Special handling is needed on group HO indication, e.g. RNTI configuration or signalling design.

Proposal 3: RAN2 to study group HO when great benefits of group HO on signalling overhead are identified in NTN.

Reference

- RAN2#119bis-e meeting report;

- 38.331, V17.2.0, NR; Radio Resource Control (RRC) protocol specification, Release 17;