3GPP TSG-RAN WG2 Meeting #120	R2-2211323

Toulouse, France, 14th – 18th November, 2022

Source: 	vivo

Title:  	Discussion on cell reselection enhancement in NR NTN

Agenda Item:	8.7.4.1

Document for:	Discussion and Decision

Introduction

In RAN2#119bis-e [1], the following agreements regarding enhancements to cell reselection procedure were achieved:

Agreements:

- For NTN-NTN cell reselection with earth moving cell, RAN2 will consider providing parameters of serving cell to  UE, for UE to estimate when the serving cell stops providing coverage at the present UE location (FFS whether this will be an optional UE feature) (this does not exclude any time-based or location-based approach) (other solutions can also be considered)

- To enhance NTN-TN cell reselection, means are defined for a UE to differentiate when camping in an area only covered by NTN network (earth-moving or earth-fixed) vs an area where TN network(s) is/are also available.

Agreements:

- System information is the basic means for providing necessary parameters to assist UE to estimate when the serving cell stops providing coverage at the present UE location.

- UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage.

- The method of detecting the transmission energy or SIB presence to determine the NTN coverage when a UE currently camps on a TN cell is not pursued.

- In Earth-moving cell, the reference location and distance threshold of serving cell are provided by network for UE to estimate when the serving cell stops providing coverage at the present UE location. FFS how the reference location and/or distance threshold are provided to the UE

In this contribution, we provide our analyses and proposals to resolve the remaining issues on cell reselection enhancement, mainly including the following three aspects:

- Location-based cell reselection enhancement for earth-moving cell;

- Time-based cell reselection enhancement for earth-moving cell;

- Power saving for the NTN-TN mobility.

Discussion

Location-based cell reselection for earth-moving cell

4.   In Earth-moving cell, the reference location and distance threshold of serving cell are provided by network for UE to estimate when the serving cell stops providing coverage at the present UE location. FFS how the reference location and/or distance threshold are provided to the UE

According to the above agreements in RAN2#119bis-e, in the earth-moving cell, the reference location and distance threshold of serving cell are provided by the network to the UE. In our understanding, when reference location and distance threshold of serving cell in earth-moving cell are obtained by the UE, regardless of the signalling details on how these parameters are provided, the location-based measurement rules as specified in Rel-17 NTN for quasi-earth fixed cell case can be reused in the earth-moving cell case, i.e., if the distance between UE and the serving cell reference location is larger than distance threshold, the UE shall perform neighbour cell measurement for cell reselection purpose.

Proposal 1: For the earth-moving cell, if the distance between the UE and reference location of the serving cell is larger than a configured distance threshold, the UE shall perform neighbour cell measurement for cell reselection purpose.

However, based on the agreements above, the signalling details on how the reference location and/or distance threshold are provided to the UE is still left as an FFS point. For the distance threshold of serving cell in earth-moving cell, it can be broadcast by the network and the existing distanceThresh IE can be reused simply.

Proposal 2: For the earth-moving cell, the existing parameter distanceThresh can be reused to provide the distance threshold for the location-based cell reselection.

Regarding how to signal the time-variant reference location for the earth-moving cell, during the discussion in [2] in last meeting, several companies proposed the NW-based solution for the UE to determine how the reference location varies w.r.t. to time, i.e., the reference location is provided with also the validity time information to track its variance by the network, and the UE updates the reference location once the reference location currently used becomes is invalid. This NW-based solution is simple and straightforward, with the UE simply needing to follow the NW indication on how to update the reference location, and we prefer such a NW-based solution to the solution that requires the UE to derive/update the time-variant reference location based on the satellite ephemeris autonomously.

With respect to the signalling details on how reference location and its validity time information are provided, there were two ways proposed by companies in [2]. As one possible way, the network provides a series of multiple reference location values each of which is associated with the validity time that it applies at one time, and the UE determines the reference location based on the current time. AS the other possible way, the network provides one reference location and its validity timer at a given time; if the reference location becomes invalid, e.g., upon the validity timer expiry, the UE re-acquires the reference location from the network and use the updated. For the former way, there seems additional signalling overhead caused to the system information than the latter solution. So the latter way, i.e. reference point with associated validity timer, is slightly preferred from our perspective.

Proposal 3: For the earth-moving cell, the reference location is signalled with an associated validity timer in the system information.

Proposal 4: If the validity timer of the reference location expires, UE re-acquires the reference location from the network.

Since validity timer of satellite ephemeris and common TA parameters (i.e., T430) was introduced in Rel-17, it is natural to come up with the question regarding whether to introduce a new validity timer specific to this reference location update or to reuse the existing T430. From our perspective, the reference location is a two-dimensional point on the ground, so it cannot be guaranteed that its variance can be completely covered by the variance of the satellite ephemeris which indicates the motion of the satellite in space. Since the T430 was introduced to mainly cover the change of the satellite ephemeris it may not be able to well cover the change of the two-dimension reference location as discussed here, we think a new timer specific to the validity of the reference location should be introduced.

Proposal 5: For the earth-moving cell, a new timer should be introduced as the validity timer for the reference location of the serving cell.

According to the discussion in [2], another issue raised by companies is whether the cell type (i.e., quasi-earth fixed cell or earth-moving cell) should be provided explicitly or not for this cell reselection enhancement purpose. In our understanding, different reference location parameters are provided by earth-moving cell and quasi-fixed cell (i.e. time-variant or not) for location-based measurement initiation, and the UE simply needs to follow the configuration from the NW and perform corresponding procedures. As a result, an explicit cell type indication is not needed.

Proposal 6: An explicit cell type indication (i.e., quasi-earth fixed cell or earth-moving cell) to the UE is not needed.

Time-based cell reselection for earth-moving cell

For earth-moving cell case, there can be two types of serving cell stop time which are respectively caused by service link switch (as shown in figure 1 below), and caused by feeder link switch (as shown in figure 2 below). Regardless of the reason why the network stops providing service to the UE, UE needs to perform cell reselection. These two types of stop time are different in nature from the cell reselection perspective, and cannot be covered by a common solution in the earth moving cell case. We use the below figures to illustrate their difference.

Figure 1: Service link switch without feeder link switch for moving cell

Figure 2: Feeder link switch without service link switch for moving cell

For the stop time due to service link switching as in Figure 1, this is the stop time caused by “the coverage of the satellite beam sliding over the earth surface” as consistently emphasized by companies. More specifically, the UEs distributed at different places in the cell may face different stop time, i.e, the stop time due to service link switch in earth-moving cell is UE-specific. A UE can estimate when the serving cell stops providing coverage at the present UE location due to service link switching based on the time-variant reference location and distance threshold, i.e., when the UE considers the distance between UE and reference location of serving cell is larger than distance threshold, it judges the stop time reaches/approaches. In a word, the stop time due to the service link switching is a UE specific time, and it can be judged with a similar rule as that used for the location-based cell reselection enhancements in Proposal 1.

By contrast, when feeder link switch occurs as shown in Figure 1, the service cell stops providing service to all the UE in the cell suddenly and all UEs need to perform cell reselection. That is, the stop time due to feeder link switching in earth-moving cell is common to all UEs, and is irrelevant to the stop time caused by the “cell coverage sliding over the earth surface” (see the sub-figure in the middle). In a word, when talking about the stop time due to feeder link switching in the earth-moving cell, it refers to a stop time that is common to all UEs in the cell, and has Nothing to do with the movement of the cell coverage over the earth surface. Actually, the situation caused by the stop time due to feeder link switching has no big difference between the earth-moving cell case and the quasi-earth fixed cell case, so the stop time due to feeder link switching can be broadcast by the network similar as t-Service specified in Rel-17.

Observation 1: In the earth-moving cell, the stop time due to service link switching is caused by “cell coverage sliding over the earth surface” and it is a UE specific value. By contrast, the stop time due to feeder link switching is irrelevant to the stop time caused by “cell coverage sliding over the earth surface” and it is a UE-common value.

In the practical deployment, one cannot guarantee that feeder link switching never happens in the earth-moving cell case (though perhaps not happening as frequently as service link switching), the stop time due to feeder link switching needs to be considered together with the stop time due to service link switching. Since the stop time due to service link switch and that due to feeder-link switch needs different handling as illustrated above, we propose that cell reselection enhancements due to these two types of stop time should be supported in the earth-moving cell case. Specifically, the UE shall start neighbour measurements before either type of stop time is reached (e.g. though the UE may not necessarily be made aware of whether a stop time is caused by service link switch or feeder link switch).

Proposal 7: For the time-based measurement initiation in the earth moving cell, two types of stop time (which intends to handle the stop time due to service-link switching and feeder-link switching respectively) should be supported.

Proposal 8: Similar to t-Service, the gNB configures a cell-level serving cell stop time that covers the stop time due to feeder-link switching in the earth-moving cell.

Proposal 9: Similar to the location-based criterion, the UE calculates a stop time based on the distance between the UE and reference location of serving cell to cover the stop time due to service-link switching.

Proposal 10: The UE should start neighbour cell measurements before either type of stop time is reached.

Power saving for the NTN-TN mobility

Regarding power saving for NTN-TN mobility. The following agreements were reached in the last meeting:

2.  UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage.

Referring to the figure below, in our understanding, the situation described in the above agreement is that for UE camping in an NTN cell and the UE is not in any TN network coverage (e.g., UE1 in figure 3), the UE is not required to perform neighbour cell measurements for TN neighbour cells. Simply following the current agreement, UEs that are in the area where there is no TN network coverage at all can save power with TN neighbour cells not measured, but for the UE that is in the TN area, UE may need to measure all the TN frequencies once the TN frequency measurement is initiated, regardless of by which specific TN frequency the UE is actually covered. This, i.e. requiring the UE to measure all possible TN frequencies, is obviously not power efficient, as it is not guaranteed that the UE can find TN cell on whatever TN frequency it measures. So to realize the power saving gain, we think for the UE in TN coverage for some frequencies, the UE does not need to measure the other TN frequencies if the UE is not in the coverage of that TN frequency. Take UE2 in figure 4 as an example, UE2 is in an area where TN cell on f3 provides coverage, UE2 only needs to perform measurement TN neighbour cells on f3, but is not required to perform measurement TN neighbour cells on f1 and f2.

Figure 3: Example of UE camping on NTN cell which has TN neighbour cells

Proposal 11: For NTN-TN mobility, an RRC_IDLE/RRC_INACTIVE UE is not required to perform neighbour cell measurements for a TN frequency in the area where there is no coverage of that frequency.

The next issue that needs to be discussed is what specific assistance information should be provided by the network for UE decides whether it is within/adjacent to the TN network coverage. For simplicity, we think a reference point and distance threshold can be used to describe TN coverage. Compared with other complicated solution, e.g. providing TN coverage border, this solution is much less complicated and friendly to UE implementation, and due to the irregular shape of the cell coverage formed by radio signals, it is hard to say that the TN coverage border can really be accurately reflected, even with much more complicated solution is used. Furthermore, in order to save power for the UE that is in the TN area according to Proposal 11 above, the assistance information can be provided at a frequency level.

Proposal 12: A reference point and a distance threshold can be used to indicate TN coverage. They can be configured per frequency to indicate the TN coverage provided by the corresponding frequency.

Conclusions

Based on the analyses given above, we have the following observations and proposals:

Observation 1: In the earth-moving cell, the stop time due to service link switching is caused by “cell coverage sliding over the earth surface” and it is a UE specific value. By contrast, the stop time due to feeder link switching is irrelevant to the stop time caused by “cell coverage sliding over the earth surface” and it is a UE-common value.

Proposal 1: For the earth-moving cell, if the distance between the UE and reference location of the serving cell is larger than a configured distance threshold, the UE shall perform neighbour cell measurement for cell reselection purpose.

Proposal 2: For the earth-moving cell, the existing parameter distanceThresh can be reused to provide the distance threshold for the location-based cell reselection.

Proposal 3: For the earth-moving cell, the reference location is signalled with an associated validity timer in the system information.

Proposal 4: If the validity timer of the reference location expires, UE re-acquires the reference location from the network.

Proposal 5: For the earth-moving cell, a new timer should be introduced as the validity timer for the reference location of the serving cell.

Proposal 6: An explicit cell type indication (i.e., quasi-earth fixed cell or earth-moving cell) to the UE is not needed.

Proposal 7: For the time-based measurement initiation in the earth moving cell, two types of stop time (which intends to handle the stop time due to service-link switching and feeder-link switching respectively) should be supported.

Proposal 8: Similar to t-Service, the gNB configures a cell-level serving cell stop time that covers the stop time due to feeder-link switching in the earth-moving cell.

Proposal 9: Similar to the location-based criterion, the UE calculates a stop time based on the distance between the UE and reference location of serving cell to cover the stop time due to service-link switching.

Proposal 10: The UE should start neighbour cell measurements before either type of stop time is reached.

Proposal 11: For NTN-TN mobility, an RRC_IDLE/RRC_INACTIVE UE is not required to perform neighbour cell measurements for a TN frequency in the area where there is no coverage of that frequency.

Proposal 12: A reference point and a distance threshold can be used to indicate TN coverage. They can be configured per frequency to indicate the TN coverage provided by the corresponding frequency.

Reference

- Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online

- R2-2210860 Report of [AT119bis-e][117][NR NTN Enh] cell reselection enhancements (Intel)