3GPP TSG-RAN WG2 Meeting #120		R2-2211484

Toulouse, France, 14th Nov. – 18th Nov. 2022

Source:	vivo

Title:	Improve Handover Performance with LTM

Agenda Item:	8.4.2.1.1

Document for:	Discussion and Decision

- Introduction

In RAN2#119bis-e meeting, the target performance enhancements for LTM in Rel-18 further NR mobility enhancements WI [1] were discussed, with the below agreements [2]:

- No security update support in Rel-18 with L1/L2 based mobility. - FFS whether ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration. FFS if this need to be specified. - For UE processing, the following (not exhaustive) is assumed to be performed after receiving the cell switch command: MAC/RLC reset (when configured) RF retuning (e.g. needed for inter-frequency), baseband retuning - R2 assumes that the following items may be discussed by RAN1 and RAN4 (and may be scenario specific): - Whether to perform DL synchronization to candidate/target cell before receiving the cell switch command. R2 assumes this is feasible at least for the case that the target cell is already an active serving cell. - Whether to support of performing TRS tracking and CSI measurement of candidate/target cell before/by cell switch command - L1L2 based mobility supports the following CA scenarios: PCell change without SCell change PCell change with SCell change - Support NR-DC scenario in L1L2 based mobility, at least for the PSCell change without MN involvement case, i.e. intra-SN.

In this contribution, we will further discuss the potential target performance enhancements for LTM and the scenarios.

- Discussion

  - Target Performance Enhancements for LTM

    - Background

In RAN2 #119-e meeting [3], companies reached a consensus on the components of mobility latency illustrated in Figure 1.

Figure 1: Components of Mobility Latency

Besides, RAN2 reached the following agreement to further study relevant enhancements including UE reconfiguration, DL synchronization, UL synchronization, TRS tracking and CSI measurement for LTM.

- Assumption: To reduce HO interruption time, investigate e.g. solutions to reduce the time for UE reconfiguration (already in the WID), downlink and uplink synchronization after handover decision (other parts of dynamic switch not precluded).

In RAN2#119 bis-e meeting, there was general agreement that DL synchronization, early TRS tracking and CSI measurement should be discussed by RAN1 and RAN4. And, RAN1 will discuss the L1 measurement enhancements.

- R2 assumes that the following items may be discussed by RAN1 and RAN4 (and may be scenario specific): - Whether to perform DL synchronization to candidate/target cell before receiving the cell switch command. R2 assumes this is feasible at least for the case that the target cell is already an active serving cell. - Whether to support of performing TRS tracking and CSI measurement of candidate/target cell before/by cell switch command - RAN2 assumes that RAN1 will drive discussions on L1 measurement enhancements, if any. If RAN1 identifies the need for e.g. event reporting, filtering etc, RAN2 can then be involved if needed.

Based on the above, we think RAN2 need not to further discuss the enhancement on DL synchronization, TRS tracking and CSI measurement unless RAN1/RAN4 triggers and could focus on the enhancements on UE reconfiguration, UE process and uplink synchronization.

    - Pre-configuration process

In RAN2#119bis-e meeting [2], RAN2 discussed whether ASN.1 decoding and validity/compliance check of candidate cell configuration need to be performed upon reception of the candidate cells configuration. The majority of companies supported early ASN.1 decoding and valid/compliance check of candidate cell configuration in order to reduce the LTM latency. However, there were concerns raised on check failures, e.g., the early decoding/check may bring more failures, and the UE behaviour when this check fails. Hence, RAN2 didn’t reach an agreement on this issue.

- FFS whether ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration. FFS if this need to be specified.

Regarding the failure of valid/compliance check, we think it is caused by error implementation of the candidate node and happens rarely. Since the essential target of the LTM is to reduce the handover latency, ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration is crucial. Besides, performing ASN.1 decoding and validity/compliance check of candidate cell configuration upon reception of the candidate cells configuration can be used by UE to obtain the RACH configuration of candidate cell. The RACH configuration of the candidate cell is essential to enable the early  RACH, which was widely proposed in RAN2 and is discussed in RAN1. Hence, we propose:

Proposal 1:  ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon the reception of the candidate cells configuration.

    - L2 reset

In RAN2#119e meeting [3], whether L2 is continued or reset was discussed and reached following conclusion:

R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery.

Besides, in RAN2#119bis-e meeting [2], RAN2 also agreed that the MAC/RLC will be reset when configured. However, it is not clear in what scenarios MAC/RLC should be reset.

For UE processing, the following (not exhaustive) is assumed to be performed after receiving the cell switch command: - MAC/RLC reset (when configured) - RF retuning (e.g. needed for inter-frequency), baseband retuning

For intra-CU inter-DU mobility scenario, the serving DU is changed after the cell switch, i.e. the MAC and RLC entities in the network side are released in the source DU and set up in the target DU during the mobility. Hence, the peer MAC and RLC entities in UE need to be reset and re-established.

Besides, according to the agreement in RAN2#119bis-e meeting:

No security update support in Rel-18 with L1/L2 based mobility.

Hence, PDCP re-establishment isn’t needed for LTM. However, the PDCP data recovery for AM DRB needs to be initiated by the UE to avoid data loss caused by RLC re-establishment. In addition, the PDCP SDU/PDU discarding for SRB needs to be performed, to flush the outdated RRC message for the source cell.

Proposal 2: In intra-CU inter-DU mobility scenario, UE needs to be configured to perform MAC reset, RLC re-establishment. In addition, UE needs to be configured to perform PDCP data recovery for RLC AM DRB and PDCP SDU/PDU discarding for SRB.

For intra-DU mobility scenario, the serving DU doesn’t change after the cell switch. Hence, UE could continue to use the PDCP entity, RLC entity for DRB without re-establish.  Besides, the PDCP data recovery is also not needed since the buffered data in RLC entity won’t be discarded during the cell switch. But the PDCP SDU/PDU discarding for SRB, as well as the re-establishment of SRB RLC entity, are still needed. Otherwise, the RRC message intended to be sent to the source cell will be transmitted to the target cell by mistake.

Proposal 3: In intra-DU mobility scenario, UE is not configured to perform PDCP reestablishment/data recovery, but is configured to perform PDCP SDU/PDU discarding for PDCP entity associated with SRB.

Proposal 4: In intra-DU mobility scenario, UE is not configured to perform RLC reestablishment for DRB, but is configured to perform RLC reestablishment for SRB.

Partial MAC reset was discussed in RAN2#119bis-e meeting [4][5][6]. We agree that the traditional MAC reset may not be needed for intra-DU mobility scenario. For instance, the state of the token bucket for LCP (i.e. Bj) could be maintained and UE could continue the Logical Channel Prioritization (LCP) procedure after the cell switch.  Besides, HARQ entity is configured per cell. If one cell is in both the source and target Cell Groups(i.e. the Cell is not changed during the LTM), the HARQ entity associated with the cell can be kept.

Proposal 5: In intra-DU mobility scenario, UE can be configured to perform partial MAC reset instead of MAC reset. During the partial MAC reset:

- State of token bucket for LCP (i.e. Bj) could be kept;

- HARQ entity associated with unchanged serving cell could be maintained.

    - Early RACH

The meaning of early RACH is UE performs RACH towards a candidate cell before the reception of cell switch command. Early RACH is a typical RACH-less solution for LTM and was proposed by many companies in RAN2.  In the last RAN2 meeting, RACH enhancement in LTM has been studied in RAN1 and RAN2 for latency reduction. And according to the agreement in RAN2#119bis-e, RAN2 expects RAN1 will determine the feasibility of RACH-less procedures for LTM.

RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this.

Meanwhile, RAN1 has agreed the early TA acquisition is supported in RAN1#110bis-e meeting:

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility.

The agreement of RAN1 implies UE doesn’t need to perform RACH towards the target cell after cell switch command is received. Hence, the feasibility of RACH-less procedures for LTM has already been confirmed. Hence, we propose RAN2 to study how to support the early RACH for LTM.

Proposal 6:  RAN2 assumes early RACH(i.e. UE performs RACH towards candidate cell before the reception of cell switch command) is supported in LTM and studies how to support the early RACH.

If proposal 4 is agreed, an LS to RAN1/3/4 is expected, e.g. to ask whether early RACH is feasible for inter-DU or inter-frequency cases.

Proposal 7: Send an LS to RAN1/3/4 if proposal 4 is agreeable.

  - Scenarios for LTM

    - NR-DC scenario

In RAN2#119bis-e meeting [2], RAN2 discussed the NR-DC scenario in LTM, and agreed the PSCell change without MN involvement case is supported.

Support NR-DC scenario in L1L2 based mobility, at least for the PSCell change without MN involvement case, i.e. intra-SN.

However, whether to support the NR-DC scenario that the PSCell change with MN involvement has not been discussed. According to the WID [1], the procedure of LTM is only applicable to the NR-DC case with serving cell change within one CG. In our understanding, at least PSCell change triggered by a L1/L2 cell switch command from MCG should be excluded. That is to say, only the DU of the SN could determine the PSCell change while the DU of the MN couldn’t.

Proposal 8: For NR-DC scenario, PSCell change triggered by a L1/2 cell switch command from MCG is not supported. Similarly, PCell change triggered by a L1/2 cell switch command from SCG is not supported.

    - Collision between LTM and L3 mobility

Even after introducing LTM, the L3 mobility is still needed for some cases, e.g. handover for inter-CU case, or security key refresh for intra-DU case. It is possible that the UE may be instructed to perform LTM and L3 mobility at the same time since the handover procedures are initiated by different network entities (i.e. CU for L3 mobility by and DU for LTM).

An ongoing L3 mobility procedure for key refresh may cause UE to reset MAC and also delay a concurrent LTM procedure. As the LTM procedure is triggered by worse link quality of the serving cell, the delay of LTM procedure may lead to unexpected radio link failure.

In order to avoid such issue, we suggest RAN2 discuss how to handle the collision between LTM and L3-based mobility.

proposal 9: RAN2 to discuss how to handle the collision issue between LTM and L3 mobility.

- Conclusion

In this contribution, we discuss the potential enhancements and the scenario for LTM. Based on the discussion, we have the following observation and proposals:

Proposal 1:  ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon the reception of the candidate cells configuration.

Proposal 2: In intra-CU inter-DU mobility scenario, UE needs to be configured to perform MAC reset, RLC re-establishment. In addition, UE needs to be configured to perform PDCP data recovery for RLC AM DRB and PDCP SDU/PDU discarding for SRB.

Proposal 3: In intra-DU mobility scenario, UE is not configured to perform PDCP reestablishment/data recovery, but is configured to perform PDCP SDU/PDU discarding for PDCP entity associated with SRB.

Proposal 4: In intra-DU mobility scenario, UE is not configured to perform RLC reestablishment for DRB, but is configured to perform RLC reestablishment for SRB.

Proposal 5: In intra-DU mobility scenario, UE can be configured to perform partial MAC reset instead of MAC reset. During the partial MAC reset:

- State of token bucket for LCP (i.e. Bj) could be kept;

- HARQ entity associated with unchanged serving cell could be maintained.

Proposal 6:  RAN2 assumes early RACH(i.e. UE performs RACH towards candidate cell before the reception of cell switch command) is supported in LTM and studies how to support the early RACH.

Proposal 7: Send an LS to RAN1/3/4 if proposal 4 is agreeable.

Proposal 8: For NR-DC scenario, PSCell change triggered by a L1/2 cell switch command from MCG is not supported. Similarly, PCell change triggered by a L1/2 cell switch command from SCG is not supported.

proposal 9: RAN2 to discuss how to handle the collision issue between LTM and L3 mobility.

- References

- RP-221799, Revised WID: Further NR mobility enhancements, MediaTek.

- R2#119bis-e Chair Notes EOM (MediaTek)

- R2#119-e Chair Notes EOM (MediaTek)

- R2-2209480, Enhancements to improve performance for L1 L2 mobility, vivo

- R2-2210722, Target Performance Enhancements and supported scenarios, Huawei

- R2-2209600	, Discussion on latency model of L1 L2 mobility, Intel Corporation