3GPP TSG RAN WG2 #120	R2-2211518

Toulouse, France, 14 Nov – 18 Nov, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on HARQ disabling for NB-IoT NTN

Agenda Item:	8.6.2.1

Document for:	Discussion and decision

Introduction

In RAN2#119b e-meeting, the following agreements were achieved:

Agreements: - For NB-IoT, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signaling (e.g. RRCConnectionSetup). This does not preclude other options (e.g. DCI-based). We can also revert this decision if requested by RAN1. - Disabling HARQ feedback is supported for NB-IoT with single HARQ process, and it is up to eNB implementation whether to disable the HARQ feedback Working Assumption: - Blind retransmission can be used in IoT NTN when HARQ feedback is disabled and when HARQ mode B is used (RAN2 assumes there is no spec change for this) Agreements: - HARQ mode A/B for uplink transmission may be configured per UL HARQ process at least via UE specific RRC signalling for eMTC and NB-IOT NTN. We can also revert this decision if requested by RAN1

In this paper, we will further discuss HARQ disabling for NB-IoT NTN based on the latest RAN1 progress.

Discussion

As agreed in the last RAN2 meeting, for NB-IoT, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signaling (e.g. RRCConnectionSetup). In the meantime, other options (e.g. DCI-based) are not precluded.

In the last RAN1 meeting, the same issue was discussed. According to the latest RAN1 progress [1], two new options (Option 6a-1 to Option 6a-4) are discussed and left for further down selection:

For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select ONE from the following options at RAN1#111: - Option 6a-1: Support RRC signaling configured between Option 1 and Option 3 - Option 6a-4: Support Option 1 by default, and support Option 3 to override default configuration for corresponding transmission

Within the agreements above, Option 1 uses UE specific RRC signaling to configure enabling/disabling of HARQ feedback per HARQ process, and Option 3 is via explicit indication in DCI (e.g., new field or reusing existing field).

Based on the progress in RAN1, RAN2 can have a further discussion.

For Option 6a-1, the network has the flexibility to select between Option 1 and Option 3. Specifically, for CP solution in NB-IoT NTN, NW may use DCI to dynamically enable/disable HARQ as RRC reconfiguration is not supported. While for UP solution, NW may use RRC or DCI to enable/disable HARQ, which depends on whether NW want it to be semi-static or more dynamic.

For Option 6a-4, the eNB first configures the UE via RRC signaling, and then uses DCI to override the RRC configuration. Although Option 6a-4 can serve the same purpose, it is relatively more restrictive as UE should support RRC based HARQ disabling by default and more complex as UE needs to depend on mixed use of different signaling to determine the HARQ enabling/disabling.

Therefore, we propose:

- For NB-IoT NTN HARQ disabling, support both RRC based and DCI based solution. Which solution is used can be configured by RRC. The details of DCI based solution are up to RAN1.

Conclusion

In this contribution, we discussed the HARQ disabling of NB-IoT NTN and have the following proposal:

- For NB-IoT NTN HARQ disabling, support both RRC based and DCI based solution. Which solution is used can be configured by RRC. The details of DCI based solution are up to RAN1.

Reference

[1] R1-2210330	FLS#2 on disabling of HARQ feedback for IoT NTN	Moderator (Lenovo)