3GPP TSG-RAN WG2 Meeting #120	R2-2211521

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	NCR-MT RRM functions

WID/SID:	NR_NetConRepeater - Release 18

Document for:	Discussion and Decision

# 1	Introduction

A new Work Item RP-222673 was approved at 3GPP RAN Meeting #97e on Rel-18 NR Network-controlled Repeaters. One of the objectives of this WI is to:

Study the RRM functions to be supported and specify the RRM requirements of NCR-MT if necessary [RAN2, RAN4]

In RAN2#119bis-e, RAN2 began discussions on the NCR-MT RRM functions, but left most of the functions open FFS, as summarized in R2-2210808:

Agreements RRM functions supported by NCR-MR: - Cell selection is mandatory - Cell reselection, RLM, BFD, BFR are FFS

The following RRM functions were also left FFS since consensus was not reached on the above items:

- RRM measurements in RRC_IDLE and RRC_INACTIVE (if supported); - RRM measurements in RRC_CONNECTED; - Handover

In this contribution we discuss the RRM functions for NCR-MT that were left FFS in RAN2#119-bis-e.

# 2	Discussion

## 2.1	RRM Functions Applicable to NCR-MT

Like Rel-16 IAB, NCR is a stationary network device whose deployment is planned and managed by the network operator. In terms of RRM functionality, NCR-MT is therefore more comparable to IAB-MT than it is to other UEs. Hence, in our view, NCR-MT RRM functions may tentatively be derived from Rel-16 IAB-MT.

Observation 1: NCR-MT is functionally similar to Rel-16 IAB-MT, and NCR-MT RRM functions may be derived from Rel-16 IAB.

IAB Rel-16 RRM requirements and the related RRM functions are specified in TS 38.174. In Table 2.1-1, we summarize the RRM functions that are specified for Rel-16 IAB-MT and whether they are also necessary to NCR-MT.

Table 2.1-1: Rel-16 IAB-MT RRM applicability to NCR-MT

| RRM Procedure | RRM functions (FFS in NCR-MT are highlighted) | Specified for Rel-16 IAB-MT in TS 38.174? | Necessary for NCR-MT? |
| --- | --- | --- | --- |
| Idle and Inactive state mobility | Cell selection | No | Yes - cell selection was agreed mandatory for NCR-MT in RAN2#119-bis-e |
|  | Cell reselection | No | Optional |
|  | RRM measurements in RRC_IDLE | No - however, idle mode measurements are implicitly specified for IAB-MT cell selection in section 5.2.3.2 of TS 38.304 | Yes - at least, measurements for cell selection; measurements for reselection optional |
|  | RRM measurements in RRC_INACTIVE | No | No |
| Connected state mobility | Handover | No | No |
|  | RRC re-establishment | Yes | Yes |
|  | Release with Redirection | Yes | No |
|  | Radio link monitoring | Yes | Yes |
|  | Link recovery, including BFD/BFR | Yes | Yes |
|  | RRM measurements in RRC_CONNECTED: - NR intra-frequency measurements - NR inter-frequency measurements - Inter-RAT measurement | No | No |

Regarding the NCR-MT RRM functions left FFS, we make the following observations and proposals.

It is expected that a network operator would typically deploy the NCR with LOS to the serving gNB. Furthermore, in our view, the NCR-MT will normally operate in RRC_CONNECTED state, while RRC_IDLE would only be used during initial network access or when the C-link fails.

Cell selection is already agreed, and NCR-MT would need to support basic measurement functionality for cell selection.

Like IAB-MT, cell reselection is not an expected requirement for NCR-MT since the device is stationary and would normally have LOS coverage. However, depending on the network deployment scenario (e.g. whether or not the NCR is “locked” onto a specific cell, or may extend coverage from a set of cells) it may be desirable to support cell reselection in some instances.

Observation 2: NCR-MT could in theory follow typical UE functionality for cell reselection, but it is not essential to support this function.

Proposal 1: Cell reselection is optional for NCR-MT.

Observation 3: RRM measurements in RRC_IDLE will be necessary if cell reselection is supported by NCR-MT.

Proposal 2: In RRC_IDLE, NCR-MT supports basic measurements required for cell selection while RRM measurements for cell reselection are supported only when NCR-MT supports cell reselection.

Likewise, handover is also not an expected requirement for NCR-MT. In our view, handover should not be necessary if the C-link can be made robust through other RRM functions. In rare cases where the NCR-MT needs to connect to a different cell due to a severely degraded C-link, NCR-MT may optionally do so after transitioning back to RRC_IDLE (since we already proposed that cell reselection is optional in Proposal 1). This also means that NCR-MT does not need to support RRM measurement procedures related to handover.

Proposal 3: NCR-MT does not support handover and does not support RRM measurements related to handover.

Similarly, we do not view it as necessary to support RRC connection release with redirection.

Proposal 4: NCR-MT does not support RRC release with redirection.

In our view, NCR-MT will mainly operate in RRC_CONNECTED state and should minimize time spent in RRC_IDLE state in order to maintain responsiveness to side-control from the gNB. The C-link between the NCR-MT and gNB should therefore be made robust via radio link monitoring, RRC connection re-establishment procedure, and link recovery procedures (e.g. beam failure detection and recovery). This is similar to the what is required by IAB-MT. Note that RAN4 would need to determine whether RLM and BFD should be based on SSB and CSI-RS, as well as other timing requirements.

Proposal 5: NCR-MT supports RLM, BFD, BFR, RRC re-establishment procedures.

# 3	Conclusion

Based on the discussion in this contribution we made the following observations:

Observation 1: NCR-MT is functionally similar to Rel-16 IAB-MT, and NCR-MT RRM functions may be derived from Rel-16 IAB.

Observation 2: NCR-MT could in theory follow normal UE functionality for cell reselection, but it is not essential to support this function.

Observation 3: RRM measurements in RRC_IDLE will be necessary if cell reselection is supported by NCR-MT.

And proposed the following:

Proposal 1: Cell reselection is optional for NCR-MT.

Proposal 2: In RRC_IDLE, NCR-MT supports basic measurements required for cell selection while RRM measurements for cell reselection are supported only when NCR-MT supports cell reselection.

Proposal 3: NCR-MT does not support handover and does not support RRM measurements related to handover.

Proposal 4: NCR-MT does not support RRC release with redirection.

Proposal 5: NCR-MT supports RLM, BFD, BFR, RRC re-establishment procedures.