3GPP TSG-RAN WG1 Meeting #120 	R2-2211591

Toulouse, France, November 14-18, 2022

Agenda item:	    8.3.4

Source: 		Qualcomm Incorporated

Title: 	NES-aware Cell selection/re-selection

Document for:	Discussion

# 1	Introduction

The SID on Rel-18 network energy saving holds the following objectives [1]:

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed. Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments.

It was agreed to allow Cells enabling specific NES techniques to bar legacy UEs from camping on the cell, especially when the cell is running NES techniques that are not compatible with legacy UEs [2]:

- There is a need to allow NES cells to prevent legacy UEs from camping. FFS the definition of NES cells.

- Whether to bar legacy UEs is configurable by NES cells in Idle/Inactive mode and the network should be able to allow NES-capable UEs to camp on the NES cell.   Options to bar UEs to be considered are 1) UseIntra/InterFreqExcludedCellList (FFS on the exact mechanism and spec impact) and 2) use cellBarred or cell reservation fields in MIB/SIB.

- The network should be able to configure NES capable UEs to (de)prioritize NES cells.  mechanism such as can be considered for both frequency and cell levels cell selection/reselection (de)prioritization.  FFS on whether the existing mechanism is sufficient.

An offline email discussion took place to attempt to identify the remaining issues [3], with the following current proposals:

List of remaining issues on cell selection/reselection: (De)prioritize NES cells by NES capable UEs: a) Whether de-prioritization is sufficient for NES cells, or even prioritization of NES cells need to be supported b) Applicability of existing mechanisms, e.g. frequency priorities, cell offset to (de)prioritize cells c) Potential new mechanism description, and potential specification impacts d) Mechanisms to incentivize and disincentivize NES-capable UEs from camping on cells according to their NES states

# 2 	Preventing Legacy UEs from camping

We start by first clarifying that since there may be multiple NES techniques standardized not compatible with legacy UEs. Thus, there is no need to define an “NES-cell” since it would likely end up meaning a cell applying one or more NES unrelated techniques.

Observation 1: No need to specify an NES-cell definition that is incompatible with legacy.

As agreed in the meeting, a gNB applying an NES technique not compatible with legacy needs to be able form preventing legacy UE from camping. This can be done via the two existing methods discussed in email discussion [3]:

Proposal 1: A cell applying NES techniques incompatible with legacy can us existing options in the spec to prevent legacy UEs from camping, e.g.,

- Use Intra/InterFreqExcludedCellList

- Use cellBarred or cell reservation fields in MIB/SIB

# Barring NES-capable UEs from camping

Aside from preventing legacy from camping on an NES-cell, there is a need to replicate the cell barring and frequency prioritization functionalities for NES-capable UEs that will bypass the initial Cell Barring or Cell blocklisting indication intended for legacy.

Observation 2: There is a need to replicate the barring and blocklisting used to prevent legacy from camping to the NES-capable UEs that may automatically bypass the legacy prevention.

It has been proposed by companies to reuse NTN or IAB mechanisms to enable the NW to prevent NES-capable UEs that will bypass the legacy mechanisms. At the SI phase, RAN2 does not need a specific mechanism for preventing NES-capable UEs from camping on a cell applying NES technique.

Proposal 2: RAN2 to define dedicated IEs for barring and blocklisting NES capable Rel-18 UEs to not impact barring/blocklisting behavior of legacy UEs

# NES-aware Cell (Re)Selection

In NR, Priority based reselection is based on frequency prioritization. Cells with the same frequency priority are ranked based on the cell level RSRP. It is useful to first assess what is the target of possible enhancement of idle mode mobility. Since NES-cells are expected to go into a sleeping mode occasionally for NES reasons, there are reasons to disincentivize an idle UE from selecting (and possibly waking up this cell). On the other hand, using legacy tools such as frequency priority/blocklisting to deprioritize certain cells means that the UE may select a prioritized cell with relatively poor radio quality. Thus, we propose a two-step prioritization as follows:

Proposal 3: Network can influence UE cell (re)selection based on a two-step procedure as follows:

- Legacy Inter-frequency Prioritization

- NES-aware Intra-freq or equal freq ranking Cell (re)selection

While the first step is the legacy inter-frequency prioritization available in SIB, the proposal is to enhance the cell-(re)selection procedure when cells have equal priority to balance these two objectives:

Proposal 4: NES-aware Cell reselection balances the following two objectives:

- NES-state of the cell, i.e, UE is discouraged from accessing a sleeping cell if a comparable non-sleeping cell is measured.

- Cell-level radio quality, a UE does not select a non-sleeping cell if a sleeping cell has much better radio quality.

Having identified the objective for the SI phase, we think RAN2 can discuss how the achieve the above objectives, specifically whether legacy methods such as Qoffset are sufficient or if new enhancements are needed:

Proposal 5: RAN2 to discuss how to achieve NES aware cell ranking.

Finally, we note that WUS of a sleeping cell is already being discussed in RAN1, which if agreed, would influence the process of cell (re)selection, so it is worth to indicate that the UE cell selection/ranking can be combined with WUS, if agreed to constitute an NES-aware cell selection mechanism.

Observation 3: DRS/WUS if agreed, would influence the cell selection procedure as the UE need to select a cell possibly based on DRS (or some other non-legacy measurements).

Proposal 6: WUS and DRS, if agreed, can be combined with the NES-aware cell selection/ranking procedure to obtain an overall UE cell selection mechanism for NES cells.

# Conclusion

Observation 1: No need to specify an NES-cell definition that is incompatible with legacy.

Proposal 1: A cell applying NES techniques incompatible with legacy can us existing options in the spec to prevent legacy UEs from camping, e.g.,

- Use Intra/InterFreqExcludedCellList

- Use cellBarred or cell reservation fields in MIB/SIB

Observation 2: There is a need to replicate the barring and blocklisting used to prevent legacy from camping to the NES-capable UEs that may automatically bypass the legacy prevention.

Proposal 2: RAN2 to define dedicated IEs for barring and blocklisting NES capable Rel-18 UEs to not impact barring/blocklisting behavior of legacy UEs

Proposal 3: Network can influence UE cell (re)selection based on a two-step procedure as follows:

- Legacy Inter-frequency Prioritization

- NES-aware Intra-freq or equal freq ranking Cell (re)selection

Proposal 4: NES-aware Cell reselection balances the following two objectives:

- NES-state of the cell, i.e, UE is discouraged from accessing a sleeping cell if a comparable non-sleeping cell is measured.

- Cell-level radio quality, a UE does not select a non-sleeping cell if a sleeping cell has much better radio quality.

Proposal 5: RAN2 to discuss how to achieve NES aware cell ranking.

Observation 3: DRS/WUS if agreed, would influence the cell selection procedure as the UE need to select a cell possibly based on DRS (or some other non-legacy measurements).

Proposal 6: WUS and DRS, if agreed, can be combined with the NES-aware cell selection/ranking procedure to obtain an overall UE cell selection mechanism for NES cells.

# References

- RP-213544: New SI: Study on network energy savings for NR; TSG RAN Meeting #94, Electronic Meeting, Dec 6 - 17, 2021

- RAN2 119 bise – Chairman Notes

[3] R2-221142 Report of [POST119bis][304][NES] TP on cell selection/reselection and SSB/SIB-less (Huawei)