3GPP TSG-RAN WG2 Meeting #120	R2-2211599

Toulouse, France, 14 – 18 November 2022	R2-2209557

Agenda item:	8.5.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	PDU Discard for XR

WID/SID:	FS_NR_XR_enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This contribution discuses PDU discard for XR services.

# 2	Need for Discard

An LS from SA4 was received in R2-2206969. Although the applicability of the IPB frames was generally questioned, the discarding of PDUs was still mentioned: "In some implementations (note that neither the video codec specifications, nor the IETF RFC, nor 3GPP specifications up to today provide any requirements or recommendation on implementations), the loss of one fragmentation packet of the NAL Unit may result in discarding the entire NAL unit and hence the second part of the PDU definition (which are of same importance requirement at application layer. All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information.) applies. In other implementations, receivers may use the data up to the first lost fragmentation unit to recover at least parts of the video data included in the NAL unit and apply error concealment afterward. In this case, the third part of the PDU Set definition (the application layer can still recover parts of the information unit, when some PDUs are missing) applies, but in this case the equal importance part of the PDU Set definition (which are of same importance requirement at application layer) may be misleading (Note that in this operation mode, as an example if the first packet of the PDU Set is lost, all other packets of the fragmentation units are useless, whereas of the last packet is lost, the decoder can use all packets except the last one."

This has been taken into account by SA2 in the PDU set definition [TR 23.700-60]: "A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services, as used in TR 26.926 [27]). In some implementations all PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In other implementations, the

Observation: discarding of PDUs can be frequent for XR services.

# 3	Discarding PDUs

The discarding of PDUs is currently quite basic in PDCP [38.323] and RLC [38.322]:

-	PDCP reads the following:

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers. For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs. NOTE: Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

-	 While RLC reads the following:

When indicated from upper layer (i.e. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

At PDCP, the current discard procedure can introduce SN gaps as it is left up to implementation. If outOfOrderDelivery is not configured, this will translate into reordering delays.

Observation: discarding data at PDCP can trigger reordering delays, unless outOfOrderDelivery is always configured.

At RLC, the current discard procedure is limited to the scenarios where neither the RLC SDU nor a segment thereof has been submitted to the lower layers. This limitation is aligned between RLC AM and RLC UM, even though in RLC UM there seems to be no strong reason against discarding a remaining tail segment. Besides, the current discarding cannot introduce any SN gap in case of RLC AM.

Observation: requesting RLC to discard SDUs does not always trigger an actual discard.

The mechanisms are quite basic because they were designed under the assumption that discard is more of an error case. For XR services, the assumptions are different: discard can be frequent. Without any optimisations, it would not always be possible to avoid transmitting data that is known to be useless for the receiver, and as a result, the capacity and power saving gains would be limited. Thus for the sake of capacity and power consumption, when discard is triggered for XR services, it should actually occur.

Furthermore, XR services have low latency requirements and any discard procedure should not increase delays due to reordering.

Proposal: the discard procedures in PDCP and RLC should be enhanced to guarantee that discard will actually take place and without triggering additional delays.

# 4	Conclusion

Always echo in the conclusion all proposals made above.

This documents has made the following observations:

Observation: discarding of PDUs can be frequent for XR services.

Observation: discarding data at PDCP can trigger reordering delays, unless outOfOrderDelivery is always configured.

Observation: requesting RLC to discard SDUs does not always trigger an actual discard.

And proposed the following:

Proposal: the discard procedures in PDCP and RLC should be enhanced to guarantee that discard will actually take place and without triggering additional delays.

A corresponding text proposal is given below.

Text Proposal

### 5.3.2	Layer 2 Enhancements

In order to enhance the scheduling of uplink resources for XR, the following improvements are envisioned:

-	New BS table(s) to reduce the quantisation errors in BSR reporting (e.g. for high bit rates);

-	Delay knowledge of buffered data, consisting of at least remaining time, and distinguishing how much data is buffered for which delay.

Editor's Note: FFS if dynamic reporting of the uplink delay (e.g. via BSR) is needed, or whether PSDB is sufficient.

Furthermore, when, for a PDU set, the number of PDUs known to either be lost or associated to discarded SDUs, exceeds a threshold (see subclause 5.1.1), all remaining PDUs of that PDU set could be discarded at the transmitter to free up radio resources. In order to ensure that discard will actually take place and without triggering additional delays in radio protocols, the discard procedures in PDCP and RLC should be enhanced.

NOTE: this means that the granularity of the discard operation at PDCP in the transmitter should be the PDU set.

Editor's Note: the discard operation could also be captured in 5.2.2.