3GPP TSG-RAN WG2 Meeting #120                                                       R2-2211626

Toulouse, France, 14th – 18th Nov, 2022

Source:              CATT

Title:	             Further Discussion on SL-specific Consistent LBT failure

Agenda Item:	8.15.2

Document for:	Discussion and Decision

Introduction

In the RAN2#119bis meeting, the SL-specific consistent LBT failure was initially discussed, and reached the following agreements:

Agreement on consistent LBT failure:

1: 	SL-specific LBT failure indication from PHY is needed for SL-specific consistent LBT failure detection in the MAC. How/whether it is used for other purposes can be further discussed.

2:	Support SL-specific consistent LBT failure detection and recovery procedure in the MAC for SL-U. Details of recovery to be further worked on granularity of (consistent) LBT failure.

3:	Send LS to RAN1 asking “When an SL-specific LBT failure indication is notified for an SL transmission by the PHY, in which resource granularity the SL-specific LBT failure can be considered as being detected (e.g. per Resource Pool, per RB set, per SL BWP, etc.)?

- Detailed wording can be discussed during the email discussion. Some background information (e.g. why/what we (actually) ask) can be also provided.

4:	As the general principle, reuse the consistent LBT failure detection procedure in NR-U as the baseline for SL-specific consistent LBT failure detection in SL-U.

5:	As in NR-U, introduce the following parameters and variables for the SL-specific consistent LBT failure detection in SL-U as the baseline:

- An SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER);

- An SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount);

- An SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer).

6:	Reuse the following MAC behaviors on TIMER/COUNTER handling in NR-U for SL-specific consistent LBT failure detection procedure in SL-U as the baseline:

- As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is incremented by one.

- As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, start or restart the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer)

- As in NR-U, if the SL-specific LBT failure indication counter value is equal to or larger than the SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount), consistent LBT failure is triggered/declared by the MAC entity.

- As in NR-U, if the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) expires, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0.

- As in NR-U, if the maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount) or SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) is reconfigured, SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0.

7:	Support the mechanism that a mode-1 UE can indicate the SL-specific consistent LBT failure to the gNB. FFS on a mode-2 UE in RRC_CONNECTED.

In this contribution, we will further discuss the remaining issues in detailed.

Discussion

Whether SL-specific consistent LBT failure detection is corresponding to cast type/DST/unicast link?

In [1], whether SL-specific consistent LBT failure detection is corresponding to cast type/DST/unicast link was initially discussed, but no conclusion was reached. In our understanding, the LBT is performed per radio resource, it is only Tx UE behavior, not related to cast type/DST/unicast link.

Proposal 1: SL-specific consistent LBT failure detection is not relevant to cast type/DST/unicast link.

How to handle the SL-specific consistent LBT failure for mode-2 UE?

In the last RAN2 meeting, for mode-2 UE, if SL-specific consistent LBT failure happens, there are two issues needs further discussed:

- Issue a): Whether an autonomous SL-specific consistent LBT failure recovery mechanism is needed for a mode-2 UE in SL-U?

- Issue b): Whether mode-2 UE in RRC_CONNECTED can indicate SL-specific consistent LBT failure to gNB?

Regarding to issue a), whether autonomous SL-specific consistent LBT failure recovery is needed also depends on the resource granularity for SL LBT. If the granularity is per SL BWP/SL carrier, considering there is only one SL BWP/SL carrier configured for UE, autonomous recovery is unnecessary. However, if finer granularity is used, e.g., per resource pool or per RB set, it is possible to introduce autonomous recovery.

Proposal 2: For mode-2 UE in RRC_CONNECTED, if the resource granularity of SL LBT is SL BWP/SL carrier, UE autonomous SL-specific consistent LBT failure recovery is unnecessary; otherwise, UE autonomous SL-specific LBT failure recovery can be supported.

Regarding to issue b), similar as the analysis in issue a), whether mode -2 UE should report the SL-specific consistent LBT failure indication to gNB also depends on the resource granularity for detecting the SL LBT failure. If the granularity is per SL BWP/SL carrier, considering there is only one SL BWP/SL carrier configured for the UE, once consistent LBT failure on the SL BWP/SL carrier, UE should report SL RLF to gNB. If finer granularity is used, e.g., per resource pool or per RB set, it is possible for autonomous recovery, only when autonomous recovery is failed, it means SL RLF happens, hence UE should report it to gNB.

Proposal 3: For mode-2 UE in RRC_CONNECTED, if the resource granularity of LBT is SL BWP/SL carrier, UE should indicate to gNB if SL-specific consistent LBT failure happens; otherwise, UE should indicate SL RLF to gNB if SL-specific LBT failure recovery failed.

Which signalling can be used for indicating SL-specific consistent LBT failure to gNB at least for mode-1 UE?

In the last RAN2 meeting, it was agreed that:

Support the mechanism that a mode-1 UE can indicate the SL-specific consistent LBT failure to the

gNB.

But there is no conclusion on which signaling can be used. In our understanding, which signaling can be used depends on the resource granularity for detecting the SL LBT failure:

- If SL LBT failure is detected based on SL BWP/SL carrier, considering there is only one SL BWP/SL carrier, once SL LBT failure is detected, SL RLF will be triggered, hence RRC signaling is valid.

- If SL LBT failure is detected based on more finer granularity, e.g,. based on resource pool or RB sets, MAC CE can be used similar as NR-U.

Proposal 4: If the resource granularity of LBT is SL BWP/SL carrier, once SL-specific consistent LBT failure is detected, SL RLF should be triggered and CONNECTED UE should report the SL RLF to gNB; otherwise, SL-specific LBT failure MAC CE can be used to indicate the failure to gNB.

Whether there is any impact on other MAC procedures due to SL-specific consistent LBT failure?

From RAN2 perspective, the SL-specific consistent LBT failure may impacts on SR, LCP, DRX and CG procedures. The detailed impacts are analysed below:

SR

If SL-specific LBT failure MAC CE is introduced, once SL-specific LBT failure MAC CE is triggered and there is no UL grant, SR should be triggered. The SR resource configured for Sidelink CSI Reporting MAC CE can be reused for SL-specific LBT failure MAC CE.

Proposal 5: If SL-specific LBT failure MAC CE is introduced, once it is triggered and there is no UL grant, SR can be triggered and SR resource configured for Sidelink CSI Reporting MAC CE can be reused.

LCP

If SL-specific LBT failure MAC CE is introduced, it should discuss the impact on LCP in Uu. Currently, the LCP priority in Uu is listed below:

- MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; - MAC CE for Sidelink Configured Grant Confirmation; - MAC CE for LBT failure; - MAC CE for Timing Advance Report; - MAC CE for SL-BSR prioritized according to clause 5.22.1.6; - MAC CE for (Extended) BSR, with exception of BSR included for padding; - MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR; - MAC CE for Positioning Measurement Gap Activation/Deactivation Request; - MAC CE for the number of Desired Guard Symbols; - MAC CE for Case-6 Timing Request; - MAC CE for (Extended) Pre-emptive BSR; - MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding; - MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; - data from any Logical Channel, except data from UL-CCCH; - MAC CE for Recommended bit rate query; - MAC CE for BSR included for padding; - MAC CE for SL-BSR included for padding.

Regarding to the SL-specific LBT failure MAC CE, it should have similar priority as the LBT failure MAC CE of NR-U, hence, its priority can be between MAC CE for LBT failure and the MAC CE for Timing Advance Report.

Proposal 6: If SL-specific LBT failure MAC CE is introduced, its priority should between MAC CE for LBT failure and the MAC CE for Timing Advance Report.

DRX

With the introduction of LBT in SL-U, the PSFCH transmission may also impacts by LBT. The following agreements were reached in RAN1:

Agreement If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied - Alt 1: PSFCH resources are (pre-)configured - Alt 2: PSFCH resources are dynamically indicated - Combination of above alternatives are not precluded - FFS details of above alternatives

Agreement - Type 1 SL channel access procedure is applicable to the following transmissions by a UE: - PSSCH/PSCCH transmission(s) scheduled or configured by a gNB in SL Mode 1 resource allocation. - PSSCH/PSCCH transmission(s) from the UE in SL Mode 2 resource allocation. - Other SL transmissions including S-SSB and PSFCH transmissions from a UE - FFS: how to set CAPC for S-SSB and PSFCH - Note: Type 1 can be used to initiate a COT

Agreement To address PSFCH transmission dropping due to LBT failure, the followings are to be studied: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Alt 2: PSFCH resources are dynamically indicated - Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE - Alt 4: drop PSFCH transmission - Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB - Combination of above alternatives are not precluded - FFS details of above alternatives

Based on the above agreements, it is obvious that RAN1 has agreed that LBT is applicable for PSFCH, and in order to address the PSFCH transmission dropping due to LBT failure, many solutions were proposed in RAN1 but there is no final conclusion was reached.  If the PSFCH can be finally transmitted by using RAN1 solutions, there is no impact on SL DRX procedure in RAN2. But if there is still the possibility that PSFCH cannot be transmitted due to LBT failure, the impact on SL DRX should be considered in RAN2. In our understanding, this case should be treated similar as PSFCH is not transmitted due to UL/SL prioritization. But if Alt 1 is used (Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission), in case of PSFCH is not transmitted, the sl-drx-HARQ-RTT-Timer should be started based on which PSFCH should be further discussed.

Proposal 7: If only one PSFCH can be used per PSCCH/PSSCH transmission, UE should start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback regardless of the SL LBT outcome.

Proposal 8: RAN2 should wait for RAN1 conclusion before discussing how to handle the sl-drx-HARQ-RTT-Timer for the case that more than one PSFCH occasions are used per PSCCH/PSSCH transmission.

Besides the SL DRX timer maintenance, in NR-U, some companies also proposed to extend the active time in case of LBT failure, e.g., if LBT failure happens during the one onduration period, UE may need to postpone the transmission to the next onduration period, which will increase the latency. But in NR-U, this enhancement was not agreed in NR-U. Whether it is needed for SL-U can be further discussed.

Proposal 9:  RAN2 to discuss whether SL DRX active time can be extended in case of SL LBT failure in SL-U.

UE autonomous retransmission using CG

In NR-U, UE autonomous retransmission for two cases:

- Case 1: PDCCH scheduling for CG retransmission failed due to LBT failure, in this case, cg-RetransmissionTimer was introduced and if the cg-RetransmissionTimer expiries, UE consider it as NACK and UE autonomous retransmission can be performed.

- Case 2: New UL transmission is failed due to LBT failure, UE can use the subsequent CG for autonumous retransmission.

For Case 1, in SL-U, if the HARQ FB is enabled and no ACK is received due to LBT failure of PSFCH, UE will consider it as NACK according to the current description in MAC spec. Hence, retx can be performed based on UE implementation (mode 2) or based on network scheduling (mode 1). Hence, it seems unnecessary to introduce UE autonoumous retransmission and cg-RetransmissionTimer for this case.

For Case 2, blind retransmission is already supported for SL retransmission by configuring sl-MaxTransNum, it means UE autonomous retransmission is already supported in SL.

Proposal 10: Enhancements on UE autonomous retransmisson is not needed in this release.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: SL-specific consistent LBT failure detection is not relevant to cast type/DST/unicast link.

Proposal 2: For mode-2 UE in RRC_CONNECTED, if the resource granularity of SL LBT is SL BWP/SL carrier, UE autonomous SL-specific consistent LBT failure recovery is unnecessary; otherwise, UE autonomous SL-specific LBT failure recovery can be supported.

Proposal 3: For mode-2 UE in RRC_CONNECTED, if the resource granularity of LBT is SL BWP/SL carrier, UE should indicate to gNB if SL-specific consistent LBT failure happens; otherwise, UE should indicate SL RLF to gNB if SL-specific LBT failure recovery failed.

Proposal 4: If the resource granularity of LBT is SL BWP/SL carrier, once SL-specific consistent LBT failure is detected, SL RLF should be triggered and CONNECTED UE should report the SL RLF to gNB; otherwise, SL-specific LBT failure MAC CE can be used to indicate the failure to gNB.

Proposal 5: If SL-specific LBT failure MAC CE is introduced, once it is triggered and there is no UL grant, SR can be triggered and SR resource configured for Sidelink CSI Reporting MAC CE can be reused.

Proposal 6: If SL-specific LBT failure MAC CE is introduced, its priority should between MAC CE for LBT failure and the MAC CE for Timing Advance Report.

Proposal 7: If only one PSFCH can be used per PSCCH/PSSCH transmission, UE should start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback regardless of the SL LBT outcome.

Proposal 8: RAN2 should wait for RAN1 conclusion before discussing how to handle the sl-drx-HARQ-RTT-Timer for the case that more than one PSFCH occasions are used per PSCCH/PSSCH transmission.

Proposal 9:  RAN2 to discuss whether SL DRX active time can be extended in case of SL LBT failure in SL-U.

Proposal 10: Enhancements on UE autonomous retransmisson is not needed in this release.

Reference

- R2-2210935 Summary of [AT119bis-e][504][V2XSL] Consistent SL LBT failure (vivo)