3GPP TSG-RAN2 Meeting 120		 R2-2211647

Toulouse, 14th – 18th November, 2022

Agenda item:	5.2.3

Source:	LG

Title:	Summary on user plane correction for NR Rel-15 and Rel-16 (LG)

Document for:	Discussion and Decision

- Introduction

This is the summary of below contributions.

-  R2-2211240, R2-2211269, R2-2211395, R2-2211396, R2-2211561, R2-2211562, R2-2211942, R2-2211943, R2-2211944, R2-2211945, R2-2211946

- Discussion

2.1 For changes in R2-2211240 (For Rel-16)/R2-2211269(For Rel-17)

2.1.1 change:

Reason for change:

In 5.22.1.1, the parameter name of MinSubChannelNumPSSCH and MaxSubchannelNumPSSCH is wrong and should be corrected into sl-MinSubChannelNumPSSCH and sl-MaxSubchannelNumPSSCH.

Change: In section 5.22.1.1, change MinSubChannelNumPSSCH and MaxSubchannelNumPSSCH to sl-MinSubChannelNumPSSCH and sl-MaxSubchannelNumPSSCH.

3>	select an amount of frequency resources within the range, if configured by RRC, between sl-MinSubChannelNumPSSCH and sl-MaxSubchannelNumPSSCH included in sl-PSSCH-TxConfigList and, if configured by RRC, overlapped between sl-MinSubChannelNumPSSCH and sl-MaxSubchannelNumPSSCH indicated in sl-CBR-PriorityTxConfigList for the highest priority of the logical channel(s) allowed on the carrier and the CBR measured by lower layers according to clause 5.1.27 of TS 38.215 [24] if CBR measurement results are available or the corresponding sl-defaultTxConfigIndex configured by RRC if CBR measurement results are not available;

Rapporteur summary: Editorial change can be acceptable.

Proposal 1. RAN2 is to agree on the correction in the R2-2211240.

2.2 For changes in R2-2211395 (For Rel-16)/R2-2211396(For Rel-17)

2.2.1 change:

Reason for change: Sidelink UE cannot select exceptional resource pool when its sensing result is available.

d.

Change: In section 5.22.1.1, Sidelink UE can select any resource pool except the exceptional resource pool for both one shot and multiple-transmission.

1>	if the MAC entity has selected to create a selected sidelink grant corresponding to transmissions of multiple MAC PDUs, and SL data is available in a logical channel:

2>	if the MAC entity has not selected a pool of resources allowed for the logical channel:

3>	if sl-HARQ-FeedbackEnabled is set to enabled for the logical channel:

4>	select any pool of resources configured with PSFCH resources among the pools of resources except the resource pool in sl-TxPoolExceptional;

3>	else:

4>	select any pool of resources among the pools of resources except the resource pool in sl-TxPoolExceptional;

~

1>	if the MAC entity has selected to create a selected sidelink grant corresponding to transmission(s) of a single MAC PDU, and if SL data is available in a logical channel, or an SL-CSI reporting is triggered:

2>	if SL data is available in the logical channel:

3>	if sl-HARQ-FeedbackEnabled is set to enabled for the logical channel:

4>	select any pool of resources configured with PSFCH resources among the pools of resources except the resource pool in sl-TxPoolExceptional;

3>	else:

4>	select any pool of resources among the pools of resources except the resource pool in sl-TxPoolExceptional;

2>	else if an SL-CSI reporting is triggered:

3>	select any pool of resources among the pools of resources except the resource pool in sl-TxPoolExceptional.

Rapporteur summary: Correction is not critical. No one would interpret that UE would choose an exectpional pool. We can listen to companie’s views and make a decision on the correction.

Proposal 2. RAN2 is to discuss on the correction in the R2-2211395.

2.3 For changes in R2-2211561 (For Rel-16)/R2-2211562(For Rel-17)

2.3.1 change:

Reason for change: In TS 38.321, in 5.22.1.3.1a Sidelink process, it is described how to use sl-MaxTransNum for sidelink configured grant of resource allocation mode 1. However, the using of the maximum transmission number (i.e. sl-MaxTxTransNumPSSCH)  in resource allocation mode 2 is missing in the normtive text.

Change: The using of the maximum transmission number (i.e. sl-MaxTxTransNumPSSCH)  in mode 2 is added in the normtive text.

To generate a transmission, the Sidelink process shall:

1>	if there is no uplink transmission; or

1>	if the MAC entity is able to simultaneously perform uplink transmission(s) and sidelink transmission at the time of the transmission; or

1>	if the other MAC entity and the MAC entity are able to simultaneously perform uplink transmission(s) and sidelink transmission at the time of the transmission respectively; or

1>	if there is a MAC PDU to be transmitted for this duration in uplink, except a MAC PDU obtained from the Msg3 buffer, the MSGA buffer, or prioritized as specified in clause 5.4.2.2, and the sidelink transmission is prioritized over uplink transmission:

2>	instruct the physical layer to transmit SCI according to the stored sidelink grant with the associated Sidelink transmission information;

2>	instruct the physical layer to generate a transmission according to the stored sidelink grant;

2>	if HARQ feedback has been enabled for the MAC PDU according to clause 5.22.1.4.2:

3>	instruct the physical layer to monitor PSFCH for the transmission and perform PSFCH reception as specified in clause 5.22.1.3.2.

2>	if sl-PUCCH-Config is configured by RRC for the stored sidelink grant:

3>	determine transmission of an acknowledgement on the PUCCH as specified in clause 5.22.1.3.2.

1>	if this transmission corresponds to the last transmission of the MAC PDU:

2>	decrement SL_RESOURCE_RESELECTION_COUNTER by 1, if available.

NOTE 1:	If the number of HARQ retransmissions selected by the MAC entity has been reached, or if a positive acknowledgement to a transmission of the MAC PDU has been received, or if a negative-only acknowledgement was enabled in the SCI and no negative acknowledgement was received for the transmission of the MAC PDU, the MAC entity determines this transmission corresponds to the last transmission of the MAC PDU for Sidelink resource allocation mode 2. How to determine the last transmission in other cases is up to UE implementation.

1>	if sl-MaxTransNum corresponding to the highest priority of the logical channel(s) in the MAC PDU has been configured in sl-CG-MaxTransNumList for the sidelink grant by RRC and the number of transmissions of the MAC PDU has been reached to sl-MaxTransNum; or

1>	if sl-MaxTxTransNumPSSCH has been configured in SL-PSSCH-TxConfigList for the sidelink grant by RRC and the number of transmissions of the MAC PDU has been reached to sl-MaxTxTransNumPSSCH; or

1>	if a positive acknowledgement to this transmission of the MAC PDU was received according to clause 5.22.1.3.2; or

1>	if negative-only acknowledgement was enabled in the SCI and no negative acknowledgement was received for this transmission of the MAC PDU according to clause 5.22.1.3.2:

2>	flush the HARQ buffer of the associated Sidelink process.

Rapporteur summary: We can listen to companie’s views and make a decision on the correction.

Proposal 3. RAN2 is to discuss on the correction in the R2-2211561.

2.4 For changes in R2-2211942(discussion paper)/R2-2211943 (Correction for Rel-16)/R2-2211944(Correction for Rel-17)

2.4.1 change:

Reason for change: During RAN2#119-e meeting, RAN2 agreed UL skipping is not applied to sidelink. Therefore, a new condition to check if the UE is configured with sl-ScheduledConfig should be introduced for UE to judge if UL skipping should be performed or not.

Change: In section 5.4.3.1.3, add a new condition to check if the UE is configured with sl-ScheduledConfig should be introduced for UE to judge if UL skipping should be performed or not.

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR; and

2>	if Sidelink resource allocation mode 1 is not configured by RRC:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR; and

2>	if Sidelink resource allocation mode 1 is not configured by RRC:

3>	not generate a MAC PDU for the HARQ entity.

Rapporteur summary: RAN2 is to discuss on the correction in the R2-2211943. For your information, in #504 offline discussion, there was a slight majority companies (8 out of 14) that do not want to make any change in the RAN2 spec.

Porposal 4. RAN2 is to discuss on the correction in the R2-2211943.

2.5 For changes in R2-2211945 (For Rel-16)/R2-2211946(For Rel-17)

2.4.1 change:

Reason for change: According to the current specification, if the transmission of a sidelink MAC PDU has been positively acknowledged, then HARQ entity will flush the HARQ buffer of the associated sidelink process and the following retransmission sidelink grants will be ignored. Therefore, there is no need to clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU upon reception of the dynamic sidelink grant.

Change: In section 5.22.1.1, delete the corresponding description that if the transmission of a MAC PDU has been positively acknowledged, the UE should clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU.

5.22.1.1	SL Grant reception and SCI transmission

Sidelink grant is received dynamically on the PDCCH, configured semi-persistently by RRC or autonomously selected by the MAC entity. The MAC entity shall have a sidelink grant on an active SL BWP to determine a set of PSCCH duration(s) in which transmission of SCI occurs and a set of PSSCH duration(s) in which transmission of SL-SCH associated with the SCI occurs. A sidelink grant addressed to SLCS-RNTI with NDI = 1 is considered as a dynamic sidelink grant.

If the MAC entity has been configured with Sidelink resource allocation mode 1 as indicated in TS 38.331 [5], the MAC entity shall for each PDCCH occasion and for each grant received for this PDCCH occasion:

1>	if a sidelink grant has been received on the PDCCH for the MAC entity's SL-RNTI:

2>	if the NDI received on the PDCCH has not been toggled compared to the value in the previously received HARQ information for the HARQ Process ID:

3>	use the received sidelink grant to determine PSCCH duration(s) and PSSCH duration(s) for one or more retransmissions of a single MAC PDU for the corresponding Sidelink process according to clause 8.1.2 of TS 38.214 [7].

2>	else:

3>	use the received sidelink grant to determine PSCCH duration(s) and PSSCH duration(s) for initial transmission and, if available, retransmission(s) of a single MAC PDU according to clause 8.1.2 of TS 38.214 [7].

1>	else if a sidelink grant has been received on the PDCCH for the MAC entity's SLCS-RNTI:

2>	if PDCCH contents indicate retransmission(s) for the identifed HARQ process ID that has been set for an activated configured sidelink grant identified by sl-ConfigIndexCG:

3>	use the received sidelink grant to determine PSCCH duration(s) and PSSCH duration(s) for one or more retransmissions of a single MAC PDU according to clause 8.1.2 of TS 38.214 [7].

2>	else if PDCCH contents indicate configured grant Type 2 deactivation for a configured sidelink grant:

3>	trigger configured sidelink grant confirmation for the configured sidelink grant.

2>	else if PDCCH contents indicate configured grant Type 2 activation for a configured sidelink grant:

3>	trigger configured sidelink grant confirmation for the configured sidelink grant;

3>	store the configured sidelink grant;

3>	initialise or re-initialise the configured sidelink grant to determine the set of PSCCH durations and the set of PSSCH durations for transmissions of multiple MAC PDUs according to clause 8.1.2 of TS 38.214 [7].

Rapporteur summary: For clarity of UE operation in mode 1, the corresponding text is required.

Proposal 5. RAN2 is to discuss on the correction in the R2-2211945.

- Conclusion

Proposal 1. RAN2 is to agree on the correction in the R2-2211240.

Proposal 2. RAN2 is to discuss on the correction in the R2-2211395.

Proposal 3. RAN2 is to discuss on the correction in the R2-2211561.

Porposal 4. RAN2 is to discuss on the correction in the R2-2211943.

Proposal 5. RAN2 is to discuss on the correction in the R2-2211945.